3GPP TSG-RAN WG4 Meeting 93	R4-1915398

# Reno, USA, 18th – 22nd Nov, 2019

Source: 	Skyworks Solutions, Inc.

Title: 	DC_(n)41 and DC_41_n41 Data in Support for MPR&AMPR Improvements

Agenda Item:	10.14.2 Improvements to A-MPR/MPR for 26 dBm n41 and B41/n41 EN-DC   [LTE_NR_B41_Bn41_PC29dBm]

Document for:	Discussion

# Introduction

In RAN4#93 meeting, we have provided input to intra-band ENDC MPR/AMPR improvements in [1]. In this contribution, we provide the set of DC_(n)41 and DC_41_n41 data in support of NS04 AMPR and NS01 MPR improvements in terms of antenna isolation and better fit of back-off curves[1,3], power sharing and allocation types discussed in [2].

# Discussion

The data provided here spans a large set of measurements comprising:

- DC_(n)41 and DC_41_n41 contiguous and non-contiguous intra-band ENDC combinations

- NS01 and NS04 requirements

- QPSK CP-OFDM waveforms principally (highest MPR) with corner case DFT-s-OFDM waveforms

- LTE channel of 20 MHz and NR channel of 40 MHz both 15 kHz SCS and 20MHz gap for non-contiguous intra-band ENDC

- NR/LTE Waveform pairs as follows (LTE channel is at the lowest frequency):

  - QPSK CP-OFDM 1RB-40MHz 			QPSK_20MHz_1RB99		Closest 1RB+1RB case

  - QPSK CP-OFDM 1RB215-40MHz		QPSK_20MHz_1RB0		Furthest 1RB+1RB case

  - QPSK CP-OFDM 1RB215-40MHz		QPSK_20MHz_2RB0		Furthest A=0.54 case

  - QPSK CP-OFDM 2RB214-40MHz		QPSK_20MHz_1RB0		Furthest A=0.54 case

  - QPSK CP-OFDM 2RB214-40MHz		QPSK_20MHz_2RB0

  - QPSK CP-OFDM 3RB213-40MHz		QPSK_20MHz_3RB0

  - QPSK CP-OFDM 4RB212-40MHz		QPSK_20MHz_2RB0

  - QPSK CP-OFDM 4RB212-40MHz		QPSK_20MHz_8RB0

  - QPSK CP-OFDM 6RB210-40MHz		QPSK_20MHz_6RB0

  - QPSK CP-OFDM 20RB196-40MHz		QPSK_20MHz_1RB0		Unbalanced allocation

  - QPSK CP-OFDM 20RB196-40MHz		QPSK_20MHz_20RB0

  - QPSK CP-OFDM 108RB108-40MHz		QPSK_20MHz_1RB99		Unbalanced allocation

  - QPSK CP-OFDM 108RB108-40MHz		QPSK_20MHz_50RB0		Unbalanced allocation

  - QPSK CP-OFDM 108RB0-40MHz		QPSK_20MHz_100RB0	In-gap ACLR case

  - QPSK CP-OFDM 216RB0-40MHz		QPSK_20MHz_100RB0	Full allocation

  - QPSK DFT-s-OFDM 1RB215-040MHz	QPSK_20MHz_1RB0		Furthest 1RB+1RB case

  - QPSK DFT-s-OFDM 216RB0-40MHz	QPSK_20MHz_100RB0 	Full allocation

- 29 dBm to 11 dBm Ptotal range

- with LTE/NR power differences as follows: -18, -13, -9, -6, -4, -2.5, -1, 0, 1, 2.5, 4, 6, 9, 13, 18 db

- Data is provided in terms of Ptotal back-off or PLTE/PNR contours

Tables provide the raw back-off values for different NS01/NSO4 requirements and are provided either in terms of back-off (positive value) or value at Pmax if it already fulfills the requirement (negative values). “KO” cases can be ignored as they are cases where IM3 and IM5 fall within the LTE and/or NR channels. To elaborate further on actual NS01/NS04 MPR/AMPR one needs to find the worst performance (highest back-off) for the relevant criteria and corresponding allocations types.

For example, NS01 contiguous must meet ACLR, -10 dBm/MHz SEM, -13 dBm/MHz IMD3 and -30 dBm/MHz IMD5.

Note that in our analysis we do the analysis across the 15 LTE/NR power pairs over a 15 to 18 dB Ptotal range thus the tables provided are a small subset of the data analyzed to provide our conclusions. Note that measurements are within 0.5 dB in absolute value and reproducibility is about 0.3 dB this explains that the PLTE/PNR contour plots are not smo.th (the smoothing option from excel is removed in the plots).

Finally this raw data is for an ideal 3GPP configuration and as discussed in [3] margin must be taken to account for PA frequency, temperature and process variations, worst case over power share range (not necessarily equal power), and different behavior of ET and APT implementations.

Curves similar to Figures 1 and 2 will be provided. They provide the reachable PNR vs PLTE and have the 26dBm power share (red) and the allocation size related AMPRtot power share (black) curves.

## DC_(n)41 Data

This data could not be analyzed in time for RAN4#92b but is provided here. Unfortunately, it does not include all the contiguous/non-contiguous and inner/outer allocations that have been discussed in [4] and thus some further measurements will be needed.

Table 1: Raw back-off values for equal power case and 10 dB isolation

Table 2: Raw back-off values for equal power case and 13 dB isolation

Observations versus antenna isolation: about 1dB improvement is shown for worst cases

Observations versus allocation size: 1RB+1RB case requires IM3 at -25 dBm/MHz worst case for NS01 but -13 dBm/MHz IM3 and -30 dBm/MHz IM5 (always met for IM3 backoff)

Observations versus allocation type: Inner allocations need to meet ACLR, -10 dBm/MHz SEM, -13 dBm/MHz IMD5 and thus a low back-off.

Observations versus power sharing range: Narrow inner allocation can reach close to 29 dBm total power

Figure 1: Inner and outer allocations for contiguous case and narrow allocations

## DC_41_n41 Data

This data could not be fully analyzed in time for RAN4#92b but full analysis is provided here. Data is rounded within 0.5dB and hides some of the differences better data will be provided in later revision.

Table 3: Raw back-off values for equal power case and 10 dB isolation

Table 4: Raw back-off values for equal power case and 13 dB isolation

Observations versus antenna isolation:

Observations versus allocation size:

Observations versus allocation type:

Observations versus power sharing range: 29 dBm cannot be reached even with the best case allocations

Figure 2: Inner and outer allocations for non-contiguous case and narrow allocations

# Conclusions

In this contribution, we provided a large set of DC_(n)41 and DC_41_n41 data in support of NS04 AMPR and NS01 MPR improvements in terms of antenna isolation and better fit of back-off curves[1,3], power sharing and allocation types discussed in [2] allowing a number of observations:

Due to lack of time, the full data and related observations will be provided in a later revision.

# References

[1] R4-1915047, A-MPR&MPR improvements for DC_(n)41 DC_41_n41 and DC_3_n3, Skyworks Solutions Inc., RAN4#93

[2] R4-1911618 Introduction of inner allocations for intra-band contiguous ENDC and NR UL CA, Skyworks Solutions Inc., RAN4#92b

[3] R4-1911647 Potential Improvements for Band 41 Intra-band ENDC and Applicability to Other Cases, Skyworks Solutions Inc., RAN4#92b

[4] R4-1914273 [NR ULCA] types of contiguous and non-contiguous allocation for class B and C UL CA, Skyworks Solutions Inc., RAN4#93