3GPP TSG-RAN WG4 Meeting #93	R4-1915599

# Reno, Nevada, USA, Nov 18th - 22th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0261 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on interruption requirements due to one-shot timing adjustment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The interruptions shall be allowed when UE is required to perform one-shot timing adjustment, and the corresponding interruption requirements have not been defined in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the interruption requirements due to one-shot timing adjustment for EN-DC in new section 8.2.1.2.8. - To introduce the interruption requirements due to one-shot timing adjustment for NR SA in new section 8.2.2.2.7. - To introduce the interruption requirements due to one-shot timing adjustment for NE-DC in new section 8.2.3.2.8. - To introduce the interruption requirements due to one-shot timing adjustment for NR-DC in new section 8.2.4.2.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption requirements due to one-shot timing adjustment will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.8, 8.2.2.2.7, 8.2.3.2.8, 8.2.4.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.1.2.8	Interruption due to one-shot timing adjustment requirement

When UE is required to perform one-shot timing adjustment on a serving cell as defined in section 7.1.2.2, the UE is allowed to cause an interruption of up to 1 symbol to the serving cell due to one-shot timing adjustment.

When intra-band carrier aggregation in FR1 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in the same band.

When intra-band carrier aggregation in FR2 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any activated serving cell in FR2.

<End of Change 1>

<Start of Change 2>

8.2.2.2.7	Interruption due to one-shot timing adjustment requirement

When UE is required to perform one-shot timing adjustment on a serving cell as defined in section 7.1.2.2, the UE is allowed to cause an interruption of up to 1 symbol to the serving cell due to one-shot timing adjustment.

When intra-band carrier aggregation in FR1 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in the same band.

When intra-band carrier aggregation in FR2 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in FR2.

<End of Change 2>

<Start of Change 3>

8.2.3.2.8	Interruption due to one-shot timing adjustment requirement

When UE is required to perform one-shot timing adjustment on a serving cell as defined in section 7.1.2.2, the UE is allowed to cause an interruption of up to 1 symbol to the serving cell due to one-shot timing adjustment.

When intra-band carrier aggregation in FR1 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in the same band.

When intra-band carrier aggregation in FR2 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in FR2.

<End of Change 3>

<Start of Change 4>

8.2.4.2.8	Interruption due to one-shot timing adjustment requirement

When UE is required to perform one-shot timing adjustment on a serving cell as defined in section 7.1.2.2, the UE is allowed to cause an interruption of up to 1 symbol to the serving cell due to one-shot timing adjustment.

When intra-band carrier aggregation in FR1 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in the same band.

When intra-band carrier aggregation in FR2 is performed, the interruption of up to 1 symbol due to one-shot timing adjustment is to any other activated serving cell in FR2.

<End of Change 4>