3GPP TSG-RAN WG4 Meeting #93   	R4-1915865

Reno, USA, 18th – 22th Nov, 2019

Agenda item:	9.18.2.2

Source:	Samsung

Title:	Initial simulation results for NR demodulation performance in Rel-16

Document for:	Discussion

# 1	Introduction.

During the RAN4#92 meeting, the additional BS demodulation requirement with FR2 PUSCH 2T2R was discussed. It was agreed that MCS 12 will replace the existing requirements for MCS 16, considering the OTA testability limit.

In this contribution, the initial ideal and impairment results are provided for performance requirement.

# 2	Discussion

## 2.1	Requirements for MCS12 with FR2 2T2R

In this subsection, considering the OTA test limitation, it is agreed to replace all the existing FR2 2T2R requirements with MCS 16 by MCS12 in TS 38.104 and TS 38.141-2, and reuse the previous agreed simulation assumption for FR2 2T2R cases with MCS16. As shown in table 1, the assumption is provided.

Table1: simulation assumption for FR2 2T2R with MCS 12

| Parameter | value |
| --- | --- |
|  | FR2 |
| Transform precoding | Disabled |
| Number of Tx | 2 |
| Number of Rx | 2 |
| Number of layers | 2 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1(0), 1+1(0,8) |
| symbols length | 10 |
| Start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | Full RB allocation of the applicable BW |
| MCS index | 12 |
| Carrier frequency (GHz) | 30 |
| Propagation channel | MCS 12 TDLA 30 ns, 300Hz |
| SCS and BW | 60KHz (FR2): 50MHz, 100MHz; 120KHz: 50MHz, 100MHz, 200MHz |
| Timing offset | 0 |
| Frequency offset | 0 |
| PTRS | 16 (Configured with KP_TRS =2, LP_TRS =1) and (Not configured) |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Limited buffer rate matching | Disabled |
| Number of HARQ transmission | 4 |

Based on offline checking with other companies, the FRC tables with new FR2 test cases are updated as follows:

Table 2: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 57024 | 114048 | 27648 | 57024 | 114048 |
| Total symbols per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Table3: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

## 2.2	Simulation results for MCS12 with FR2 2T2R

In this subsection, based on assumption, the ideal and impairment simulation results are provided for requirements.

Table 4: ideal and impairment simulation results for FR2 2T2R with MCS 12

| SCS,BW | MCS | Tx, Rx | Number DMRS | PTRS | channel | SNR targeting 70% of TP (ideal) | SNR targeting 70% of TP (impairment ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60kHz: 50MHz | 12 | 2T2R | 1(0) | With PTRS | TDL-A 30ns, 300Hz | 13.07 | 15.57 |
| 60kHz: 100MHz | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 13.91 | 16.41 |
| 120kHz: 50MHz | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 12.08 | 14.58 |
| 120kHz: 100MHz | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 11.72 | 14.22 |
| 120kHz: 200MHz | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 11.81 | 14.31 |
| 60kHz: 50MHz | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.07 | 13.57 |
| 60kHz: 100MHz | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.24 | 13.74 |
| 120kHz: 50 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.44 | 13.94 |
| 120kHz: 100 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.35 | 13.85 |
| 120kHz: 200 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.41 | 13.91 |
| 60kHz: 50 | 12 | 2T2R | 1(0) | Without PTRS | TDL-A 30ns, 300Hz | 12.62 | 15.12 |
| 60kHz: 100 | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 13.30 | 15.80 |
| 120kHz: 50 | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 11.75 | 14.25 |
| 120kHz: 100 | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 11.42 | 13.92 |
| 120kHz: 200 | 12 | 2T2R | 1(0) |  | TDL-A 30ns, 300Hz | 11.52 | 14.02 |
| 60kHz: 50 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 10.74 | 13.24 |
| 60kHz: 100 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 10.87 | 13.37 |
| 120kHz: 50 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.09 | 13.59 |
| 120kHz: 100 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 10.95 | 13.45 |
| 120kHz: 200 | 12 | 2T2R | 1+1(0,8) |  | TDL-A 30ns, 300Hz | 11.02 | 13.52 |

# 3	Conclusion

In this contribution, the updated FRC tables for FR2 new cases are provided. Meanwhile, we provide the initial the ideal and impairment results for FR2 2T2R MCS requirements.

Table 2: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 57024 | 114048 | 27648 | 57024 | 114048 |
| Total symbols per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Table3: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

# References

R4-1907241, WF on NR PUSCH demodulation requirements in Rel-15, Nokia