3GPP TSG-RAN4 Meeting #93	 R4-1916052

# Reno, Nevada, USA, November 18th -22nd, 2019

Source:	Ericsson

Title:	WF for random data content of physical channels for NR test modes

Agenda item:	7.8.3.3

Document for:	Approval

# 1.	Introduction

During RAN4#93, discussion on the possible need to update the NR TM with random data rather than using ‘all zero’ data was discussed.  Companies providing measurement results due to the difference in random data versus ‘all zero’ data was presented and discussed [1,2].

# 2.	Background

The following block diagram captures the data generation for both LTE and NR.

All zeros

Scramble

RNTI

Cell ID

Subframe #

LTE

All zeros

Scramble

RNTI

Cell ID

NR

The scrambler in LTE generates 10 different seeds due to the subframe number. However, in NR, the seed is the same for all slots.

For dynamic range testing in NR, the number of symbols over which averaging is performed was increased to allow for more “randomness” in the data.[3]

# 3.	Way Forward

Further study of the waveform properties that are affected due to design of test models in conjunction with “all zero” data and scrambling. To mitigate and correct the issue in relation to scrambling, to further study and investigate “random” data generation and its impact on essential waveform properties.. Other solutions than “random” data generation are not precluded.

Companies are encouraged to provide analysis in next meeting for RAN4 to make decision on whether NR TMs shall be updated to address the issue of the NR TMs not having an amplitude statistic corresponding to Rayleigh distribution.  Considerations of auto correlation of the NR TMs should also be considered.

The main issues to be answered next meeting:

- Impact of “all zero” data versus “random” data for all  NR TMs

- Impact due to PN sequence lengths. e.g. PN15, PN23

- Impact due to cell-ID for following configuration for test with multiple cells:

NIDcell=1 for the lowest configured carrier, NIDcell=2 for the 2nd lowest configured carrier,…, NIDcell=n for the nth configured carrier

- Requirement impacts due to ‘all zero’ data in NR TMs such as TX signal quality (EVM), emissions etc.

- Impacts on 5 MHz and 20 MHz cases are prioritized.

# 3.	References

[1]	R4-1914558, “Further elaboration on NR Test Model”, Ericsson

[2] 	R4-1916050, “CCDF analysis result with use of random data in Test Model payload”, Keysight

[3]	R4-1912332, “On Total power dynamic range conformance test for NR BS”, Nokia,Nokia Shanghai Bell