3GPP TSG-RAN WG4 Meeting #93                                     R4-1916124

Reno, USA, 18th – 22th November, 2019

Source: 	Skyworks Solutions, Inc.

Title: 	Experimental Measurements for n1 40MHz CBW REFSENS and Band n34 Protection

Agenda Item:	10.18.1 UE RF (38.101-1) [NR_n1_BW-Core]

Document for:	Approval

# Introduction

At RAN plenary meeting 85, a new WI [1] has been agreed to introduce NR Band n1 with up to 40 MHz UE channel bandwidth (CBW) support. This contribution provides transmitter (TX) noise measurements in receiver (RX) band and IMD transmit power levels for several DFT-S-OFDM QSPK SCS15kHz waveforms.  We propose reference sensitivity levels and UL configurations and raw power class power 3 back-off levels to protect band 39/34 when n1 is operated at 30MHz and 40MHz channel bandwidth (CBW). This data can be used to calibrate simulation results. Final A-MPR tables need to take additional margin to measured values to account for various PA control schemes.

# Experimental Measurements

## Power Amplifier Calibration and Test Waveforms

- A band 1 power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

- According to WF [2], Tx noise in Rx band power level measurements are performed with the following waveforms for which the allocated Resource Blocks (RBs) are located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth.

  - 25 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz with following RB allocations: 128RB5,

  - 30 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz with following RB allocations: 128RB32,

  - 40 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz with following RB allocations: 128RB88,

  - Image and Local Oscillator (LO) rejection is set to -28 dBc,

  - Counter-IM3 (C-IM3) set to -60 dBc,

  - Counter-IM5 (C-IM5) set to -70 dBc,

  - TX noise is measured at the band 1 duplex distance of 190 MHz.

Figure 1 below shows the spectrum of the PA input waveform used for TX noise measurement in RX band for 40MHz CBW operation.

Figure 1: PA input test waveform for 40MHz TX noise in RX band measurements.

## Measurement Results

The measured TX noise levels in RX band are:

- 25 MHz: -53.1 dBm,

- 30 MHz: -52.1 dBm,

- 40 MHz: -50.4 dBm.

# MSD and Reference Sensitivity Estimation

We assume:

- 50 dB Tx to RX and Tx to Antenna duplexer rejection in Rx band, as a worst case assumption,

- 10 dB Primary antenna to diversity antenna isolation,

- 4 dB Post PA loss,

- 4 dB Antenna to LNA front-end losses,

- 3 dB Antenna diversity gain

- -1 dB SNR.

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

Figure 2 below shows summarizes the MSD analysis and shows that n1 MSD levels are:

- 25 MHz CBW: 0.3 dB,

- 30 MHz CBW: 0.3 dB,

- 40 MHz CBW: 0.4 dB.

Figure 2: n1 MSD analysis for 25,30,40 MHz CBW

Considering the less than 0.5 dB MSD levels, we confirm that previously agreed REFSENS levels for 25MHz and 30MHz and we make the following REFSENS and UL configuration proposal for n1 operation at 40MHz CBW:

Proposition 1: Adopt the following n1 REFSENS specifications as listed in table 1 and table 2 below.

Table 1: Proposed UL configuration for n1 REFSENS.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.8 |  |  |  |  |  |  |

Table 2: Proposed n1 REFSENS levels.

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 301 | 301 |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# n1 A-MPR Checks for Band 39/34 Protection

Table 3 below summarizes the band 39/34 requirements for A-MPR analysis.

Table 3: Additional requirements for B39/B34 Protection

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1880 | - | 1895 | -40 | 1 |  |
| Frequency range | 1895 | - | 1915 | -15.5 | 5 | 1, 2 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1, 2 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

For band 34 protection, we assume a worst case filter rejection of 11dB.

Measurements are performed with the following RF parameters:

- A band 1 power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at channel edge.

- IMD power levels are measured at the PA output port with:

  - 30 MHz CBW, carrier = 1935 MHz, QPSK DFT-s-OFDM SCS 15 kHz with following RB allocations:1RB0,1RB5,1RB10,1RB20,6RB0,6RB5,6RB10,6RB20,10RB0,10RB5,10RB10,10RB20,20RB0,20RB5,20RB10,20RB20,30RB0,30RB5.

  - 40 MHz CBW, carrier = 1960 MHz, QPSK DFT-s-OFDM SCS 15 kHz with following RB allocations:1RB215,1RB201,1RB196,1RB190,1RB175,10RB206,10RB200,10RB196,1RB0,1RB5,1RB10,1RB15,1RB20,1RB25,1RB33.

  - Image and Local Oscillator (LO) rejection is set to -28 dBc

  - Counter-IM3 (C-IM3) set to -60 dBc.

  - Counter-IM5 (C-IM5) set to -70 dBc.

## n1 30MHz CBW Measurement Results

Measurement results can be found in table 4 below.

Table 4: Measured PC3 back-off levels for band 39/34 protection at 30 MHz CBW, carrier = 1935 MHz.

| RB allocation | PC3 back-off [dB] |
| --- | --- |
| 1RB0 | 8.4 |
| 1RB5 | 7.4 |
| 1RB10 | 7.4 |
| 1RB20 | 5.1 |
| 6RB0 | 5.3 |
| 6RB5 | 5.2 |
| 6RB10 | 4.8 |
| 6RB20 | 4.9 |
| 10RB0 | 4.4 |
| 10RB5 | 5 |
| 10RB10 | 5 |
| 10RB20 | 4.9 |
| 20RB0 | 4.3 |
| 20RB5 | 4.2 |
| 20RB10 | 4.8 |
| 20RB20 | 8.5 |
| 30RB0 | 4.2 |
| 30RB5 | 4.6 |

The intention here is to provide reference measurements points to calibrate simulation results. The highest observed raw back-off level is approximately 8.5 dB for PC3 operation in n1 30MHz CBW.

Observation 1: The highest raw back-off observed for PC3 operation in n1 30MHz CBW is approximately 8.5dB. A-MPR tables must add a margin to the values in Table 4 to account for all possible PA control schemes (eg. envelope tracking).

## n1 40MHz CBW Measurement Results

Table 5: Measured PC3 back-off levels for band 39/34 protection at 40 MHz CBW, carrier = 1960 MHz.

| RB allocation | A-MPR PC3 back-off [dB] |
| --- | --- |
| 1RB215 | 12.3 |
| 1RB201 | 12.1 |
| 1RB196 | 4.1 |
| 1RB190 | 4 |
| 1RB175 | 3.8 |
| 10RB206 | 9.6 |
| 10RB200 | 9.5 |
| 10RB196 | 9.5 |
| 1RB0 | 5.2 |
| 1RB5 | 4.8 |
| 1RB10 | 4.5 |
| 1RB15 | 4.4 |
| 1RB20 | 4.8 |
| 1RB25 | 4.8 |
| 1RB33 | 4.8 |

The intention here is to provide reference measurements points to calibrate simulation results. The highest observed raw back-off level is approximately 12.3 dB for PC3 operation in n1 40MHz CBW.

Observation 2: The highest raw back-off observed for PC3 operation in n1 40MHz CBW is approximately 12.3 dB. A-MPR tables must add a margin to the values in Table 5 to account for all possible PA control schemes (eg. envelope tracking).

# Conclusion

In this contribution we propose n1 UL configuration for REFSENS and n1 REFSENS levels based on power amplifier TX noise in RX band measurements. We also provide preliminary measurement raw back-off levels for power class 3 operation in n1 at 30MHz and 40 MHz CBW to protect band 39/34.

Proposition 1: Adopt the following n1 REFSENS specifications as listed in table 1 and table 2 below.

Table 1: Proposed UL Configuration for n1 REFSENS.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.8 |  |  |  |  |  |  |

Table 2: Proposed n1 REFSENS Levels.

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 301 | 301 |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: The highest raw back-off observed for PC3 operation in n1 30MHz CBW is approximately 8.5dB. A-MPR tables must add a margin to the values in Table 4 to account for all possible PA control schemes (eg. envelope tracking).

Observation 2: The highest raw back-off observed for PC3 operation in n1 40MHz CBW is approximately 12.3 dB. A-MPR tables must add a margin to the values in Table 5 to account for all possible PA control schemes (eg. envelope tracking).

# References

[1]		RP-192009, New WID on adding wider channel bandwidths in Band n1, Swisscom, Ericsson

[2]		R4-1912978, WF on UE RF requirements for adding channel BW to band n1, Ericsson, Qualcomm, Huawei, China Unicom, RAN4#92-bis, Chongqing, China, October 14-18 2019.