3GPP TSG RAN WG4 #93 meeting                   	R4-1916144

Reno, Nevada, U.S., 18th – 22nd Nov., 2019

Agenda Item:	9.4.4

Source:	LG Electronics

Title:	TP on 5G V2X UE RF requirements for SL operation at licensed whole band

Document for:	Approval

- Introduction

In the last RAN4 #92bis meetings, RAN4 agreed the NR V2X operating scenarios and priorities for 5G V2X service.

Based on the 1st priority of NR V2X service, we provide general RF core requirements for SL operation at whole licensed band.

****************** Start of the TP in clause 7 & 8 of TR38.886 ************************

7	Operating bands and channel arrangement

7.1	Operating bands

7.1.1	Operating bands in FR1

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 NR V2X operating bands in FR1

| NR V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |
| # n381 |  | 2570 MHz |  | - | 2620 MHz | 2570 MHz |  | - | 2620 MHz | TDD |  | PC5 |  |
| n47 |  | 5855 MHz |  | - | 5925 MHz | 5855 MHz |  | - | 5925 MHz | TDD |  | PC5 |  |
| Note1: |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

7.2	Channel bandwidth

7.2.1	Channel bandwidth in FR1

The NR V2X Communication channel bandwidths and operating bands are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 7.2.1-1 NR V2X Communication channel bandwidth

|  |  | NR V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n47 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |

<Unchanged sections are omitted>

8	Transmitter characteristics

8.1	NR V2X UE Tx requirements in FR1

8.1.1	Maximum output power for NR V2X UE

The following V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 8.1-1: V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| n38 |  |  |  |  | 23 | ±2 |  |  |
| n47 |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: NR Band n47 is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

8.1.2	UE maximum output power reduction

[Editor Note]: Maximum output power reduction requirements for NR V2X service will be updated after RAN1 decision for the detail physical channel design of each sidelink channel.

The following assumption can serve as a starting point for MPR simulation assumptions.

Table 8.1.2-1: MPR simulation assumption for NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 2.7GHz / 5.9GHz |
| Bandwidth | 10/20/40MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/[60kHz] |
| Modulation | QPSK/16QAM/[64QAM] |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | [45dBc] |

8.1.3	UE maximum output power with additional requirements

[Editor Note]: Additional transmitted output power requirements for NR V2X service will be added after RAN1 decision for the detail physical channel design of each sidelink channel according to additional emission requirements at both licensed band and n47 band.

8.1.4	Configured transmitted power for NR V2X UE

[Editor Note]: Additional configured transmitted power requirements for NR V2X service will be added after RAN1 decision for the detail power control method of each sidelink channel.

8.1.5	Minimum output power for NR V2X UE

The minimum controlled output power of the NR V2X UE is defined as the broadband transmit power of the UE, i.e. the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 8.1-1, it is proposed that the existing requirements as specified for legacy NR UE shall apply. The minimum output power is defined as the mean power in at least one sub-frame 1 ms.

Table 8.1.5-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | [-30] | 9.375 |
| 20 | [-30] | 19.095 |
| 40 | [-27] | 38.895 |

8.1.6	Transmit OFF power NR V2X UE

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 8.1-1, it is proposed that the existing requirements as specified for legacy NR UE shall apply. Transmit off power is defined as the mean power in at least one sub-frame 1 ms.

Table 8.1.6-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50 | 9.375 |
| 20 | -50 | 19.095 |
| 40 | -50 | 38.895 |

8.1.7	ON/OFF time mask for NR V2X UE

For NR V2X service, additional requirements on ON/OFF time masks for V2X physical channels and signals are specified in this clause.

The existing general time mask requirements of NR UE in sub-clause 6.3.3.2 in TS38.101-1 apply for NR V2X sidelink UE for NR V2X operating bands in Table 8.1-1.

8.1.8	Power control for NR V2X UE

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 8.1-1, the requirements in sub-clause 8.1.8.1 apply for NR V2X transmission.

- 8.1.8.1	Absolute power tolerance

The existing absolute power tolerance requirements of legacy NR UE shall apply for V2X transmission.

Table 8.1.8.1-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

8.1.9	Transmit signal quality for NR V2X UE

- 8.1.9.1	Frequency error

The UE modulated carrier frequency for NR V2X sidelink transmissions in Table 8.1-1 shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of 1 ms compared to the relative frequency in case of using the gNB or V2X UE sidelink synchronization signals.

- 8.1.9.2	Transmit modulation quality

- 8.1.9.2.1	 Error Vector Magnitude

For V2X sidelink physical channels PSCCH and PSSCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in Table 8.1.9.2.1-1 for the corresponding modulation and transmission bandwidth for NR V2X operating bands in Table 8.1-1. When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the slot, the EVM measurement interval is reduced by one symbol, accordingly.

Table 8.1.9.2.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16QAM | % | 12.5 | 12.5 |
| 64QAM | % | 8 | 8 |

Table 8.1.9.2.1-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 8.1.5-1 |
| Operating conditions |  | Normal conditions |

- 8.1.9.2.2	 Carrier leakage

It is proposed that no changes to the existing NR requirements for carrier leakage are required for NR V2X UE for NR V2X operating bands in Table 8.1-1.

- 8.1.9.2.3 In-band emissions

For V2X sidelink physical channels PSCCH and PSSCH, In-band emissions requirements of legacy NR UE in sub-clause 6.4.2.3 shall apply according to the corresponding modulation and transmission bandwidth for NR V2X operating bands in Table 8.1-1. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the slot, the In-band emissions measurement interval is reduced by one symbol, accordingly.

- 8.1.9.2.4 EVM equalizer spectrum flatness

The existing requirements for EVM equalizer spectrum flatness for NR UE in sub-clause 6.4.2.4 in TS38.101-1 shall apply for NR V2X transmissions for NR V2X operating bands in Table 8.1-1.

8.1.10	spectrum emission mask for V2X UE

For V2X, the existing NR general spectrum emission mask can be reused for all 10MHz, 20MHz and 40MHz channel bandwidth. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB), the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1in TS 38.101-1 for the specified channel bandwidth for NR V2X operating bands in Table 8.1-1.

- 8.1.10.1	Additional spectrum emission mask for V2X UE

In LTE V2X WI, RAN4 already studied the additional spectrum emission mask in ITS band. So RAN4 reuse the additional spectrum emission mask in session 6.6.2.2.4 and 6.6.2.2.7 in TS36.101 for LTE V2X UE at n47.

When "NS_33" or “NS_34” is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 8.1.10.1-1.

Table 8.1.10.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

[Editor Note]: Additional spectrum emission requirements for NR V2X service at licensed band will be added after RAN4 decision for the protection frequency and required emission levels according to regulatory requirements in each regions.

8.1.11	ACLR requirements for V2X UE

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

Based on co-existence simulation results, a 30dB ACLR can guarantee that V2X co-exists well with DSRC, i.e. the PRR degradation of V2X to DSRC is comparable with that of DSRC to DSRC. Hence the existing ACLR requirement for licensed bands can be reused for NR V2X for NR V2X operating bands in Table 8.1-1.

Table 8.1.11-1: ACLR for NR V2X

|  | Channel bandwidth /ACLR1 / Measurement bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| ACLR1 |  |  |  | 30 dB |  | 30 dB | 30 dB |
| NR V2X channel Measurement bandwidth |  |  |  | 9.375 MHz |  | 19.095 MHz | 38.895 MHz |
| Adjacent channel centre frequency offset [MHz] |  |  |  | +10 / -10 |  | +20 / -20 | +40 / -40 |

8.1.12	Spurious emission for V2X UE

General spurious emission requirement for V2X operation should be extended to 26GHz same with that in LAA BS. In addition, UE should also meet regional requirement which defined in ETSI EN 302 571 as additional spurious emission requirement for NR V2X operating bands in Table 8.1-1.

Table 8.1.12-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz – 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for NR Band n22, Band n42 and Band n43. NOTE 2: Applies for NR Band n47. |  |  |  |

8.1.13	Spurious emission band UE co-existence for V2X UE

This clause specifies the spurious emission requirements for the specified NR V2X band, for coexistence with protected bands

Table 8.1.13-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 4, 5, 6 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 4, 5 |
| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 1,2,3 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 1,3, 7 |
| NOTE 1: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 2: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 3: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 in TS36.101 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 6: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 7: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

In Note 2, the frequency range with -30dBm/MHz will be further discussed

8.1.14	Transmit intermodulation for NR V2X UE

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 8.1-1, the requirements in sub-clause 6.5.4 in TS38.101-1 apply for NR V2X sidelink transmission.

8.2	NR V2X UE Tx requirements in FR2

9	Receiver characteristics

9.1	NR V2X UE Rx requirements in FR1

9.1.1	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB/NRB) +( NFV2X+ IM)

Where

-	kTB: Thermal noise level in units of dBm. -104 dBm, -101 dBm and -98dBm can be used for 10MHz, 20MHz and 40MHz V2X requirements, separately.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. 9~10dB NF will be considered for licensed bands.

-	IM: 2.5 dB is assumed.

-	Other parameters are FFS

9.1.2	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

For V2X, the requirement are defined for 10MHz, 20MHz and 40MHz. The parameters of reference measurement channel shall be changed according to the physical layer design specific to NR V2X for NR V2X operating bands in Table 8.1-1.

Table 9.1.2-1: Maximum input level for V2X

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | [-22~ -25] |  |  |  |  |  |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |

[9.1.3	Adjacent Channel Selectivity (ACS)]

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

ACS reflects the digital domain filter attenuation capability, which is model together with ACLR in ACIR. Based on PRR metric. In co-existence study, the existing NR ACS requirement  reuse for both NR UE to V2X UE and DSRC UE to V2X UE urban interference scenarios, PRR loss is less than 5%. Therefore, it is proposed that the same ACS values shall be kept unchanged for V2X UE at n47.

In licensed band, RAN4 shall reuse the same ACS values since there was legacy NR or LTE system.

As 10MHz is typical channel bandwidth used in the V2X band, the interferer bandwidth shall be used with 10MHz for NR V2X operating bands in Table 8.1-1.

Table 9.1.3-1: Adjacent channel selectivity for V2X

|  |  | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| ACS | dB |  |  |  | 33.0 |  | 27.0 | 24.0 |

Table 9.1.3-2: Test parameters for Adjacent channel selectivity for V2X, Case 1

| Rx Parameter |  | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration |  | dBm | PREFSENS_V2X + 14 dB |  |  |  |  |  |  |
| PInterferer |  | dBm |  |  |  | REFSENS +45.5dB |  | REFSENS +39.5dB | REFSENS +36.5dB |
| BWInterferer |  | MHz |  |  |  | 10 |  | 10 | 10 |
| FInterferer (offset) |  | MHz |  |  |  | 10 / -10 |  | 15 / -15 | 25 / -25 |
|  | NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |  |  |  |

Table 9.1.3-3: Test parameters for Adjacent channel selectivity for V2X, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm |  |  |  | -53.5 |  | -47.5 | -44 |
| PInterferer | dBm | [-22 ~ -25] |  |  |  |  |  |  |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 |
| FInterferer (offset) | MHz |  |  |  | 10 / -10 |  | 15 / -15 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |  |  |  |

[9.1.4	Blocking characteristics]

- [9.1.4.1	In-band blocking]

In EN 302 571, the blocking level shall not be less than -30 dBm. And the blocking is tested at ± 50 MHz, ± 100 MHz, and ± 200 MHz frequency offset. Blocking testing shall be performed at least at 6 different frequency offset positions. The blocking level is larger than exiting case 2 blocking interferer in the specification, which is in the second alternate channel offset from the wanted signal. Considering the total 70MHz band width of the V2X band, ± 50 MHz is already outside the operating band or close at the band edge depends on the placement of the carrier.

RAN4 reuse the In-band blocking requirements in LTE V2X UE as shown in Table 9.1.4.1-1 and Table 9.1.4.1-2 NR V2X UE in Table 8.1-1.

Table 9.1.4.1-1: In band blocking parameters for V2X

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 | 12 |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 |
| FIoffset, case 1 | MHz |  |  |  | 15 |  | 15 | 15 |
| FIoffset, case 2 | MHz |  |  |  | 25 |  | 25 | 25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |  |  |

Table 9.1.4.1-2: In-band blocking for V2X

| NR band | Parameter | Unit |  | Case 1 | Case 2 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 |  | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 |  | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| n47 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |

- [9.1.4.2	Out-of-band blocking]

Out-of-band band blocking in existing specification for licensed bands is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity shall be applied.

RAN4 reuse the Out-of- band blocking of LTE V2X UE as shown in Table 9.1.4.2-1 and Table 9.1.4.2-2 for NR V2X UE in Table 8.1-1.

Table 9.1.4.2-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 | 12 |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |

Table 9.1.4.2-2: Out of band blocking

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| n47 | FInterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. |  |  |  |  |  |

[9.1.5	Spurious response]

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained.

For V2X, 10MHz, 20MHz and 40MHz channel bandwidths are considered. RAN4 reuse the spurious response of LTE V2X UE as shown in Table 9.1.5-1 and Table 9.1.5-2 for V2X UE in Table 8.1-1.

Table 9.1.5-1: Spurious response parameters for V2X

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 | 12 |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |  |

Table 9.1.5-2: Spurious response for V2X

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

[9.1.6	Intermodulation characteristics]

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The wide band intermodulation requirement is defined using modulated NR carrier and CW signal as interferer. 10MHz channel bandwidth will be chosen as the modulated interfering signal for both 10MHz and 20MHz channel bandwidth.

RAN4 reuse the intermodulation characteristics of LTE V2X UE as shown in Table 9.1.6-1 for NR V2X UE in Table 8.1-1.

Table 9.1.6-1: Wide band intermodulation

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz | 40 MHz |
| n47 | Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  |  |  | 9 | 12 |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  | 10 |  |  | 10 | 10 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  | -BW/2 – 15 / +BW/2 + 15 |  |  | -BW/2 – 15 / +BW/2 + 15 | -BW/2 – 15 / +BW/2 + 15 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is FFS NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |  |  |  |  |  |

9.2	NR V2X UE Rx requirements in FR2

<End of Changes>