3GPP TSG RAN WG4 Meeting #93	R4-1913104

Reno, USA, November 18th – 22nd, 2019

3GPP TSG-RAN WG1 Meeting #98bis 	R1-1911616

# Chongqing, China, October 14th – 20th, 2019

Title:	LS on applicable timing for pathloss RS activated/updated by MAC-CE

Response to:

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	3GPP TSG-RAN WG1

To:	3GPP TSG-RAN WG4

Contact Person:

#### Name:	Jonghyun Park

E-mail Address:	jonghyun10.park@lge.com

Attachments:	None

1. Overall Description:

RAN1 has discussed updating pathloss reference RS for PUSCH/SRS via MAC CE, and concluded on the following working assumptions:

Working assumption:

Pathloss reference RS for PUSCH can be activated/updated via a MAC CE

- The MAC CE message can activate/update the value of PUSCH-PathlossReferenceRS-Id corresponding to sri-PUSCH-PowerControlId.

  - Note(Informative): In TS38.331, the mapping is given by SRI-PUSCH-PowerControl, in which the linkage is between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS(s) if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

- Send an LS to RAN4 asking opinion on this working assumption.

Working assumption:

Pathloss reference RS for AP-SRS/SP-SRS can be activated/updated via a MAC CE.

- A UE can be configured with multiple pathloss RSs by RRC and one of them can be activated/updated via the MAC CE for a SRS resource set.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

- Send an LS to RAN4 asking opinion on this working assumption.

The working assumptions state that filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

2. Actions:

To RAN4 group:

ACTION: 	RAN1 kindly asks RAN4 feedback whether there is any concern on the applicable timing and whether it should be captured in RAN4 specs or RAN1 specs.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #99 	18 - 22 November 2019    Reno, USA

TSG-RAN WG1 Meeting #100 	24 – 28 February 2020    Athens, Greece