3GPP TSG-RAN WG4 Meeting #93	R4-1913144

Reno, NV, USA, 18-22 Nov 2019

Title:	TP to TR38.831: On REFSENS for Inter-band CA

Source:	Qualcomm Incorporated

Agenda item:	9.14.7

Release:	Rel-16

Work Item:	NR_RF_FR2_req_enh

Responsible WG:	RAN4

Document for:	Approval

# 1.	Introduction

In [1], we identified some RF requirements that would require special consideration for FR2 inter-band CA. In this contribution we propose a TP to capture REFSENS assumptions for inter-band CA. The proposal is crafted to be agnostic of whether the gNBs for the participating bands are co-located or not.

Note that the TP itself is introduced without change marks, so change marks can be reserved for delta change to the standard.

# 2. 	Text Proposal

## 4.7	FR2 UE requirements for inter-band DL CA

4.7.x	REFSENS

The REFSENS requirement in TS38.101-2 for inter-band CA can be captured as follows (changes to v15.7 show up with change marks):

< start of changes to TS38101-2 >

## 7.3A	Reference sensitivity for CA

### 7.3A.1	General

### 7.3A.2	Reference sensitivity power level for CA

(unchanged sub-sections 7.3A.2.1 and 7.3A.2.2)

:

:

#### 7.3A.2.3	Inter-band CA

For inter-band carrier aggregation, the requirement applies per band, with one component carrier per operating band. The inter-band requirement applies with common AoA for all active component carriers. The uplink is assigned to one band. The throughput for each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity for each carrier specified in Table 7.3.2.1-1. The requirement on each component carrier applies when the power in the component carriers in other bands correspond to their respective EIS levels for that AoA and test condition. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

< end changes to TS38101-2 >