3GPP TSG RAN WG4 meeting #93						              R4-1913266

Reno, Nevada, USA, November 18 - 22, 2019

Agenda Item: 9.8.2.2

Source: MediaTek

Title: Discussion on Cell-phase Synchronization Between gNBs

Document for:	Discussion

# Introduction

In RAN4#92-bis meeting, it has been online discussed whether or not to introduce requirements on cell-phase synchronization between gNBs, but no conclusion is reached. In this contribution, we provide our views on this topic.

# Discussion on Cell-phase Synchronization Between gNBs

In DL-TDOA positioning, the network configures a UE to measure the arrival time of DL-PRS transmitted from several cells. The UE estimates RSTDs between cells and reports the measured RSTDs to the network. The network then estimates the UE’s position based on the coordinates of the cells and the received RSTD report from the UE.

It is generally recognized that the cell-phase synchronization between gNBs is a key factor which heavily impacts the accuracy of final positioning. If such synchronization is not controlled within a certain acceptable range, the DL-TDOA positioning would fail to meet the commercial requirements for all scenarios.

For DL-TDOA positioning, the following table summarizes the best estimated positioning error of 80% UEs for indoor office and UMi scenarios given in TR 38.855 [1], assuming T1 = 50ns gNB synchronization error.

| Company | Indoor office | UMi |
| --- | --- | --- |
| HW | 18.9m | 22.2m |
| Vivo | 21.1m | 19.5m |
| ZTE | 18.12m | 26.22m |
| Intel | 23 | 14 |
| QC | 24 | 10.8 |
| MTK | N/A | 12.7 |
| ESA | N/A | 18 |

In TR 38.855 [1], for DL-TDOA positioning, it has been verified by companies that with perfect gNB synchronization, the commercial accuracy requirements can be meet for indoor office and UMi scenarios. However, with 50ns gNB synchronization error, the commercial accuracy requirements cannot be meet for indoor office and UMi scenarios, as shown in the table above.

Proposal 1: Introduce requirements on cell-phase synchronization between gNBs, and the synchronization error is recommended to be at least within [-35ns, 35ns]

Therefore, we have the following proposal:

# Conclusions

In this contribution, we have the following proposal:

Proposal 1: Introduce requirements on cell-phase synchronization between gNBs, and the synchronization error is recommended to be at least within [-35ns, 35ns]

# Reference

[1] TR 38.855, “Study on NR positioning support”