3GPP TSG-WG RAN4 Meeting #93	R4-1913288

# Reno, NV, USA, 18 – 22 November 2019

Agenda item:	8.6.3 [LTE_CA_R16_xBDL_2BUL-Core]

Source:	Nokia, Nokia Shanghai Bell, [Verizon]

Title:	TP to TR 36.716-03-02 for 3DL/2UL CA_2-13-46

Document for:	Approval

# 1	Introduction

For CA_2-13-46 band combinations, 2 UL CA_2A-13A is already completed for DL CA_2A-13A-46D, however, the fallback CAs are not completed yet.

This TP is to include missing fallbacks for CA_2-13-46. The text proposal to TR 36.716-03-02 is attached in the annex.

Annex: TEXT PROPOSAL TO TR 36.716-03-02 v0.7.0

<Start of Changes>

## 6.10 LTE-A inter-band CA: Band 2 and Band 13 and Band 46 DL with 2 bands UL

### 6.10.1	List of specific combination issues

#### 6.10.1.1	Channel bandwidth per operating band for CA

Table 6.10.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-46A | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_2A-13A-46C | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-13A-46D | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

#### 6.10.1.2	Co-existence studies for LTE-A inter-band UL CA_2A-13A and DL CA_2A-13A-46A

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.10.1.2-1.

Table 6.10.1.2-1: Co-existence study for UL CA_2A-13A and DL CA_2A-13A-46A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1554 | 1574 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2331 | 2361 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2126 | 2266 | 5254 | 5394 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 | 6181 | 7104 | 7304 |

In Table 6.10.1.2-1, 3rd harmonic from the band 2 can impact the band 46. Since the band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101, there is no need to study for this harmonic problem.

#### 6.10.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-46A, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.10.1.3-1: IMD frequency range for UL CA_2A-13A and DL CA_2A-13A-46A

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_2A-13A-46A | CA_2A-13A | 3*fc_13A + 1*fc_2A | 3*BW_13A + 1*BW_2A |

The same analysis as CA_2A-13A-46A is applicable to CA_2A-13A-46C and CA_2A-13A-46D.

#### 6.10.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

<End of Changes>