3GPP TSG-WG RAN4 Meeting #93	R4-1913290

# Reno, NV, USA, 18 – 22 November 2019

Agenda item:	8.6.3 [LTE_CA_R16_xBDL_2BUL-Core]

Source:	Nokia, Nokia Shanghai Bell, [Verizon]

Title:	TP to TR 36.716-03-02 for 3DL/2UL CA_2-46-48

Document for:	Approval

# 1	Introduction

For CA_2-46-48 band combinations, 2 UL CA_2A-48A is already completed for DL CA_2A-46D-48A and CA_2A-46E-48A, however, fallback CAs and some higher order CA are not completed yet.

This TP is to include missing fallbacks for CA_2A-48A-46A and CA_48C-46A. Furthermore, CA_2A-46D-48C, CA_2A-46C-48C, and CA_2A-46A-48C are included, too.

The text proposal to TR 36.716-03-02 is attached in the annex.

Annex: TEXT PROPOSAL TO TR 36.716-03-02 v0.7.0

<Start of Changes>

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes self-interference problems for LTE-A inter-band carrier aggregation (CA) 3 bands DL with 2 bands UL.

Table 5.1.5-1: Summary of self-interference analysis for 3 bands DLwith 2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A | CA_2A-13A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-13A | - | 4th IMD | - | 6.2 dB |
| CA_2A-2A-4A-5A | CA_2A-5A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-5A | - | 2nd IMD 5th IMD | - | - 2nd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_4A-5A of TS36.101. |
| CA_2A-2A-5A-66A-66A CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |
|  | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-46D | CA_2A-5A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_5A-46D-66A | CA_5A_46A | - | 5th IMD | - | 0.3 dB |
|  | CA_5A_66A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_2A-13A-66A-66B CA_2A-13A-66A, CA_2A-2A-13A-66A, CA_2A-13A-66A-66A, CA_2A-13A-66B, CA_2A-13A-66C, | CA_2A-13A | - | 4th IMD | - | 7.2 dB |
|  | CA_13A-66A | - | 4th IMD | - | 6.2 dB |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A | CA_13A-66A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_2A-13A-46D CA_2A-13A-46A-46D CA_2A-13A-46A-46C CA_2A-13A-46C CA_2A-13A-46A-46A CA_2A-13A-46A CA_2A-13A-46E | CA_2A-13A | 3rd Harmonic | 4th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_1A-3A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_2A-12A-66A | CA_2A-12A | 3rd Harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_12A-66A | 3rd Harmonic impact from B12 to B66 | Side lobe impact of 4th IMD | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. - 0 dB |
| CA_1A-3A-42D | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD problem was already covered in Table 7.3.1A0g:3DL/2UL_CA_1A-3A-42A/CA_1A-3A of TS36.101. |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_1A-42C | - | - | - | N/A |
|  | CA_3A-42C | - | - | - | N/A |
| CA_2A-46E-48A, CA_2A-46D-48A CA_2A-46D-48C CA_2A-46C-48C CA_2A-46D-48A CA_2A-46A-48C CA_2A-46C-48A CA_2A-46A-48A | CA_2A-48A | 3rd Harmonic | 2nd IMD 3rd IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101 - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101 - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_2A-48A-66A | CA_2A-48A | - | 4th IMD | - | - FSS for MSD |
|  | CA_48A-66A | - | 2nd IMD 5th IMD | - | - 28.3 dB for IMD2 - 0 dB for IMD5 |
| CA_3A-8A-38A | CA_3A-8A | - | 2nd IMD 3rd IMD | - | - 26.4 dB for IMD2 - 15.7 dB for IMD3 |
| CA_1A-1A-3C-5A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-5A | - | - | - | N/A |
|  | CA_3A-5A | - | - | - | N/A |
| CA_1A-1A-3C-28A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-28A | - | 5th IMD | - | - 4 dB for IMD5 |
|  | CA_3A-28A | 3rd Harmonic | 4th IMD | - | - 3rd harmonic impact from B28 to B1 was covered in Table 7.3.1A-0a of TS36.101. - 11 dB for IMD5 |
| CA_1A-3A-3A-7A, CA_1A-3A-3A-7A-7A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |
| CA_2A-14A-30A | CA_2A-14A | - | - | - | N/A |
|  | CA_14A-30A | - | - | - | N/A |
| CA_2A-14A-66A CA_2A-2A-14A-66A CA_2A-14A-66A-66A CA_2A-2A-14A-66A-66A CA_2A-14A-66A-66A-66A | CA_2A-14A | - | 4th IMD | - | -FFS for MSD |
|  | CA_14A-66A | - | 4th IMD | - | -FFS for MSD |
| CA_14A-30A-66A CA_14A-30A-66A-66A | CA_14A-30A | - | - | - | N/A |
|  | CA_14A-66A | - | - | - | N/A |
| CA_2A-46E-66A CA_2A-46D-66A CA_2A-46C-66A CA_2A-46A-66A | CA_2A-66A | 3rd harmonic | 3rd IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101. - FFS for exclusion zone |
| CA_13A-48A-66A | CA_13A-48A | - | 3rd IMD | - | -FFS for MSD |
|  | CA_13A-66A | 2nd harmonic | 5th IMD | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS36.101. -FFS for MSD |
|  | CA_48A-66A | - | - | - | N/A |
| CA_46D-48C-66A CA_46C-48C-66A CA_46A-48C-66A CA_46D-48A-66A CA_46C-48A-66A CA_46A-48A-66A | CA_48A-66A | 3rd harmonic | 2nd IMD 3rd IMD | - | - No need to study for 3rd harmonic impact since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. - FFS for exclusion zone |
| CA_1A-7A-20A | CA_1A-7A | - | 5th IMD | - | -FFS for MSD |
|  | CA_1A-20A | - | - | - | N/A |
|  | CA_7A-20A | - | - | - | N/A |
| CA_1A-3A-3A-8A | CA_1A-3A | - | - | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | - | - | - | N/A |
|  | CA_3A-8A | - | - | - | N/A |
| CA_1A-7A-8A CA_1A-7A-7A-8A | CA_7A-8A | - | - | - | N/A |
| CA_1A-7A-7A-8A | CA_1A-7A | - | - | - | N/A |
|  | CA_1A-8A | 3rd harmonic | - | - | - 3rd harmonic impact from B8 to B7 was covered in Table 7.3.1A-0a of TS36.101. |

<Next Changes>

## 6.15 LTE-A inter-band CA: Band 2 and Band 46 and Band 48 DL with 2 bands UL

### 6.15.1	List of specific combination issues

#### 6.15.1.1	Channel bandwidth per operating band for CA

Table 6.15.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-46E-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46D-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46C-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46A-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46D-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46C-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46A-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

<End of Changes>