3GPP TSG-RAN WG4 Meeting #93                                            R4-1913334

Reno,USA, 18th – 22th Nov, 2019

Agenda Item:   9.5.1

Source:        Samsung

Title:           Simulation on ACLR and min TX power for IAB-MT

Document for:  Discussion

# Introduction

This contribution provides further simulation results for FR2 Layout 1 IAB co-existence case according to AH agreement.

Study until next meeting:

ACLR (20, 24,28dB) vs. minimum Tx power for 5% degradation

# Simulation results

For each candidate ACLR value of IAB-MT, the TX min power is configured to derive the level which will result in 5% throughput loss. In the simulation presented here the bottleneck is the 5% edge user throughput loss as shown in table below. The throughput loss approximates 5% with 1dB as step of Tx min power of TRP level.

|  |  | IAB TX min power(dBm) | Average user throughput (Kbps) | Throughput loss | 5% edge user throughput (Kbps) | Throughput loss |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline without IAB | UE ACLR: 17dB gNB ACS:23dB |  | 379963.322 | -0.33 | 295116.710 |  |
| IAB 100% active | IAB MT ACLR: 28dB | -5 | 378695.948 | -0.52 | 287762.060 | -2.49 |
|  |  | -4 | 378005.668 | -0.74 | 283740.850 | -3.85 |
|  |  | -3 | 377168.524 | -0.57 | 279496.570 | -5.29 |
|  | IAB MT ACLR: 24dB | -7 | 377787.186 | -0.71 | 283821.310 | -3.83 |
|  |  | -6 | 377247.689 | -0.89 | 280866.080 | -4.83 |
|  |  | -5 | 376569.874 | -1.09 | 276478.870 | -6.32 |
|  | IAB MT ACLR: 20dB | -18 | 375834.709 | -1.09 | 274443.610 | -7.01 |
|  |  | -19 | 375837.734 | -1.09 | 274449.690 | -7 |
|  |  | -21 | 375839.912 | -1.09 | 274449.690 | -7 |
|  |  | -25 | 375840.336 | -0.33 | 274449.690 | -7 |

According to simulation results shown in the table we have below observations:

Observation 1: for 28dB IAB-MT ACLR, to get 5% edge user throughput loss the IAB MT TX min power would be between -3 to -4dBm.

Observation 2: for 24dB IAB-MT ACLR, to get 5% edge user throughput loss the IAB MT TX min power would be between -5 to -6dBm.

Observation 3: for 20dB IAB-MT ACLR, to get 5% edge user throughput loss the IAB MT TX min power would be lower than -25dBm.

# 4 Reference

[1] R4-1913052, Ad-hoc meeting mintues for IAB RF