3GPP TSG-RAN WG4 #93	R4-1913413

Reno, United States, 18th – 22nd November 2019

Agenda item:	9.1.1.5

Source: 	Qualcomm Incorporated

Title: 	Synchronization raster in NR-U

Document for:	Approval

# Introduction

In RAN4 #92bis a WF for the definition of synchronization raster for NR-U operation was agreed [4]. In this WF, a set of GSCN points is defined but some of them were left in square brackets pending decision on the channel raster. In this contribution we express our views towards finalization of the synchronization raster for NR-U.

# Synchronization raster

In RAN4 #92bis, RAN4 agreed the following proposal [4]:

Considering a frequency offset of minimum 3 full RBs at 30kHz SCS to account for intra-carrier guardbands for each sub-band, define the following GSCN points: GSCN = {[8996], 9010, 9024, [9037/9038], 9051, 9065, 9079, 9093, 9107, [9121], [9218], 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, [9371], 9385, 9402, 9416, 9430, 9444, 9458, [9471], [9485], [9499], [9513]} Proposed values are conditioned on NR minimum channel guard bands If it is later found that larger guard bands are needed, it is common understanding that some of the SSB points may need to be changed

If we map the GSCN points in square brackets to their correspondent frequency location, we notice that most of them correspond to sub-bands whose relative channel raster point was not agreed yet. Table 1 explicates this mapping.

Table 1. GSCN to sub-band frequency mapping

| GSCN | SSB center frequency [MHz] | Correspondent sub-band center frequency [MHz] |
| --- | --- | --- |
| 8996 | 5155.68 | 5160 |
| 9037/9038 | 5214.72/5216.16 | 5220 |
| 9121 | 5335.68 | 5340 |
| 9218 | 5475.36 | 5480 |
| 9371 | 5695.68 | 5700 |
| 9471 | 5839.68 | 5845 |
| 9485 | 5859.84 | 5865 |
| 9499 | 5880 | 5885 |
| 9513 | 5900.16 | 5905 |

In our companion contribution [5], we expressed our views on the definition of the channel raster points left for further discussion. To avoid overlapping with ITS band, we proposed to remove the points above 5835MHz whose channels would partly overlap with the frequency range 5850MHz – 5925MHz (ITS band). Based on this rationale, we see no advantage in defining the last four GSCN points in Table 1, since they would correspond to unused channels and would only result in additional UE energy consumption in initial acquisition. Conversely, we see benefit in defining channel raster points for sub-bands 5160MHz, 5340MHz, 5480MHz, 5700MHz [5], and hence definition of corresponding GSCN points 8996, 9121, 9218, 9371 is necessary.

Proposal 1: define the following synchronization raster GSCN points for NR-U operation in band n46:

{GSCN = 8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458}

The entry in synchronization raster Table 5.4.3.3-1 in [3] would read:

| NR Operating Band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n46 (NOTE) | 30kHz | [] | 8993 - <1> - 9530 |

NOTE: The following GSCN are allowed for operation in band n46:

{GSCN = 8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458}

# Conclusions

In this contribution we presented our views on the definition of synchronization raster for NR-U operation in band n46.

Based on the above observations, we made the following proposal:

Proposal 1: define the following synchronization raster GSCN points for NR-U operation in band n46:

{GSCN = 8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458}

The entry in synchronization raster Table 5.4.3.3-1 in [3] would read:

| NR Operating Band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n46 (NOTE) | 30kHz | [] | 8993 - <1> - 9530 |

NOTE: The following GSCN are allowed for operation in band n46:

{GSCN = 8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458}

# Reference

- RP-181339, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated.

- RP-182878, “NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated.

- 3GPP 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v15.4.0.

- R4-1912982, “WF for sync raster for NR-U”, Qualcomm Incorporated.

- R4-1913414, “Channel raster in NR-U”, Qualcomm Incorporated.