3GPP TSG-RAN WG4 Meeting #93																		 R4-1913458

Reno, USA, 18th – 22nd Nov, 2019

Agenda Item:	9.8.1.

Source:	Intel Corporation

Title:	Further discussion on specification structure for NR positioning RRM requirements in Rel16

Document for:		Discussion

# Introduction

The work item of “NR positioning” was approved [1]. As agreed in the last RAN4 meeting, the new separated section in TS38.133 Section 9 and 10 will be used for NR positioning measurement requirements. However, the sub agendas being included shall be FFS.

R4#92bis Agreements: Group NR positioning measurements into single section in TS 38.133 9 Measurement Procedure 9.X NR measurements for positioning 10 Measurement Performance requirements 10.X NR measurements for positioning. Detailed sub-agendas are FFS

In this contribution we provide the more details on the origination for NR positioning RRM requirements in Rel16 in TS 38.133.

# TS 38.133 specification structure for NR positioning

In LTE TS 36.133 the RRM requirements for positioning techniques are strongly coupled with the underlying positioning technique (e.g. RSTD measurements are applied for the OTDA positioning techniques).

8.1.2.6 E-UTRAN Inter-Frequency OTDOA Measurements 88 8.1.2.6.1 E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements 89 8.1.2.6.1.1 RSTD Measurement Reporting Delay 90 8.1.2.6.2 E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements 90 The requirements in section 8.1.2.6.1 also apply for this section, assuming f1 is a TDD frequency and f2 is an FDD frequency. 90 8.1.2.6.3 E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements 90 8.1.2.6.3.1 RSTD Measurement Reporting Delay 91 8.1.2.6.4 E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements 92 8.1.2.7 E-UTRAN E-CID Measurements 92 8.1.2.7.1 E-UTRAN FDD UE Rx-Tx Time Difference Measurements 92 8.1.2.7.2 E-UTRAN TDD UE Rx-Tx Time Difference Measurements 93

Observation 1: In LTE specification the RRM requirements for positioning techniques are strongly coupled with the underlying positioning technique.

In NR positioning, multiple RAT dependent positioning methods are supported, e.g. DL/UL OTDoA, ECID, multi-RTT. In accordance to the LS [3] RAN1 agreed on the tables below that provide mapping between reference signals and measurements based on the latest status of RAN1 discussion. It is important to emphasize that the positioning techniques are indicated for information purpose as examples and do not intend to influence higher layer signalling to be developed by RAN2. Furthermore it is expected that some measurements can be reused across different positioning techniques and also the new positioning techniques can be introduced in the future.

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 DL PRS | DL RSTD | DL-TDOA |
| Rel.16 DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| Rel.16 DL PRS / Rel.16 SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| Rel. 15 SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM) | E-CID |

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 SRS for positioning | UL RTOA | UL-TDOA |
| Rel.16 SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| Rel.16 SRS for positioning, Rel.16 DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| Rel.16 SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

Observation 2: For NR positioning, the following UE/gNB measurement requirements shall be considered:

- UE measurements

  - DL PRS RSTD

  - DL PRS RSRP

  - UE Rx-Tx time difference

  - SS-RSRP/SS-RSRQ and CSI-RSRP/CSI-RSRQ

- gNB measurements

  - UL RTOA

  - UL SRS-RSRP

  - gNB Rx-Tx time difference

  - AoA and ZoA

Observation 3: For NR positioning the same UE/gNB measurements may be used for different positioning techniques.

Observation 4: If the requirements for NR positioning is structured by the positioning method, there will be much redundant contents duplicating in the multiple sections.

In principle, these requirements defined in TS 38.133 shall be kept good readability and scalability, especially considering the requirements were defined by the physical metric (e.g. SSB based RSRP or CSI-RSRP) which is regardless the usage of these metrics.

Therefore, we propose the following generic specification structure to capture the UE requirements:

Proposal 1: In TS38.133, the NR positioning measurements requirements can be organized with the types of physical measurement metrics.

9 Measurement Procedure 9.X NR measurements for positioning 9.X.1 Introduction 9.X.2 UE DL PRS RSTD measurements 9.X.3 UE DL PRS RSRP measurements 9.X.4 UE Rx-Tx time difference measurements 9.X.5 UE SS-RSRP/SS-RSRQ and CSI-RSRP/CSI-RSRQ measurements 10 Measurement Performance requirements 10.X NR measurements for positioning 10.X.1 UE DL PRS RSTD measurements 10.X.2 UE DL PRS RSRP measurements 10.X.3 UE Rx-Tx time difference measurements 10.X.4 UE SS-RSRP/SS-RSRQ and CSI-RSRP/CSI-RSRQ measurements 10.X.5 gNB Rx-Tx time difference measurements 10.X.6 UL SRS (PRS)-RSRP 10.X.7 UL SRS (PRS) RTOA 10.X.8 UL SRS (PRS) AoA

# Conclusion

In this contribution, the view on the specification structure for NR positioning RRM requirements is provided and the specification skeleton for NR positioning RRM requirements was provided in our CR [6]

Observation 1: In LTE specification the RRM requirements for positioning techniques are strongly coupled with the underlying positioning technique.

Observation 2: For NR positioning, the following UE/gNB measurement requirements shall be considered:

- UE measurements

  - DL PRS RSTD

  - DL PRS RSRP

  - UE Rx-Tx time difference

  - SS-RSRP/SS-RSRQ and CSI-RSRP/CSI-RSRQ

- gNB measurements

  - UL RTOA

  - UL SRS-RSRP

  - gNB Rx-Tx time difference

  - AoA and ZoA

Observation 3: For NR positioning the same UE/gNB measurements may be used for different positioning techniques.

Observation 4: If the requirements for NR positioning is structured by the positioning method, there will be much redundant contents duplicating in the multiple sections.

Proposal 1: In TS38.133, the NR positioning measurements requirements can be organized with the types of physical measurement metrics.

9 Measurement Procedure 9.X NR measurements for positioning 9.X.1 Introduction 9.X.2 UE DL PRS RSTD measurements 9.X.3 UE DL PRS RSRP measurements 9.X.4 UE Rx-Tx time difference measurements 9.X.5 UE SS-RSRP/SS-RSRQ and CSI-RSRP/CSI-RSRQ measurements 10 Measurement Performance requirements 10.X NR measurements for positioning 10.X.1 UE DL PRS RSTD measurements 10.X.2 UE DL PRS RSRP measurements 10.X.3 UE Rx-Tx time difference measurements 10.X.4 UE SS-RSRP/SS-RSRQ and CSI-RSRP/CSI-RSRQ measurements 10.X.5 gNB Rx-Tx time difference measurements 10.X.6 UL SRS (PRS)-RSRP 10.X.7 UL SRS (PRS) RTOA 10.X.8 UL SRS (PRS) AoA

# References

- R4-191156, “Revised WID: NR Positioning”

- R4-1910001, “On RAN4 time plan for NR positioning”

- R1-1909796, “LS on DL/UL Reference Signals and Measurements for NR Positioning”

- 3GPP TS 38.133 v15.7.0: "NR: Requirements for support of radio resource management "

- 3GPP TS 36.133 v15.8.0: "E-UTRA: Requirements for support of radio resource management "

- R4-1913459, “Draft CR on TS 38.133 structure for NR positioning RRM requirements”