3GPP TSG-RAN WG4 Meeting #93	R4-1913657

# Reno, NV, USA, 18 – 22 November, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0175 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.105 on Reciever Intermodulation signal offset correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the NR specifications, the General and Narrowband Receiver Intermodulation requirements are stated with signal freqeuncy offsets without brackets. (Brackets were removed in the recent meeting for NB Rx IM in R4-1911217 and R4-1911218). In the MSR and AAS specifications, the corresponding freqeuncy offset in case of NR interferers remain for both General and Narrowband Receiver Intermodulation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The brackets are removed for the signal freqeuncy offsets of NR signals for General and Narrowband Receiver Intermodulation. Some offset values for NR are also updated according to the agreed changes in the CR for NR in R4-1913786. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Receiver Intermodulation requirement would remain as not fully finalized and would not be aligned with the NR specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.104 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.141, 37.145-1, TS 37.145-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | 7.7.2.1, 7.7.2.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.7	Receiver intermodulation

### 7.7.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. The requirement applies per TAB connector.

### 7.7.2	Minimum requirement for MSR operation

#### 7.7.2.1	General intermodulation minimum requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal as specified in 3GPP TS 37.104 [9], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For multi-band TAB connectors, the requirement applies in addition inside any Inter RF Bandwidth gap at those connectors, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the TAB connector, using the parameters in tables 7.7.2.1-1 and 7.7.2.1-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any UTRA TDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.105 [7], subclause 7.2.1.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined for BS type 1-H in TS 38.104 [28], subclause 7.2.2

Table 7.7.2.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48 + y (NOTE 6) | PREFSENS +x dB (NOTE 2, 5) | See table 7.7.2.1-2 |
| Medium Range BS | -44 + y (NOTE 6) | PREFSENS +x dB (NOTE 3, 5) |  |
| Local Area BS | -38 + y (NOTE 6) | PREFSENS +x dB (NOTE 4, 5) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2.2. NOTE 2: For WA BS not supporting NR, "x" is equal to 6 in case of E-UTRA or UTRA wanted signals. NOTE 3: For MR BS not supporting NR, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signal. NOTE 4: For LA BS not supporting NR, "x" is equal to 12 in case of E-UTRA wanted signals, 6 in case of UTRA wanted signal. NOTE 5: For a BS supporting NR and not supporting UTRA, x is equal to 6. NOTE 6: For a BS not supporting NR, "y" is equal to zero for all BS classes. For a BS that supports NR and supporting UTRA; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. |  |  |  |

Table 7.7.2.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2,0 (BC1 and BC3) / ±2,1 (BC2) | CW |
|  | ±4,9 | 1,4 MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4,4 (BC1 and BC3) / ±4,5 (BC2) | CW |
|  | ±10,5 | 3 MHz E-UTRA signal |
| UTRA FDD and E-UTRA 5 MHz | ±7,5 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7,375 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7,25 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7,125 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| GSM/EDGE | ±7,575 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| 1,28 Mcps UTRA TDD | ±2,3 (BC3) | CW |
|  | ±5,6 (BC3) | 1,28 Mcps UTRA TDD signal |
| NR 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 10 MHz | ±7.465 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 15 MHz | ±7.43 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 20 MHz | ±7.395 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 25 MHz | ±7.465 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 30 MHz | ±7.43 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz | ±7.45 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz | ±7.35 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20MHz E-UTRA signal |

#### 7.7.2.2	General narrowband intermodulation minimum requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal as specified in 3GPP TS 37.104 [9], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For TAB connector supporting operation in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.7.2.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For multi-band TAB connectors, the requirement applies in addition inside any Inter RF Bandwidth gap at those connectors in case the gap size is at least as wide as the E-UTRA interfering signal in table 7.7.2.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the TAB connector, using the parameters in tables 7.7.2.2-1 and 7.7.2.2-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any UTRA TDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.105 [7], subclause 7.2.1.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined for BS type 1-H in TS 38.104 [28], subclause 7.2.2

Table 7.7.2.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 | PREFSENS +x dB (NOTE) | See table 7.7.2.2-2 |
| Medium Range BS | -47 |  |  |
| Local Area BS | -44 |  |  |
| NOTE: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2.2. "x" is equal to 6 dB in case of E-UTRA or UTRA or NR wanted signals. |  |  |  |

Table 7.7.2.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or edge of the sub-block |  | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidth edge or edge of sub-block inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- |
| E-UTRA 1.4 MHz |  | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  |  | ±970 (BC1 and BC3) / ±790 (BC2) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 3 MHz |  | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  |  | ±960 (BC1 and BC3) / ±780 (BC2) | 3,0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 5 MHz |  | ±360 | CW |
|  |  | ±1 060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 10 MHz (NOTE 2) |  | ±325 | CW |
|  |  | ±1 240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 15 MHz (NOTE 2) |  | ±380 | CW |
|  |  | ±1 600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 20 MHz (NOTE 2) |  | ±345 | CW |
|  |  | ±1 780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD |  | ±345 (BC1 and BC2) | CW |
|  |  | ±1 780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE |  | ±340 | CW |
|  |  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| 1,28 Mcps UTRA TDD |  | ±190 (BC3) | CW |
|  |  | ±970 (BC3) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz |  | ±360 | CW |
|  |  | ±1420 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz |  | ±370 | CW |
|  |  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz (Note 2) |  | ±380 | CW |
|  |  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz (Note 2) |  | ±390 | CW |
|  |  | ±2320 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz (Note 2) |  | ±325 | CW |
|  |  | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz (Note 2) |  | ±335 | CW |
|  |  | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz (Note 2) |  | ±355 | CW |
|  |  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz (Note 2) |  | ±375 | CW |
|  |  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz (Note 2) |  | ±395 | CW |
|  |  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz (Note 2) |  | ±415 | CW |
|  |  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz (Note 2) |  | ±435 | CW |
|  |  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz (Note 2) |  | ±365 | CW |
|  |  | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz (Note 2) |  | ±385 | CW |
|  |  | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

## 10.8	OTA Receiver intermodulation

### 10.8.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS : the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS : the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

### 10.8.2	Minimum requirement for MSR operation

#### 10.8.2.1	General intermodulation minimum requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal as specified in 3GPP TS 37.104 [9], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals at the RIB, using the parameters in tables 10.8.2.1-1 and 10.8.2.1-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined for BS type 1-O in TS 38.104 [28], subclause 10.3.2

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 10.8.2.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE 1) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS +x dB (NOTE 2, 5) | See table 10.8.2.1-2 |
|  | -48 + y – ΔmnSENS (NOTE 6) | EISminSENS + x dB (NOTE 2, 5) |  |
| Medium Range BS | -44 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS + x dB (NOTE 3, 5) |  |
|  | -44 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 3, 5) |  |
| Local Area BS | -38 + y- ΔOTAREFSENS (NOTE 6) | EISREFSENS +x dB (NOTE 4, 5) |  |
|  | -38 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 4, 5) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2. NOTE 2: For WA BS not supporting NR, "x" is equal to 6 in case of E-UTRA or UTRA wanted signals. NOTE 3: For MR BS not supporting NR, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signal. NOTE 4: For LA BS not supporting NR, "x" is equal to 12 in case of E-UTRA wanted signals, 6 in case of UTRA wanted signal. NOTE 5: For a BS supporting NR and not supporting UTRA, x is equal to 6. NOTE 6: For a BS not supporting NR, "y" is equal to zero for all BS classes. For a BS that supports NR and supporting UTRA; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class |  |  |  |

Table 10.8.2.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2,0 (BC1 and BC3) / ±2,1 (BC2) | CW |
|  | ±4,9 | 1,4 MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4,4 (BC1 and BC3) / ±4,5 (BC2) | CW |
|  | ±10,5 | 3 MHz E-UTRA signal |
| UTRA FDD and E-UTRA 5 MHz | ±7,5 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7,375 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7,25 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7,125 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| GSM/EDGE | ±7,575 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| 1,28 Mcps UTRA TDD | ±2,3 (BC3) | CW |
|  | ±5,6 (BC3) | 1,28 Mcps UTRA TDD signal |
| NR 5 MHz | [±7.5] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 10 MHz | [±7.45] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 15 MHz | [±7.43] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 20 MHz | [±7.38] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 25 MHz | [±7.45] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 30 MHz | [±7.43] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 40 MHz | [±7.45] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 50 MHz | [±7.35] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 60 MHz | [±7.49] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 70 MHz | [±7.42] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 80 MHz | [±7.44] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 90 MHz | [±25] | CW |
|  | [±7.43] | 20MHz E-UTRA signal |
| NR 100 MHz | [±7.45] | CW |
|  | [±25] | 20MHz E-UTRA signal |

#### 10.8.2.2	General narrowband intermodulation minimum requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal as specified in 3GPP TS 37.104 [9], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIB supporting operation in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 10.8.2.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least as wide as the E-UTRA interfering signal in table 10.8.2.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals at the RIB, using the parameters in tables 10.8.2.2-1 and 10.8.2.2-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [17], subclause 7.2.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 10.8.2.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 - ΔOTAREFSENS | EISREFSENS + 6 dB | See table 10.8.2.2-2 |
|  | -52 – ΔminSENS | EISminSENS + 6 dB |  |
| Medium Range BS | -47 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | -47 – ΔminSENS | EISminSENS + 6 dB |  |
| Local Area BS | -44 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | -44 – ΔminSENS | EISminSENS + 6 dB |  |
| NOTE EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2. |  |  |  |

Table 10.8.2.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or edge of the sub-block | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidthedge or edge of sub-block inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3,0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 5 MHz | ±360 | CW |
|  | ±1 060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 10 MHz (NOTE 2) | ±325 | CW |
|  | ±1 240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 15 MHz (NOTE 2) | ±380 | CW |
|  | ±1 600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 20 MHz (NOTE 2) | ±345 | CW |
|  | ±1 780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1 780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| 1,28 Mcps UTRA TDD | ±190 (BC3) | CW |
|  | ±970 (BC3) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz | [±360] | CW |
|  | [±1420] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz | [±325] | CW |
|  | [±1780] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz (Note 2) | [±380] | CW |
|  | [±1600] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz (Note 2) | [±345] | CW |
|  | [±1780] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz (Note 2) | [±325] | CW |
|  | [±1990] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz (Note 2) | [±320] | CW |
|  | [±1990] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz (Note 2) | [±310] | CW |
|  | [±2710] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz (Note 2) | [±330] | CW |
|  | [±3250] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz (Note 2) | [±350] | CW |
|  | [±3790] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz (Note 2) | [±400] | CW |
|  | [±4870] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz (Note 2) | [±390] | CW |
|  | [±4870] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz (Note 2) | [±340] | CW |
|  | [±5770] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz (Note 2) | [±340] | CW |
|  | [±5770] | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |