3GPP TSG-RAN WG4 Meeting #93                                                                 R4-1913725

# Reno, USA, 18th–22nd Nov., 2019

Source: 	CATT

Title: 	Discussion on HST PUSCH demodulation requirement

Agenda item:	9.17.2.3.1

Document for:	Agreement

- Introduction

In the recent RAN4#92bis meeting, a WF [1] on NR HST BS demodulation requirements was approved.  There are some open issues for PUSCH demodulation needing to be solved in RAN4#93.This paper will provide our view on those open issues based on evaluation results for NR HST PUSCH.

- Discussion

The agreed channel models for NR HST in WF [1] are shown below:

- HST single tap channel model for 350km/h and 500km/h - Tunnel - Ds = 300m - Dmin = 2m - Open space - Ds = 700m - Dmin = 150m

The above propagation conditions (Tunnel and Open space) are agreed for evaluation, and other detailed simuation asumptions for 350km/h and 500km/h are shown in Table A-1 in Annex.

The evaluation results for 350km/h at 2.1GHz (15kHz SCS) and for 350km/h at 3.6GHz (30kHz SCS) are shown in Table 2-1 and Table 2-2 respectively. The evaluation results for 500km/h at 2.1GHz (15kHz SCS) and for 500km/h at 3.6GHz (30kHz SCS) are shown in Table 2-3 and Table 2-4 respectively.

Table 2-1 Evaluation results for 350km/h at 2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Tunnel | 1340 | -6.32 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Open space | 1340 | -6.33 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | Tunnel | 1340 | -6.52 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | Open space | 1340 | -6.52 |

Table 2-2 Evaluation results for 350km/h at 3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Tunnel | 2334 | -5.46 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Open space | 2334 | -5.49 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | Tunnel | 2334 | -6.43 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | Open space | 2334 | -6.42 |

Table 2-3 Evaluation results for 500km/h at2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Tunnel | 1944 | -6.3 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Open space | 1944 | -5.43 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | Tunnel | 1944 | -6.5 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | Open space | 1944 | 0.8 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Tunnel | 1750 | -6.31 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Open space | 1750 | -6.34 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | Tunnel | 1750 | -6.51 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | Open space | 1750 | -3.94 |

Table 2-4 Evaluation results for 500km/h at3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Tunnel | 3334 | -5.44 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Open space | 3334 | -5.47 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | Tunnel | 3334 | -6.4 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | Open space | 3334 | -3.58 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Tunnel | 3000 | -5.34 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Open space | 3000 | -5.45 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | Tunnel | 3000 | -6.42 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | Open space | 3000 | -6.43 |

Front loaded DMRS start symbol l0:

- l0 for PUSCH mapping type A - Provide the simulation results for 350km/h and 500km/h and evaluate the following configurations and make decision: - Option 1: l0 = 3 - Option 2: l0 = 2 - Same value for both 350km/h and 500km/h can be considered

According to the evaluation results shown in Table 2-1, Table 2-2, Table 2-3 and Table 2-4, the SNR @ 70% of maximum throughput gain of l0=3 relative to l0=2 are calculated and shown in Table 2-5, Table 2-6, Table 2-7 and Table 2-8.  The SNR @ 70% of maximum throughput gain of l0=3 relative to l0=2 in Open space with 1944Hz, Open space with 1750Hz and Open space with 3334Hz are 6.23dB, 2.4dB and 1.89dB respectively, which indicates that the performance of l0=3 is better than that of l0=2. The SNR @ 70% of maximum throughput gains of l0=3 relative to l0=2 in other cases are in range of [-1, 1] dB and their absolute SNR gains are less than 1dB, which indicates that the performance of l0=3 and l0=2 are similar in these cases.

Proposal 1: Choose Option 1: l0=3.

Table 2-5 Evaluation results for 350km/h at 2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput gain of l0=3 relative to l0=2(dB) |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | Tunnel | 1340 | -0.2 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | Open space | 1340 | -0.19 |

Table 2-6 Evaluation results for 350km/h at 3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput gain of l0=3 relative to l0=2(dB) |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | Tunnel | 2334 | -0.97 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | Open space | 2334 | -0.93 |

Table 2-7 Evaluation results for 500km/h at 2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput gain of l0=3 relative to l0=2(dB) |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | Tunnel | 1944 | -0.2 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | Open space | 1944 | 6.23 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | Tunnel | 1750 | -0.2 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | Open space | 1750 | 2.4 |

Table 2-8 Evaluation results for 500km/h at 3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput gain of l0=3 relative to l0=2(dB) |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | Tunnel | 3334 | -0.96 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | Open space | 3334 | 1.89 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | Tunnel | 3000 | -1.08 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | Open space | 3000 | -0.98 |

Maximum Doppler shift for single tap HST 500km/h:

- Maximum Doppler shift - Single tap HST 500km/h - 15kHz SCS : - Option 1: 1944Hz - Option 2: 1750Hz - 30kHz SCS: - Option 1: 3334Hz - Option 2: 3000Hz

From Table 2-9, the SNR @70% of maximum throughput in Tunnel with 1944Hz and Open space with 1944Hz are -6.3dB and -5.43dB respectively, while the SNR @70% of maximum throughput in Tunnel with 1750Hz and Open space with 1750Hz are -6.31dB and -6.34dB respectively. The SNR degradations of 1944Hz compared to 1750Hz in Tunnel and Open space are 0.01dB and 0.91dB respectively, which are less than 1dB and can be acceptable. So it is more reasonable to use 1944Hz for 15 kHz SCS.

From Table 2-10, the SNR @70% of maximum throughput in Tunnel with 3334Hz and Open space with 3334Hz are -5.44dB and -5.47dB respectively, while the SNR @70% of maximum throughput in Tunnel with 3000Hz and Open space with 3000Hz are -5.34dB and -5.45dB respectively. The SNR degradations of 1944Hz compared to 1750Hz in Tunnel and Open space are -0.1dB and -0.02dB respectively, so their absolute values are less than 1dB and can be acceptable. So it is more reasonable to use 3334Hz for 30 kHz SCS.

Proposal 2: Choose Option 1: 1944Hz for 15 kHz SCS and Option 1: 3334Hz for 30 kHz SCS

Table 2-9 Evaluation results for 500km/h at 2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Tunnel | 1944 | -6.3 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Open space | 1944 | -5.43 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Tunnel | 1750 | -6.31 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Open space | 1750 | -6.34 |

Table 2-10 Evaluation results for 500km/h at 3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Tunnel | 3334 | -5.44 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Open space | 3334 | -5.47 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Tunnel | 3000 | -5.34 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | Open space | 3000 | -5.45 |

Antenna configuration:

- Antenna configuration - For tunnel - Option 1: 1x1Rx - Option 2: 1x2Rx(baseline for simulation alignment) - For open space - Option 1: 1x2Rx(baseline for simulation alignment) - Option 2: 1x8Rx

According to the minimum requirements for LTE HST PUSCH specified in TS 36.104, 1x1Rx is required for Tunnel (HST-scenario 3). In practical Tunnel scenario,1x1Rx are common antenna configuration considering trade -off between expensive leakage cable and MIMO performance.  It is more reasonable to use 1x1Rx for Tunnel. According to the minimum requirements for LTE HST PUSCH specified in TS 36.104, 1x2Rx is required for Open space (HST-scenario 1).  Although the 1x8Rx is more practical for TDD, 1x2Rx for Open space is enough for the minimum requirements of NR HST.

Proposal 3: Choose Option 1: 1x1Rx for Tunnel and Option 1: 1x2Rx for Open space.

MCS:

- MCS - Option 1: MCS#2 - Option 2: MCS#16 - Option 3: MCS#2 and MCS#16 MCS#2 and MCS#16 for simulation alignment

According to the minimum requirements for LTE HST PUSCH specified in TS 36.104, the FRC A3-x is required. The modulation order for FRC A3-x is QPSK, so the same modulation order (QPSK) is proposed to be used for NR HST PUSCH. The MCS index 2(MCS#2) denotes QPSK, and the MCS#2 is enough for the minimum requirements of NR HST.

Proposal 4: Choose Option 1: MCS#2

Carrier frequency (GHz) for 15 kHz SCS for 500km/h:

- SCS - 15kHz - 500km/h: 1.9GHz or 2.1GHz

From Table 2-11,  the SNR @70% of maximum throughput in Tunnel with 1944Hz and Open space with 1944Hz at 2.1GHz  are -6.3dB and -5.43dB respectively, while the SNR @70% of maximum throughput in Tunnel with 1944Hz and Open space with 1944Hz at 1.9GHz  are -6.3dB and -5.49dB respectively. The maximum variation of SNR @70% of maximum throughput of 2.1GHz vs. 1.9GHz is 0.06dB which is very small. So maximum Doppler shift 1944Hz is feasible for 1.9GHz and 2.1GHz

Proposal 5: Maximum Doppler shift 1944Hz is feasible for 1.9GHz and 2.1GHz.

Table 2-11 Evaluation results for 500km/h at 1.9GHz and 2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Tunnel | 1944 | -6.3 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | Open space | 1944 | -5.43 |
| CP-OFDM | 15kHz, 10MHz | 1.9 | 1+1+1 | 3 | Tunnel | 1944 | -6.3 |
| CP-OFDM | 15kHz, 10MHz | 1.9 | 1+1+1 | 3 | Open space | 1944 | -5.49 |

- Conclusion

This paper provided evaluation results for NR HST. Based on our evaluation results for NR HST PUSCH, we have the following proposals.

Proposal 1: Choose Option 1: l0=3.

Proposal 2: Choose Option 1: 1944Hz for 15 kHz SCS and Option 1: 3334Hz for 30 kHz SCS

Proposal 3: Choose Option 1: 1x1Rx for Tunnel and Option 1: 1x2Rx for Open space.

Proposal 4: Choose Option 1: MCS#2

Proposal 5: Maximum Doppler shift 1944Hz is feasible for 1.9GHz and 2.1GHz.

- References

- R4-1912809, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon, RAN4#92bis

- R4-1910983, Discussion on HST PUSCH demodulation requirement, CATT, RAN4#92bis

- Annex

Table 5-1 Simulation assumption for HST PUSCH for 350km/h and 500km/h with CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | v=350km/h | V=500km/h |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 1x2Rx(baseline for simulation alignment) | 1x2Rx(baseline for simulation alignment) |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| MCS index | Option 1: MCS#2 | Option 1: MCS#2 |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| RV | {0,2,3,1} | {0,2,3,1} |
| DMRS type | type 1 | Type1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 |
| Front loaded DMRS start symbol | Option 1: l0 = 3 Option 2: l0 = 2 | Option 1: l0 = 3 Option 2: l0 = 2 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz | 15kHz: 10MHz; 30kHz: 40MHz |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| Carrier frequency (GHz) | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz | For 15kHz SCS: 1.9GHz or 2.1GHz For 30kHz SCS: 3.6GHz |
| fd | For 15kHz SCS: 1340Hz For 30kHz SCS: 2334Hz | For 15kHz SCS: Option 1: 1944Hz Option 2: 1750Hz For 30kHz SCS: Option 1: 3334Hz Option 2: 3000Hz |
| Propagation condition | Tunnel: Ds=300m, Dmin=2m; Open space: Ds=700m, Dmin=150m | Tunnel: Ds=300m, Dmin=2m; Open space: Ds=700m, Dmin=150m |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |