3GPP TSG-RAN WG4 Meeting #93	R4-1913897

# Reno, NV, USA 18th – 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS38.133 on SINR mapping tables for L1-SINR (Section 10.1.16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core/NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | L1-SINR measurement is introduced in Rel-16 MIMO enhancement work item. Accordingly, original SS-SINR measurement mapping table for L3 reporting needs to be extended to support L1-SINR mapping, and differential SINR mapping table needs to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made to introduce support L1-SINR measurement report mapping: - SS-SINR measurement mapping table for L3 reporting is extended to support L1-SINR mapping for SS-SINR and CSI-SINR; - Differential SINR mapping table for L1 reporting is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L1-SINR measurement function is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

### 10.1.16	SINR report mapping

#### 10.1.16.1	SS-SINR measurement report mapping

The reporting range of SS-SINR for L3 reporting is defined from -23 dB to 40 dB with 0.5 dB resolution. The reporting range of SS-SINR and CSI-SINR for L1 reporting is defined from -23 dB to 40dB with 0.5dB resolution. The mapping of measured quantity is defined in Table 10.1.16.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-SINR and CSI-SINR for L1 reporting is defined from 0 dBm to -15 dB with 1 dB resolution. The mapping of measured quantity is defined in Table 10.1.16.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.16.1-1: SS-SINR and CSI-SINR measurement report mapping

| Reported value | Measured quantity value (L3 SS-SINR, L1 SS_SINR and L1 CSI-RS) | Unit |
| --- | --- | --- |
| SINR_0 | SINR<-23 | dB |
| SINR_1 | -23≤ SINR<-22.5 | dB |
| SINR_2 | -22.5≤ SINR<-22 | dB |
| SINR_3 | -22≤ SINR<-21.5 | dB |
| SINR_4 | -21.5≤ SINR<-21 | dB |
| .. | .. | … |
| SINR_123 | 38≤ SINR<38.5 | dB |
| SINR_124 | 38.5≤ SINR<39 | dB |
| SINR_125 | 39≤ SINR<39.5 | dB |
| SINR_126 | 39.5≤ SINR<40 | dB |
| SINR_127 | 40≤ SINR | dB |

Table 10.1.16.1-2: Differential SS-SINR and CSI-SINR measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured SINR from largest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR >-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR >-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR >-8 | dB |
| DIFFSINR_4 | -4≥ΔSINR >-10 | dB |
| DIFFSINR_5 | -5≥ΔSINR >-12 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-14 | dB |
| DIFFSINR_7 | -7≥ΔSINR >-16 | dB |
| DIFFSINR_8 | -8≥ΔSINR >-18 | dB |
| DIFFSINR_9 | -9≥ΔSINR >-20 | dB |
| DIFFSINR_10 | -10≥ΔSINR >-22 | dB |
| DIFFSINR_11 | -11≥ΔSINR >-24 | dB |
| DIFFSINR_12 | -12≥ΔSINR >-26 | dB |
| DIFFSINR_13 | -13≥ΔSINR >-28 | dB |
| DIFFSINR_14 | -14≥ΔSINR >-30 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

### >>>>>End of Change 1<<<<<