3GPP TSG-RAN WG4 Meeting #93	R4-1913988

Reno, US 18th – 22nd Nov 2019

Source: 	Nokia

Title: 					n26 A-MPR

Agenda Item: 			10.23.1

Document for:	Discussion

# 1	Introduction

This contribution presents simulated additional maximum power reduction (AMPR) results for two NS_13 scenarios on band n26 and related NS_13 specification.

# 2	Discussion

## 2.1	Simulation scenario and assumptions

The simulation scenario

The simulated scenarios consider NS_13 additional spurious emission requirement and UL channel positions

1)	5 MHz carrier in 817-822 MHz (Sprint)

2)	5 MHz carrier in 819-824 MHz (Sprint)

introduced in [1].

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

Modified SEM and additional spurious emissions are considered as shown Table 1, according to [1].

Table 1: NS_13 additional spurious emission requirements for n26

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |

The simulated AMPR result show total required PA backoff in case it exceeds the existing MPR specification [2].

## 2.2	Simulation Results

5 MHz carrier in 817-822 MHz

In Figure 1, AMPR results are shown considering DFT-S-OFDMA, pi/2-BPSK modulation with15 kHz and 30 kHz subcarrier spacings (SCSs). The rest of the modulations are shown in Figure 2 and Figure 3, where 15 kHz and 30 kHz SCSs are considered, respectively. 256-QAM is not shown because simulations showed no need for AMPR.

Figure 1. Simulated AMPR for pi/2 BPSK with 15 kHz (left) and 30 kHz (right) SCSs

Figure 2. Simulated AMPR for OFDMA (left) and DFT-S-OFDMA (right) using QPSK, 16QAM, and 64QAM modulations with 15 kHz SCS.

Figure 3. Simulated AMPR for OFDMA (left) and DFT-S-OFDMA (right) using QPSK, 16QAM, and 64QAM modulations with 30 kHz SCS.

5 MHz carrier in 819-824 MHz

In Figure 4, AMPR results are shown considering DFT-S-OFDMA, pi/2-BPSK modulation with15 kHz and 30 kHz subcarrier spacings (SCSs). The rest of the modulations are shown in Figure 5, where only 15 kHz SCS in considered. 256-QAM is not shown because simulations showed no need for AMPR. Similarly, with 30 kHz SCS no other modulations than pi/2-BPSK shown in Figure 4 resulted in simulated AMPR.

Figure 4. Simulated AMPR for pi/2 BPSK with 15 kHz (left) and 30 kHz (right) SCSs

Figure 5. Simulated AMPR for OFDMA (left) and DFT-S-OFDMA (right) using QPSK, 16QAM, and 64QAM modulations with 15 kHz SCS.

## 2.3	AMPR Specification for NS_13

AMPR for NS_13 is introduced in Table 2 and Table 3. Region A considers the regrowth and region B the IMD3 of the transmission channel, respectively.

Table 2: AMPR for NS_13

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB-Rbstart | A-MPR | LCRB-Rbstart | LCRB | Rbstart | A-MPR |
|  |  |  | Outer |  |  |  | Inner/Outer |
| 5 | FC = 819.5 | > 1.08 MHz / 12 / SCS | A1 | ≤ 1.08 MHz / 12 / SCS | ≤ 1.62 MHz / 12 / SCS | ≤ 0.90 MHz / 12 / SCS | A2 |
| 5 | FC = 821.5 | > 3.24 MHz / 12 / SCS | A3 | - | ≤ 0.54 MHz / 12 / SCS | ≤ 0.36 MHz / 12 / SCS | A4 |

Table 3: AMPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner/Outer | Outer | Inner/Outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 2 | 2 | 2 |
|  | QPSK | 4 | 3 | 2 | 2 |
|  | 16 QAM | 4 | 3 | - | 2.5 |
|  | 64 QAM | 4 | 3 | - | 3 |
|  | 256 QAM | - | - | - | - |
| CP-OFDM | QPSK | 5 | 4 | - | 4 |
|  | 16 QAM | 5 | 4 | - | 4 |
|  | 64 QAM | 5 | 4 | - | 4 |
|  | 256 QAM | - | - | - | - |

# 3	Conclusion

In this contribution we have presented AMPR simulations and specification for NS_13 on n26

# 4	References

- R4-1913078, “n26 Emissions requirements,” Sprint, SouthernLINC, Dish Network

- TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.” 3GPP