3GPP TSG-RAN WG4 Meeting #93	R4-1914084

Reno, US, 18th– 22nd November 2019

Agenda item:	10.16.2	BS RF (38.104)	[NR_n259-Core]

Source: 		NTT DOCOMO, INC.

Title: 	BS RF requirement for Band n259

Document for:	Approval

- Introduction

In RAN4#92bis, the need for updates related to BS RF/Conformance for the introduction of Band n259 was discussed [1]. In this contribution, we further discuss on BS RF related aspects.

2.	Discussion

In the last meeting, the necessity of updates for the introduction of Band n259 were summarized and discussed in [1]. In this section, provides approach to solve issues.

2.1	ΔfOBUE and ΔfOOB

As pointed out in [1], in order to support Band n259, ΔfOBUE and ΔfOOB in TS 38.104 shall be updated as below. In addition, corresponding changes for TS 38.141-2 shall be needed.

TS38.104

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 3250 4000 MHz | 1500 |

Table 10.5.2.3-0: ΔfOOB offset for NR operating bands in FR2

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 2-O | FUL_high – FUL_low ≤ 3250 4000 MHz | 1500 |

Proposal 1: Update tables for ΔfOBUE and ΔfOOB for FR2 to support Band n259.

2.2	Step frequency for transmitter spurious emissions (Category B) and receiver spurious emissions (Category A/B)

In Rel.15 WI [NR_newRAT-core], it was agreed to change Rx spurious emission limits for the alignment between Category A and Category B limits. According to the approved WF [4], the limits will be changed to the following limits that are align with category B transmitter spurious emissions.

Table 2: General BS receiver spurious emission limits for BS type 2-O

- A note will be added to the table for Japanese regulation.

Due to the spurious emission limit alignment of Tx spurious emissions (Category B) and Rx spurious emissions (Category A/B), step frequencies for Band n259 need to be added. Based on ERC Recommendation 74-01, step frequencies of Band n259 are proposed as shown in Table 1.

Proposal 2: Update tables for step frequencies for Tx spurious emissions (Category B) and Rx spurious emissions (Category A/B) to support Band n259.

Table 1: Step frequencies for defining the BS radiated transmitter and receiver spurious emission limits in FR2

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |

2.3	CR

The above changes are proposed in our CRs to TS 38.104 and TS 38.141-2 [2, 3].

Proposal 3: Agree on CRs to TS 38.104 and TS 38.141-2 for updating tables for ΔfOBUE and ΔfOOB and step frequencies for Band n259

3.	Conclusion

In this contribution, we further analyse the necessity to update TS 38.104 and TS 38.141-2 for the introduction of n259.

- ΔfOBUE and ΔfOOB

Proposal 1: Update tables for ΔfOBUE and ΔfOOB for FR2 to support Band n259.

- Step frequency for spurious emissions

Proposal 2: Update tables for step frequencies for Tx spurious emissions (Category B) and Rx spurious emissions (Category A/B) to support Band n259.

Table 1: Step frequencies for defining the BS radiated transmitter and receiver spurious emission limits in FR2

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |

Proposal 3: Agree on CRs to TS 38.104 and TS 38.141-2 for updating tables for ΔfOBUE and ΔfOOB and step frequencies for Band n259

References

- R4-1912448, “BS RF requirement and test tolerance for n259”, NTT DOCOMO, INC., RAN4 #92bis, October 2019

- R4-1914085, “CR for TS 38.104: Update the tables for in-band and out-of-band boundary and step frequencies for band n259”, NTT DOCOMO, INC., RAN4 #93, November 2019

- R4-1914086, “CR for TS 38.141-2: Update the tables for in-band and out-of-band boundary and step frequencies for band n259”, NTT DOCOMO, INC., RAN4 #93, November 2019

- R4-1913030, “Way forward on NR Rx spurious emissions,” Ericsson, NTT DOCOMO, RAN4 #92bis, October 2019