3GPP TSG-RAN WG4 Meeting #93	R4-1914103

Reno, US, 18th– 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0044 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.141-2: Introduction of NR PRACH performance requirements for restricted set type A and B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2019-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce PRACH requirements for restricted set type A and B under conditions assuming a UE velocity of up to 350km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new test cases of PRACH for restricted set type A and B assuming a UE velocity up to 350km/h |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRACH performance for under high speed conditions assuming a UE velocity of up to 350km/h is not ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.4.2, 8.4.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.104 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.141-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.4.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.4.1.4.2-1.

Table 8.4.1.4.2-1: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -80.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -77.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -80.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -77.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -74.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -70.1 - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 | 50 | [EISREFSENS_50M + 12 dBm / 47.52 MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  | 120 | 50 | [EISREFSENS_50M + 12 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + 18 dBm / 190.08 MHz] |

8)	Adjust the frequency offset of the test signal according to table 8.4.1.5.1-1 or 8.4.1.5.1-2 or 8.4.1.5.1-3 or 8.4.1.5.1-4 or 8.4.1.5.1-5 or 8.4.1.5.2-1 or 8.4.1.5.2-2.

9)	Adjust the equipment so that the SNR specified in table 8.4.1.5.1-1 or 8.4.1.5.1-2 or 8.4.1.5.1-3 or 8.4.1.5.1-4 or 8.4.1.5.1-5 or 8.4.1.5.2-1 or 8.4.1.5.2-2 is achieved at the BS input during the PRACH preambles.

10) The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 8.4.1.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 8.4.1.4.2-1: PRACH preamble test pattern

The timing offset base value for PRACH preamble format 0 is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.9us. Then the loop is being reset and the timing offset is set again to 50% of Ncs. The timing offset scheme for PRACH preamble format 0 is presented in Figure 8.4.1.4.2-2.

Figure 8.4.1.4.2-2: Timing offset scheme for PRACH preamble format 0

The timing offset base value for PRACH preamble format A1, A2, A3, B4, C0 and C2 is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8us. Then the loop is being reset and the timing offset is set again to 0. The timing offset scheme for PRACH preamble format A1, A2, A3, B4, C0 and C2 is presented in Figure 8.4.1.4.2-3.

Figure 8.4.1.4.2-3: Timing offset scheme for PRACH preamble format A1 A2, A3, B4, C0 and C2

8.4.1.5	Test requirement

8.4.1.5.1	Test requirement for BS type 1-O

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in tables 8.4.1.5.1-1 to 8.4.1.5.1-3.

Table 8.4.1.5.1-1: PRACH missed detection test requirements for Normal Mode, 1.25 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | [-14.2] |
|  |  | TDLC300-100 Low | 400 Hz | [-6.0] |

Table 8.4.1.5.1-2: PRACH missed detection test requirements for Normal Mode, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-9.0] | [-12.3] | [-13.9] | [-16.5] | [-6.0] | [-12.2] |
|  |  | TDLC300-100 Low | 400 Hz | [-1.5] | [-4.2] | [-6.0] | [-8.2] | [1.4] | [-4.3] |

Table 8.4.1.5.1-3: PRACH missed detection test requirements for Normal Mode, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-8.8] | [-11.7] | [-13.5] | [-16.2] | [-5.8] | [-11.6] |
|  |  | TDLC300-100 Low | 400 Hz | [-2.2] | [-5.1] | [-6.8] | [-9.3] | [0.7] | [-5.0] |

Table 8.4.1.5.1-4: PRACH missed detection test requirements for High speed Mode restricted set type A, 1.25 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |  |
| 1 | 2 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |

Table 8.4.1.5.1-5: PRACH missed detection test requirements for High speed Mode restricted set type B, 1.25 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | [2334] Hz | TBD |

--------------End of text proposal-------------