3GPP TSG-RAN WG4 Meeting #93	R4-1914351

Reno, NV, USA, 18 – 22 November 2019

Agenda Item:	9.17.1.4

Source:	Ericsson

Title:	Beam management for high speed train scenario

Document for:	Discussion

# 1	Introduction

RAN4#92bis agreed with the way forward on RRM for NR high speed train scenario [1], which also includes the agreement on beam management as follow:

SSB based Beam management - Option 1: Rel-15 beam management requirements (L1-RSRP, CBD, BFD) can be reused for NR HST - Other options are not precluded CSI-RS based Beam management - Option 1: Rel-15 beam management requirements (L1-RSRP, CBD, BFD) can be reused for NR HST - Other options are not precluded

In this contribution we discuss the impact of L1-RSRP measurements for reporting under high speed trains scenario.

# 2	Beam failure detection

In Rel-15, the evaluation period of the beam failure detection is specified based on the evaluation period of RLM in-synch. As we proposed in [2], we think RAN4 can reuse the Rel-15 RLM requirement for NR HST. We therefore propose RAN4 reuses the Rel-15 beam failure detection requirement for NR HST.

Proposal 1: Rel-15 Beam failure detection requirements can be reused for NR HST.

# 3	L1-RSRP measurements

If we consider 500km/h at the carrier frequency of 3.6GHz, the maximum Doppler frequency becomes 1666Hz. We studied the L1-RSRP measurement accuracy based on the parameters used to set the L1-RSRP measurement and accuracy requirements in Rel-15 [3].

## 3.1	SSB based L1-RSRP measurements

Table 1 shows the simulation parameters for SSB based L1-RSRP measurements in HST scenario. We also investigated 1, 3, and 5 SSB samples for HST condition. Note Rel-15 assumed the L1-RSRP measurement was based on 1 SSB sample [3].

Table 1	Simulation parameters for SSB based L1-RSRP measurements in HST scenario.

| Parameters | Values |
| --- | --- |
| SCS | 15kHz, 30kHz |
| Side condition (SNR) | -3dB |
| Number of samples (M) | 1 (Baseline), 3, 5 |
| Propagation condition | TDLA30, TDLC300 |
| Maximum Doppler frequency | 1000Hz (=300km/h at 3.6GHz), 1666Hz (=300km/h at 3.6GHz) |

Table 2	SSB based L1-RSRP estimation simulation results.

| SCS | Samples | AWGN [4] | TDLA30-1000 | TDLA30-1666 | TDLC300-1000 | TDLC300-1666 |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 1.2 (Rel-15) | 1.39 | 1.53 | 1.44 | 1.48 |
|  | 3 |  | 0.71 | 0.71 | 0.79 | 0.78 |
|  | 5 |  | 0.54 | 0.54 | 0.55 | 0.54 |
| 30kHz | 1 | 0.8 (Rel-15) | 0.88 | 0.9 | 0.97 | 0.99 |
|  | 3 |  | 0.46 | 0.49 | 0.58 | 0.58 |
|  | 5 |  | 0.36 | 0.4 | 0.42 | 0.43 |

Table 2 shows our simulation results of the SSB based L1-RSRP estimation. We put the maximum of absolute value of 5%-ile error and 95%-ile error values as the same metric as [4]. For reference we put our results in the static condition, which is used to specify the L1-RSRP measurement accuracy requirements in Rel-15.

If we compare the fading case (Doppler=1000Hz or 1666Hz) with AWGN case, we don’t observe the accuracy degradation due to the fading channel and/or high Doppler frequency.

Observation 1: SSB based L1-RSRP measurement accuracy is not impacted with the high speed scenario.

Proposal 2: Rel-15 SSB based L1-RSRP measurement/accuracy requirements can be reused for NR HST.

## 3.2	CSI-RS based L1-RSRP measurements

Table 3 shows the simulation parameters for CSI-RS based L1-RSRP measurements in HST scenario. Like SSB-based L1-RSRP, we investigated 1, 3, and 5 CSI-RS samples and larger BW for HST scenario. Note Rel-15 assumed 1 CSI-RS sample with 48PRBs [3].

Table 3	Simulation parameters for CSI-RS based L1-RSRP measurements in HST scenario.

| Parameters | Values |
| --- | --- |
| SCS | 15kHz, 30kHz |
| Side condition (SNR) | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1 (Baseline), 3, 5 |
| Number of PRBs | 48 (Baseline), 96 |
| Propagation condition | TDLA30, TDLC300 |
| Maximum Doppler frequency | 1000Hz, 1666Hz |

Table 4	CSI-RS based L1-RSRP estimation simulation results.

| SCS | Samples | PRB | AWGN [4] | TDLA30-1000 | TDLA30-1666 | TDLC300-1000 | TDLC300-1666 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 48 | 0.77 (Rel15) | 1.78 | 1.77 | 1.77 | 1.67 |
|  |  | 96 |  | 0.98 | 0.97 | 0.91 | 0.87 |
|  | 3 | 48 |  | 0.83 | 0.83 | 0.9 | 0.93 |
|  |  | 96 |  | 0.48 | 0.52 | 0.48 | 0.5 |
|  | 5 | 48 |  | 0.62 | 0.64 | 0.79 | 0.73 |
|  |  | 96 |  | 0.41 | 0.37 | 0.37 | 0.38 |
| 30kHz | 1 | 48 | 0.55 (Rel15) | 1.01 | 1.16 | 1.18 | 1.26 |
|  |  | 96 |  | 0.59 | 0.65 | 0.59 | 0.62 |
|  | 3 | 48 |  | 0.54 | 0.57 | 0.66 | 0.7 |
|  |  | 96 |  | 0.34 | 0.35 | 0.35 | 0.38 |
|  | 5 | 48 |  | 0.42 | 0.46 | 0.52 | 0.54 |
|  |  | 96 |  | 0.29 | 0.3 | 0.27 | 0.27 |

Table 4 shows our simulation results of the CSI-RS based L1-RSRP estimation. We set the same metric as SSB based L1-RSRP. Unlike SSB-based L1-RSRP results, it is observed the measurement accuracy degradation due to fading and/or high Doppler frequency, i.e., from 0.77dB to more than 1.67dB in the case of SCS=15kHz.

Observation 2: CSI-RS based L1-RSRP accuracy is degraded significantly due to the high speed scenario.

It is also observed that the increase of CSI-RS samples in time domain and/or frequency domain can maintain the same accuracy. However, more CSI-RS samples in time domain is not good solution because UE is moving very fast and the network needs the latest beam information to select the best beam. We don’t want to assume wider bandwidth for CSI-RS also because it restricts the network deployment.

Proposal 3: RAN4 study the impact of CSI-RS based L1-RSRP measurement requirements due to the HST scenario.

# 4	Candidate beam detection

Candidate beam detection (CBD) is the procedure UE detects new beam(s) whose L1-RSRP exceeds the signalled threshold. Table 5 and Table 6 show the evaluation periods for FR1 SSB based CBD and FR1 CSI-RS based CBD, respectively, specified in Rel-15. As it is observed, the L1-RSRP measurements for CBD is based on 3 samples for both SSB and CSI-RS. According to our simulation results in Table 2 and Table 4, we don’t observe the accuracy degradation due to high Doppler shift in the case of 3 samples. Therefore we propose Rel-15 Candidate beam detection requirements can be reused for NR HST.

Table 5	Evaluation period for FR1 SSB based CBD in Rel-15.

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Ceil([3]*P) * TSSB |
| DRX cycle > 320ms | Ceil([3]*P) * TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 6	Evaluation period for FR1 CSI-RS based CBD in Rel-15.

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max([25], Ceil(MCBD *P) * TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD *P) *TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Proposal 4: Rel-15 Candidate beam detection requirements can be reused for NR HST.

# 5	Summary

Proposal 1: Rel-15 Beam failure detection requirements can be reused for NR HST.

Observation 1: SSB based L1-RSRP measurement accuracy is not impacted with the high speed scenario.

Proposal 2: Rel-15 SSB based L1-RSRP measurement/accuracy requirements can be reused for NR HST.

Observation 2: CSI-RS based L1-RSRP accuracy is degraded significantly due to the high speed scenario.

Proposal 3: RAN4 study the impact of CSI-RS based L1-RSRP measurement requirements due to the HST scenario.

Proposal 4: Rel-15 Candidate beam detection requirements can be reused for NR HST.

# References

- R4-1912781, “WF on RRM for NR HST”, CMCC.

- R4-1914350, “RLM for NR HST”, Ericsson.

- R4-1904760, “Simulation summary of L1-RSRP accuracy”, Ericsson.

- R4-1904820, “Way forward on L1-RSRP accuracy requirements”, Ericsson, Nokia, CMCC.