3GPP TSG-RAN WG4 Meeting #93	R4-1914547

Reno, Nevada, US, 18 – 22 November 2019

Source:	Ericsson

Title:	Initial simulation results for Rel-16 NR HST and UL timing adjustment PUSCH demodulation performance at UE speed of 350 km/h

Agenda item:	9.17.2.3.1

Document for:	Discussion

# Introduction

In this contribution, we presented our initial simulation results for NR HST PUSCH demodulation performance for both HST scenarios and UL timing adjustment for HST speed = 350 km/h.

# Initial simulation assumptions and simulation results

In this section, our presented our simulation assumptions based on [1] in Table 1 for HST scenarios and the corresponding ideal simulation results and results with impairments for HST scenarios are given in Table 2 and Table 3.

Correspondingly, the simulation assumption for UL timing adjustment are given in Table 4 and the results are provided in Table 5 and Table 6.

HST Scenarios

| Parameters | Values |
| --- | --- |
|  | FR1 |
| Waveform | CP-OFDM |
| Number of Tx and Rx | Tunnel scenario (Dmin = 2 m, Ds = 300 m): 1Tx1Rx and 1Tx 2Rx Open space scenario (Dmin = 150 m, Ds = 700 m): 1Tx2Rx and 1Tx 8Rx |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 DM-RS symbols with l0 = 2 and l0 = 3 |
| Number of symbols | 14 |
| Time domain resource allocation type | type A |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 2, 16 |
| Carrier frequency (GHz) | fc = 2.1 GHz for SCS = 15 kHz fc = 3.6 GHz for SCS = 30 kHz |
| Propagation channel | HST tunnel channel model: Ds = 300 m and Dmin = 2 m HST open space channel model: Ds = 700 m and Dmin = 150 m |
| SCS and BW | 15kHz: 10 MHz 30kHz: 40 MHz |
| Speed (v) | 350 km / h |
| Maximum Doppler shift (fd) | 15 kHz: 1340 Hz 30 kHz: 2334 Hz |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for HST scenarios at 350 km/h.

Table 2. Ideal simulation results for NR HST PUSCH demodulation performance.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 10 MHz | 2 | -3,0 |
|  |  | 16 | 9,7 |
|  | 30 kHz / 40 MHz | 2 | -3,0 |
|  |  | 16 | 9,5 |
| Tunnel (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -6,0 |
|  |  | 16 | 6,6 |
|  | 30 kHz / 40 MHz | 2 | -6,0 |
|  |  | 16 | 6,5 |
| Open space (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -6,1 |
|  |  | 16 | 6,3 |
|  | 30 kHz / 40 MHz | 2 | -6,0 |
|  |  | 16 | 6,2 |
| Open space (1Tx8Rx) | 15 kHz / 10 MHz | 2 | -12 |
|  |  | 16 | 0,3 |
|  | 30 kHz / 40 MHz | 2 | -12,0 |
|  |  | 16 | 0,3 |

Table 3. Results with impairments for NR HST PUSCH demodulation performance.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 10 MHz | 2 | -0,9 |
|  |  | 16 | 12,1 |
|  | 30 kHz / 40 MHz | 2 | -1,0 |
|  |  | 16 | 11,7 |
| Tunnel (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -3,6 |
|  |  | 16 | 9,1 |
|  | 30 kHz / 40 MHz | 2 | -3,6 |
|  |  | 16 | 9,0 |
| Open space (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -4,0 |
|  |  | 16 | 8,5 |
|  | 30 kHz / 40 MHz | 2 | -3,6 |
|  |  | 16 | 8,5 |
| Open space (1Tx8Rx) | 15 kHz / 10 MHz | 2 | -9,9 |
|  |  | 16 | 2,6 |
|  | 30 kHz / 40 MHz | 2 | -9,5 |
|  |  | 16 | 2,4 |

UL Timing Adjustment

| Parameters | Values |
| --- | --- |
|  | FR1 – Scenario Y |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| Number of Tx and Rx | 1Tx 2Rx |
| Number of DMRS symbols | (1+1+1) DM-RS symbols |
| (A, Δω) | (10 µs, 0.13 s-1) |
| System bandwidth | SCS = 15 kHz: 10MHz SCS = 30 kHz: 40 MHz |
| Frequency domain resource | SCS = 15 kHz: 24 PRBs for each UE SCS = 30 kHz: 48 PRBs for each UE |
| MCS index | 2, 16 |
| Testing metric | SNR @70% of maximum throughput |

Table 4. Simulation assumptions for UL timing adjustment at 350 km/h (based on [1] and e-mail

discussion).

Table 5. Ideal simulation results for NR UL timing adjustment PUSCH demodulation performance.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Y | 15 kHz / 10 MHz | 2 | -6,0 |
|  |  | 16 | 6,0 |
|  | 30 kHz / 40 MHz | 2 | -6,1 |
|  |  | 16 | 6,0 |

Table 6. Results with impairments for NR UL timing adjustment PUSCH demodulation performance.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Y | 15 kHz / 10 MHz | 2 | -4,0 |
|  |  | 16 | 8,2 |
|  | 30 kHz / 40 MHz | 2 | -3,9 |
|  |  | 16 | 8,3 |

# Conclusion

In this contribution, we summarized our initial simulation results for NR HST and UL timing adjustment PUSCH demodulation performance at UE speed of 350 km/h.

Note that our opinions on the necessity of testing with 1Tx1Rx for the HST tunnel scenario and 1Tx 8Rx for the HST open space scenario are given in our companion contribution [2].

# References

R4-1912809, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon.

R4-1914544, On PUSCH antenna configurations, Ericsson.