3GPP TSG-RAN WG4 Meeting #93	R4-1914554

Reno, US, 18 – 22 November 2019

Source:	Ericsson

Title:	Simulation results for PRACH HST with 500km/hour

Agenda item:	9.17.2.3.2

Document for:	Discussion

# Introduction

During the meeting RAN4#92bis, some agreements [1] on the PRACH high speed train scenario with velocity at 500km/h were achieved, where part of them are listed here.

- PRACH format: for 500km/h velocity, use PRACH format A2/B4/C2

- Frequency offset for PRACH formats with short sequence length targeting 500km/h

  - Align with PUSCH maximum Doppler shift

Although the maximum Doppler shift for PUSCH has not been aligned yet, the options are down selected into the following ones, according to [2].

- Maximum Doppler shift for HST 500km/h

  - SCS: 15kHz

    - Option 1: 1944Hz

    - Option 2: 1750Hz

  - SCS: 30kHz

    - Option 1: 3334Hz

    - Option 2: 3000Hz

In this contribution, we will provide the simulation results for PRACH high speed train case when the velocity is 500km/h.

# Discussion

Apart from the above simulation parameters, some other implicit assumptions are provided as below,

- Carrier frequency for velocity at 500km/h

  - 2.1GHz (SCS=15kHz)

  - 3.6GHz (SCS=30kHz)

- Ncs:

  - 15kHz SCS: 23

  - 30kHz SCS: 46

- Logical root sequence: 0

- Preamble index: 0

- Total number of PRACH preambles: 64

- Number of transmitter antennas: 1 Tx

- Number of receiver antennas: 2 Rx, 4 Rx, 8 Rx

- Test metric

  - False alarm probability: 0.1%

  - Missed detection probability: 1%

The simulation results are included in the following tables.

Table 1. ideal simulation results for 2Rx PRACH format A2/B4/C2 in AWGN

| Frequency offset (Hz) | SNR (dB) for B4 | SNR (dB) for A2 | SNR (dB) for C2 |
| --- | --- | --- | --- |
| 1944 | -16,3 | -13,6 | -13,6 |
| 1750 | -16,5 | -13,8 | -13,8 |
| 3334 | -16,7 | -13,7 | -13,6 |
| 3000 | -17,0 | -13,8 | -13,8 |

Table 2. ideal simulation results for 4Rx PRACH format A2/B4/C2 in AWGN

| Frequency offset (Hz) | SNR (dB) for B4 | SNR (dB) for A2 | SNR (dB) for C2 |
| --- | --- | --- | --- |
| 1944 | -18,5 | -16,0 | -16,0 |
| 1750 | -19,0 | -16,2 | -16,2 |
| 3334 | -19,0 | -16,1 | -16,1 |
| 3000 | -19,1 | -16,3 | -16,3 |

Table 3. ideal simulation results for 8Rx PRACH format A2/B4/C2 in AWGN

| Frequency offset (Hz) | SNR (dB) for B4 | SNR (dB) for A2 | SNR (dB) for C2 |
| --- | --- | --- | --- |
| 1944 | -20,5 | -18,2 | -18,2 |
| 1750 | -21,1 | -18,3 | -18,3 |
| 3334 | -21,1 | -18,3 | -18,3 |
| 3000 | -21,2 | -18,3 | -18,4 |

Table 4. impaired results for 2Rx PRACH format A2/B4/C2 in AWGN

| Frequency offset (Hz) | SNR (dB) for B4 | SNR (dB) for A2 | SNR (dB) for C2 |
| --- | --- | --- | --- |
| 1944 | -13,8 | -11,4 | -11,2 |
| 1750 | -14,0 | -11,6 | -11,5 |
| 3334 | -14,4 | -11,5 | -11,3 |
| 3000 | -14,6 | -11,7 | -11,3 |

Table 5. impaired results for 4Rx PRACH format A2/B4/C2 in AWGN

| Frequency offset (Hz) | SNR (dB) for B4 | SNR (dB) for A2 | SNR (dB) for C2 |
| --- | --- | --- | --- |
| 1944 | -16,0 | -13,8 | -13,7 |
| 1750 | -16,8 | -13,7 | -14,0 |
| 3334 | -16,6 | -13,7 | -13,7 |
| 3000 | -17,0 | -13,8 | -13,8 |

Table 6. impaired results for 8Rx PRACH format A2/B4/C2 in AWGN

| Frequency offset (Hz) | SNR (dB) for B4 | SNR (dB) for A2 | SNR (dB) for C2 |
| --- | --- | --- | --- |
| 1944 | -18,2 | -15,9 | -16,1 |
| 1750 | -18,7 | -16,2 | -16,3 |
| 3334 | -18,7 | -15,9 | -16,3 |
| 3000 | -19,0 | -16,3 | -16,0 |

# Conclusion

Simulation results for PRACH HST scenario with format A2/B4/C2, when velocity is 500km/h, are provided in Section 2.

# Reference

| [1] | Samsung, R4-1912729, Way forward on NR HST PRACH, Chongqing, China, October 14-18, 2019. |
| --- | --- |
| [2] | Huawei, R4-1912809, Way forward on NR HST PUSCH demodulation requirements, Chongqing, China, October 14-18, 2019. |