3GPP TSG-RAN WG4 Meeting #93	R4-1914705

Reno, Nevada, USA, 18 November – 22 November 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-11-11: DC_7_n7

Agenda item:			10.3.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 1 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-11-11 to include DC_7_n7.

# 2. Text Proposal

# ---Start of changes---

## 6.4.x	DC_7A_n7A

#### 6.4.x.1	Channel bandwidths per operating band for CA

Table 6.4.x.1-1: Band combinations for intra-band non-contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7_n7 | 7 | n7 | Yes |

Table 6.4.x.1-2: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_7A_n7A | DC_7A_n7A1 | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30, 40 |  | 50 | 0 |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |

#### 6.4.x.2	Configuration for DC

Table 6.4.x.2-1:  Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A_n7A | DC_7A_n7A2 | 7A | n7A |
| NOTE 2: Only single switched UL is supported |  |  |  |

#### 6.4.x.3	Maximum output power for DC

Table 6.4.x.3-1: Maximum output power for  Intra-band non-contiguous EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_7A_n7A2 |  |  | 23 | +2/-3 |
| NOTE 2: Only single switched UL is supported |  |  |  |  |

#### 6.4.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no spurious emission band UE co-existence requirement is needed

#### 6.4.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed

#### 6.4.x.6	∆RIBNC values

The delta R IBNC values for intra-band non-contiguous EN-DC DC_7_n7 are shown in Table 6.4.x.6-1.

Table 6.4.x.6-1: Intra-band non-contiguous EN-DC with one uplink configuration for reference sensitivity

| DC configuration | Aggregated channel bandwidth (LTE+NR) | Wgap / (MHz) | UL LTE allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| DC_7A_n7A | NOTE 4 | NOTE 8 | NOTE 9 | 0 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The table only applies when the center frequency of LTE carrier is higher than the NR carrier, and the ΔRIBNC applies to the NR DL carrier only. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. NOTE 8: All applicable sub-block gap sizes. NOTE 9: The UL LTE allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1 in [4]. |  |  |  |  |  |

#### 6.4.x.7	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed

---End of changes---

# Reference

[1]		RP-191752, “Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)”, NTT DOCOMO, INC.