3GPP TSG-RAN WG4 Meeting #93	R4-1914769

Reno, US, 18 November – 22 November 2019

Source:	ZTE

Title:	Remaining issues on NR reporting criteria

Agenda item:	7.10.3

Document for:	Discussion

1. Introduction

In the RAN4#91 meeting, there were changes on reporting criteria requirements for NR. However the changes bring some issues on measurement reporting configuration on NR serving frequencies. Furthermore requirements for NE-DC reporting criteria has not been finalized. In the RAN4#92 and RAN4#92bis meetings, the remaining issues were discussed in [1, 2].

In this contribution, we further provide our views on remaining issues on reporting criteria for EN-DC and NE-DC.

2. Discussion

2.1. EN-DC reporting criteria

In the last meeting, RAN4 reached common understanding on reporting criteria for measurements on NR serving cells configured by E-UTRA PCell, which is copied as follows.

Agreement: The common understanding in RAN4 group is the reporting criterion for measurement on NR serving cells configured by E-UTRA PCell is calculated in the Ecat,EN-DC,NR.

The reporting criteria for measurement on NR serving cell carrier frequencies configured by E-UTRA PCell is specified in TS38.133 according to the above agreements. However in TS36.133 it states that reporting criteria includes measurements on NR serving cells. The reporting criteria in current spec is not aligned with RAN4 common understanding. So measurements on NR serving cells should be removed from TS36.133 and clarification that Ecat,EN-DC,NR  can be configured by E-UTRA PCell and PSCell should be added in TS38.133.

Proposal 1. Measurements on NR serving cells should be removed from TS36.133 and clarification that Ecat,EN-DC,NR  can be configured by E-UTRA PCell and and PSCell should be added in TS38.133.

There is no reporting criteria in TS 36.133 for EN-DC when E-UTRA SCell(s) are configured. It is not clear what the E-UTRA reporting criteria would be when SCell(s) is configured. It is necessary to add corresponding requirements for this scenario. Similar requirements as in TS 38.133 for serving carrier frequencies, i.e. scaled by the number of serving carrier frequencies, can be specified.

Proposal 2. The E-UTRA reporting criteria for EN-DC when E-UTRA SCell(s) are configured is necessary to be specified.

Proposal 3. Reporting criteria for EN-DC is 36+9*n when the UE is configured with E-UTRA SCell(s), and n is the number of E-UTRA SCell carrier frequencies.

2.1. NE-DC reporting criteria

The structure of reporting criteria requirements for NE-DC is quite similar to EN-DC. The reporting criteria is separated for measurement categories on NR carrier frequencies and E-UTRA carrier frequencies. Further reporting criteria for E-UTRA carrier frequencies is separated for serving carrier frequencies and non-serving carrier frequencies.

- For UE configured with NE-DC: Ecat,NE-DC,NR+Ecat,NE-DC,E-UTRA, where Ecat,NE-DC,NR=10+9×n is the total number of NR reporting criteria according to Table 9.1.4.2-1, and is the number of configured NR serving frequencies, including PCell and SCells carrier frequencies, Ecat,NE-DC,E-UTRA=Ecat,NE-DC,E-UTRA,inter-RAT+Ecat,NE-DC,E-UTRA,intra-RAT, where Ecat,NE-DC,E-UTRA,inter-RAT is the total number of inter-RAT E-UTRA reporting criteria configured by PCell except E-UTRA PSCell and E-UTRA SCell carrier frequencies, according to Table 9.1.4.2-1, Ecat,NE-DC,E-UTRA,intra-RAT is the total number of E-UTRA reporting criteria including E-UTRA PSCell and SCell carrier frequencies as specified in TS 36.133 [15] for UE configured with NE-DC.

Since measurements on NR carrier frequencies can only be configured by NR PCell, the current reporting criteria for measurements on NR carrier frequencies is fine when UE is in NE-DC operation. For reporting criteria for measurements on E-UTRA carrier frequencies, the number is not specified yet.

The UE capable of supporting and configured with NE-DC operation with PSCell and NR PCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells and E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with NE-DC operation, as follows: - [TBD] reporting criteria if the UE is not configured with any SCell or NR SCell.

Regarding the number of reporting criteria for E-UTRA carriers specified in TS36.133 we propose to use similar approach as in TS 38.133 for NR carriers which makes the spec more consistently. The requirements should be specified for both single carrier and CA case.

Proposal 4. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and  is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies.

3. Conclusion

In this contribution, we provide our views on remaining issues on reporting criteria for NR. Based on the observations following proposals are present.

Proposal 1. The reporting criteria requirements for EN-DC in TS36.133 should be revised by removing Proposal 1. Measurements on NR serving cells should be removed from TS36.133 and clarification that Ecat,EN-DC,NR  can be configured by E-UTRA PCell and and PSCell should be added in TS38.133.

Proposal 2. The E-UTRA reporting criteria for EN-DC when E-UTRA SCell(s) are configured is necessary to be specified.

Proposal 3. Reporting criteria for EN-DC is 36+9*n when the UE is configured with E-UTRA SCell(s), and n is the number of E-UTRA SCell carrier frequencies.

Proposal 4. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and  is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies.

4. References

- R4-1909535	Remaining issues on NR reporting criteria, ZTE

- R4-1911556 Remaining issues on NR reporting criteria, ZTE