3GPP TSG-RAN WG4 Meeting #93	R4-1914837

Reno, Nevada, USA, Nov 18th – 22nd, 2019

Title: 	Simulation results of CSI-RSRP measurements for NR mobility

Source: 	Huawei, HiSilicon

Agenda item:	9.16.1.1

Document for:	Discussion

Introduction

RAN4 needs to study the CSI-RS measurement requirements for L3 mobility, including measurement period, CSI-RSRP measurement accuracy and the corresponding side condition. Based on the simulation assumptions in RAN4 email discussion, we provide the simulation results of CSI-RS based measurements in NR.

Discussion

In this section, we provide the simulation results of CSI-RSRP measurement accuracy with different measurement bandwidth. The results in Tables 1-2 show 5%, 50% and 95% CDF of the delta measurements of CSI-RSRP.

Table 1: Delta measurements of measured CSI-RSRP (24RBs)

| SCS=15kHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -2.80 | -0.29 | 1.47 | 2.51 | 1.76 | -1.34 | -0.08 | 1.00 | 1.26 | 1.08 |
|  | 5 | -2.16 | -0.36 | 1.11 | 1.80 | 1.48 | -1.13 | -0.06 | 0.76 | 1.08 | 1.82 |
|  | 10 | -1.49 | -0.24 | 0.73 | 1.25 | 0.97 | -0.81 | -0.13 | 0.50 | 0.68 | 0.63 |
| EPA5 | 3 | -3.38 | -0.32 | 1.63 | 3.06 | 1.95 | -1.68 | -0.11 | 1.05 | 1.57 | 1.16 |
|  | 5 | -2.45 | -0.23 | 1.30 | 2.22 | 1.53 | -1.25 | -0.13 | 0.84 | 1.12 | 0.97 |
|  | 10 | -1.55 | -0.33 | 0.79 | 1.22 | 1.12 | -0.85 | -0.06 | 0.57 | 0.79 | 0.63 |
| ETU30 | 3 | -5.03 | -1.42 | 0.77 | 3.61 | 2.19 | -2.19 | -0.60 | 0.47 | 1.59 | 1.07 |
|  | 5 | -3.86 | -1.33 | 0.28 | 2.53 | 1.61 | -1.75 | -0.54 | 0.39 | 1.21 | 0.93 |
|  | 10 | -2.70 | -1.27 | -0.09 | 1.43 | 1.18 | -1.30 | -0.54 | 0.14 | 0.76 | 0.68 |
| ETU70 | 3 | -4.87 | -1.21 | 0.75 | 3.66 | 1.96 | -1.99 | -0.60 | 0.66 | 1.39 | 1.26 |
|  | 5 | -3.84 | -1.34 | 0.50 | 2.50 | 1.84 | -1.81 | -0.53 | 0.37 | 1.28 | 0.90 |
|  | 10 | -2.88 | -1.24 | -0.01 | 1.64 | 1.23 | -1.30 | -0.54 | 0.11 | 0.76 | 0.65 |
| SCS=30KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -2.79 | -0.31 | 1.39 | 2.48 | 1.71 | -1.45 | -0.09 | 0.89 | 1.36 | 0.98 |
|  | 5 | -2.13 | -0.24 | 1.15 | 1.89 | 1.39 | -1.04 | -0.09 | 0.70 | 0.95 | 0.79 |
|  | 10 | -1.53 | -0.25 | 0.73 | 1.28 | 0.97 | -0.76 | -0.09 | 0.52 | 0.67 | 0.61 |
| EPA5 | 3 | -3.80 | -0.38 | 1.54 | 3.42 | 1.92 | -1.74 | -0.12 | 1.06 | 1.62 | 1.18 |
|  | 5 | -2.47 | -0.27 | 1.27 | 2.20 | 1.54 | -1.27 | -0.07 | 0.84 | 1.10 | 0.91 |
|  | 10 | -1.61 | -0.26 | 0.77 | 1.35 | 1.03 | -0.84 | -0.11 | 0.60 | 0.73 | 0.71 |
| SCS= 60KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -2.85 | -0.23 | 1.47 | 2.61 | 1.71 | -1.39 | -0.13 | 0.96 | 1.26 | 1.09 |
|  | 5 | -2.16 | -0.32 | 1.08 | 1.84 | 1.41 | -1.14 | -0.08 | 0.78 | 1.06 | 0.86 |
|  | 10 | -1.49 | -0.29 | 0.80 | 1.20 | 1.09 | -0.76 | -0.09 | 0.53 | 0.67 | 0.62 |
| EPA5 | 3 | -3.26 | -0.39 | 1.49 | 2.87 | 1.88 | -1.47 | -0.10 | 0.98 | 1.37 | 1.08 |
|  | 5 | -2.15 | -0.34 | 1.16 | 1.81 | 1.50 | -1.18 | -0.14 | 0.77 | 1.04 | 0.91 |
|  | 10 | -1.52 | -0.29 | 0.76 | 1.23 | 1.05 | -0.77 | -0.11 | 0.56 | 0.66 | 0.67 |
| TDL-A 83Hz | 3 | -3.21 | -0.30 | 1.56 | 2.91 | 1.86 | -1.55 | -0.14 | 1.03 | 1.41 | 1.17 |
|  | 5 | -2.27 | -0.32 | 1.25 | 1.95 | 1.57 | -1.16 | -0.12 | 0.74 | 1.04 | 0.86 |
|  | 10 | -1.49 | -0.32 | 0.77 | 1.17 | 1.09 | -0.76 | -0.07 | 0.58 | 0.69 | 0.65 |
| TDL-C 83Hz | 3 | -3.20 | -0.46 | 1.33 | 2.74 | 1.79 | -1.42 | -0.11 | 0.97 | 1.31 | 1.08 |
|  | 5 | -2.32 | -0.35 | 1.10 | 1.96 | 1.45 | -1.10 | -0.07 | 0.84 | 1.03 | 0.91 |
|  | 10 | -1.68 | -0.35 | 0.64 | 1.33 | 0.99 | -0.78 | -0.09 | 0.55 | 0.69 | 0.64 |
| SCS= 120KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| TDL-A 83Hz | 3 | -3.88 | -0.76 | 1.27 | 3.12 | 2.03 | -1.61 | -0.09 | 0.95 | 1.52 | 1.04 |
|  | 5 | -2.78 | -0.70 | 0.73 | 2.08 | 1.43 | -1.12 | -0.12 | 0.69 | 1.00 | 0.81 |
|  | 10 | -2.09 | -0.68 | 0.36 | 1.41 | 1.04 | -0.79 | -0.08 | 0.58 | 0.71 | 0.66 |
| TDL-C 83Hz | 3 | -3.37 | -0.48 | 1.34 | 2.89 | 1.81 | -1.52 | -0.11 | 1.05 | 1.41 | 1.16 |
|  | 5 | -2.56 | -0.58 | 0.93 | 1.98 | 1.51 | -1.12 | -0.08 | 0.76 | 1.04 | 0.84 |
|  | 10 | -1.83 | -0.47 | 0.58 | 1.36 | 1.05 | -0.77 | -0.09 | 0.55 | 0.68 | 0.64 |

Table 2: Delta measurements of measured CSI-RSRP (48RBs)

| SCS=15kHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -1.85 | -0.17 | 1.15 | 1.67 | 1.32 | -1.00 | -0.07 | 0.73 | 0.93 | 0.80 |
|  | 5 | -1.38 | -0.15 | 0.87 | 1.23 | 1.02 | -0.68 | -0.04 | 0.57 | 0.64 | 0.61 |
|  | 10 | -1.00 | -0.14 | 0.60 | 0.86 | 0.74 | -0.51 | -0.05 | 0.35 | 0.46 | 0.40 |
| EPA5 | 3 | -2.10 | -0.17 | 1.29 | 1.93 | 1.46 | -1.14 | -0.06 | 0.79 | 1.08 | 0.85 |
|  | 5 | -1.48 | -0.15 | 0.95 | 1.33 | 1.10 | -0.77 | -0.05 | 0.62 | 0.72 | 0.67 |
|  | 10 | -1.06 | -0.16 | 0.63 | 0.90 | 0.79 | -0.56 | -0.03 | 0.39 | 0.54 | 0.42 |
| ETU30 | 3 | -3.28 | -1.12 | 1.41 | 2.16 | 1.53 | -1.47 | -0.50 | 0.39 | 0.97 | 0.89 |
|  | 5 | -2.74 | -1.10 | 0.13 | 1.64 | 1.23 | -1.30 | -0.51 | 0.16 | 0.79 | 0.67 |
|  | 10 | -2.17 | -1.11 | -0.22 | 1.06 | 0.89 | -1.06 | -0.50 | -0.04 | 0.56 | 0.46 |
| ETU70 | 3 | -3.39 | -1.15 | 0.39 | 2.24 | 1.54 | -1.55 | -0.53 | 0.34 | 1.02 | 0.87 |
|  | 5 | -2.73 | -1.11 | 0.09 | 1.62 | 1.20 | -1.32 | -0.49 | 0.21 | 0.83 | 0.70 |
|  | 10 | -2.18 | -1.11 | -0.23 | 1.07 | 0.88 | -0.97 | -0.48 | -0.01 | 0.49 | 0.47 |
| SCS=30KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -1.89 | -0.16 | 1.14 | 1.73 | 1.30 | -0.97 | -0.07 | 0.71 | 0.90 | 0.78 |
|  | 5 | -1.36 | -0.15 | 0.90 | 1.21 | 1.05 | -0.70 | -0.03 | 0.56 | 0.67 | 0.59 |
|  | 10 | -0.99 | -0.14 | 0.59 | 0.85 | 0.73 | -0.51 | -0.05 | 0.38 | 0.46 | 0.43 |
| EPA5 | 3 | -1.98 | -0.18 | 1.21 | 1.80 | 1.39 | -1.04 | -0.07 | 0.77 | 0.97 | 0.84 |
|  | 5 | -1.47 | -0.17 | 0.89 | 1.30 | 1.06 | -0.81 | -0.08 | 0.56 | 0.73 | 0.64 |
|  | 10 | -1.00 | -0.14 | 0.61 | 0.86 | 0.75 | -0.50 | -0.03 | 0.40 | 0.47 | 0.43 |
| SCS= 60KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -1.80 | -0.16 | 1.16 | 1.64 | 1.32 | -0.92 | -0.04 | 0.71 | 0.88 | 0.75 |
|  | 5 | -1.37 | -0.14 | 0.89 | 1.23 | 1.03 | -0.74 | -0.05 | 0.57 | 0.69 | 0.62 |
|  | 10 | -0.98 | -0.15 | 0.59 | 0.83 | 0.74 | -0.55 | -0.04 | 0.41 | 0.51 | 0.45 |
| EPA5 | 3 | -1.98 | -0.21 | 1.16 | 1.76 | 1.37 | -0.94 | -0.02 | 0.75 | 0.92 | 0.77 |
|  | 5 | -1.46 | -0.18 | 0.88 | 1.28 | 1.06 | -0.72 | -0.03 | 0.58 | 0.69 | 0.61 |
|  | 10 | -1.05 | -0.18 | 0.57 | 0.87 | 0.75 | -0.53 | -0.05 | 0.42 | 0.48 | 0.47 |
| TDL-A 83Hz | 3 | -2.03 | -0.18 | 1.20 | 1.85 | 1.38 | -1.03 | -0.04 | 0.78 | 0.99 | 0.82 |
|  | 5 | -1.48 | -0.13 | 0.96 | 1.35 | 1.09 | -0.72 | -0.05 | 0.56 | 0.67 | 0.61 |
|  | 10 | -1.01 | -0.14 | 0.62 | 0.87 | 0.75 | -0.54 | -0.05 | 0.40 | 0.49 | 0.45 |
| TDL-C 83Hz | 3 | -2.14 | -0.24 | 1.13 | 1.90 | 1.37 | -1.01 | -0.04 | 0.77 | 0.97 | 0.81 |
|  | 5 | -1.50 | -0.20 | 0.86 | 1.30 | 1.06 | -0.77 | -0.06 | 0.61 | 0.71 | 0.67 |
|  | 10 | -1.07 | -0.21 | 0.56 | 0.86 | 0.77 | -0.53 | -0.06 | 0.40 | 0.47 | 0.46 |
| SCS= 120KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| TDL-A 83Hz | 3 | -2.52 | -0.50 | 0.91 | 2.02 | 1.41 | -0.94 | -0.03 | 0.73 | 0.91 | 0.76 |
|  | 5 | -1.91 | -0.49 | 0.64 | 1.42 | 1.12 | -0.72 | -0.07 | 0.6 | 0.65 | 0.67 |
|  | 10 | -1.40 | -0.47 | 0.33 | 0.94 | 0.80 | -0.52 | -0.05 | 0.42 | 0.47 | 0.46 |
| TDL-C 83Hz | 3 | -2.27 | -0.36 | 1.00 | 1.91 | 1.36 | -1.02 | 0.07 | 0.79 | 0.95 | 0.86 |
|  | 5 | -1.65 | -0.34 | 0.74 | 1.31 | 1.08 | -0.71 | -0.04 | 0.60 | 0.67 | 0.64 |
|  | 10 | -1.24 | -0.34 | 0.45 | 0.90 | 0.79 | -0.52 | -0.04 | 0.38 | 0.48 | 0.42 |

Summary

This contribution provides the simulation results of the CSI-RSRP measurement performance for NR mobility. It is suggested to consider the above CSI-RS measurement results when defining CSI-RS based L3 measurement requirements.

Reference

- R4-1910949, “Link-level simulation assumptions for CSI-RS based measurements”, CATT