3GPP TSG-RAN WG4 RAN4#93 	R4-1914978

Reno, Nevada November 18th – 22nd, 2019

Agenda item:	9.11.2.1

Source: 	Qualcomm

Title: 	Side conditions for L1-SINR Measurement

Document for:	Discussion

# Introduction

During the last meeting, RAN4 made the following agreement regarding side condition of L1-SINR measurement [1].

RAN4 Agreement:

  - Side condition for L1-SINR measurement

    - CMR only scenarios: reuse side condition for L1-RSRP measurement

    - CMR + IMR scenarios: FFS

This contribution provides our views about these.

# Discussion

During the last meeting, RAN4 agreed that the side condition of L1-SINR measurement in CMR only scenarios is same as that of L1-RSRP measurement.

Currently, the absolute accuracy requirement of CSI-RS based L1-RSRP is applicable in the range of Es/Iot >= -3 dB.

The applicable range for L1-RSRP accuracy requirements, in terms of Es/IoT, is not bounded to a maximum value because determining L1-RSRP at high Es/Iot is not challenging. However, for CSI-RS CMR only case, signal and noise are not orthogonal. Hence, separating signal and noise from the same CSI-RS resource can be challenging and computationally expensive at high SNR.

Observation 1: The applicable range for L1-RSRP accuracy requirements, in terms of Es/IoT, is not bounded to a maximum value.

Observation 2: For CSI-RS CMR only case, signal and noise are not orthogonal. Hence, separating signal and noise from the same CSI-RS resource can be challenging and computationally expensive at high SNR.

In Rel-15, the accuracy requirements of SS-SINR was designed to be applicable in the range, in terms of Es/Iot, of -6 to 25 dB. This is captured by the following note in accuracy requirements of different scenarios (e.g. table 10.1.12.1.1-1 of 38.133).

“NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB“

This same note should be added to the side condition for L1-SINR measurement in CMR only scenarios.

Proposal 1: The accuracy requirements for L1-SINR in CMR only scenarios is applicable for CSI-RS Es/Iot <= 25 dB.

The side conditions for L1-SINR measurement in CMR + IMR scenarios can also reuse the side conditions for CSI-RS based L1-RSRP measurement. Note that, the accuracy requirement for CSI-RS based L1-RSRP is applicable when CSI-RS Es/Iot >= -3 dB. Hence, for L1-SINR measurement, the accuracy requirements can be applicable when SSB Es/Iot or CSI Es/Iot (depending on the configured RS for channel measurement) lies in the range of -3 to 25 dB.

Proposal 2: In CMR + IMR scenarios, the accuracy requirements can be applicable when SSB Es/Iot or CSI Es/Iot (depending on the configured RS for channel measurement) lies in the range of -3 to 25 dB.

# Conclusion

Observation 1: The applicable range for L1-RSRP accuracy requirements, in terms of Es/IoT, is not bounded to a maximum value.

Observation 2: For CSI-RS CMR only case, signal and noise are not orthogonal. Hence, separating signal and noise from the same CSI-RS resource can be challenging and computationally expensive at high SNR.

Proposal 1: The accuracy requirements for L1-SINR in CMR only scenarios is applicable for CSI-RS Es/Iot <= 25 dB.

Proposal 2: In CMR + IMR scenarios, the accuracy requirements can be applicable when SSB Es/Iot or CSI Es/Iot (depending on the configured RS for channel measurement) lies in the range of -3 to 25 dB.

# References

- R4-1912742, “WF on RRM requirement scope for Rel-16 NR eMIMO WI”, 3GPP RAN WG4 92bis.

- 38.133, “NR: Requirements for support of radio resource management”, V 16.1.0.