3GPP TSG-RAN WG4 Meeting #93	R4-1915120

# Reno, Nevada, USA, 18 - 22 November, 2019

Title: 	Discussion and simulation results on NR UE HST performance requirements under single-tap

Source: 	Huawei, HiSilicon

Agenda item:	9.17.2.2.3

Document for:	Discussion

Background

During RAN4#92bis meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the UE demodulation requirements for NR Rel-16 HST under single-tap channel model.

Discussion

Maximum Doppler shift

- Maximum Doppler for 500km/h - For TDD 30KHz - For HST single tap, maximum Doppler is 1667Hz - For FDD 15KHz - Single tap HST, maximum Doppler is - Option 1: 1250Hz - Option 2: 972Hz - Other values between option 1 and option 2 are not precluded If requirements for 350km/h are introduced, smaller Doppler frequency value can be considered

For Option 1, 1250Hz is targeting carrier frequency 2.7GHz with UE velocity 500 km/h. However, As per WF [2], NR BS HST performance requirement is also under discussion. There is most likely to adopt the maximum Doppler shift 1750Hz with the carrier frequency 1.9GHz. From a practical point of view, it is not reasonable to set carrier frequency 2.7GHz since BS side can’t support such frequency for FDD 15 kHz. To align with BS, it is needed to set carrier frequency 1.9GHz and the maximum Doppler shift 875Hz.

Observation1: To align with BS, it is needed to set carrier frequency 1.9GHz and the maximum Doppler shift 875Hz.

Figure 2.1-1: UE moves when close to the base station

Doppler shift changes rapidly when UE is close to a base station. The following table shows Doppler shift change assuming Dmin=2m and UE moves as per Figure 2.1-1 in ∆t times.

Table 2.1-1: Doppler shift changes in ∆t for different maximum Doppler shift

| Maximum Doppler shift[Hz] | Doppler shift change in ∆t[Hz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10ms | 20ms | 40ms | 80ms |
| 875 | 574 | 998 | 1420 | 1647 |
| 972 | 638 | 1109 | 1578 | 1829 |
| 1250 | 820 | 1426 | 2029 | 2352 |

Doppler tracking capability using TRS is 1750Hz for SCS 15kHz.From above table we can get that Doppler shift change value exceed Doppler tracking capability at 40ms and 80ms, corresponding to maximum Doppler shift 972Hz and 1250Hz.

For maximum Doppler shift 972Hz or 1250Hz, if UE is configured with DRX, and UE is not expected to receive TRS within a period of time, it is possible to lead to a Doppler shift jump which exceeds the limit. BS can assume UE can always receive TRS without configuring DRX, however, it is not practical since it is not good for saving energy for UE. If UE missed several TRS period at some bad situations, such as a lower SNR, above situation will also occur, i.e. there is no enough margin for UE.

Observation2：There is no enough margin for UE for maximum Doppler shift 972Hz or 1250Hz if UE is configured with DRX or at some bad situations, such as a lower SNR.

Proposal 1: For single-tap scenario, it is proposed to adopt maximum Doppler shift 875Hz for FDD 15 kHz.

MCS

- For all the channel models in this way forward, FFS on the MCS - Option 1: MCS4 - Option 2: MCS13 - Option 3: MCS17 - Other MCS are not precluded - MCS should be decided based on whether the maximum throughput can be achieved

As per simulation results in Section 3, we can get the following proposal:

Observation 3: For single-tap, MCS 4, 13 and 17 are all feasible. MCS 13 has better performance considering balance between throughput and SNR.

Proposal 2: It is proposed to adopt MCS 13 for single-tap.

Simulations

- Updated simulation assumption for HST-single tap Parameter; Value. FDD 15KHz SCS; TDD 30KHz SCS. Antenna configuration; 2×2; 2×4. DMRS type; type 1. Number of DMRS symbols; DMRS 1+1+1. MCS; MCS 4; MCS 13; MCS 17 based on 64QAM table. TDD pattern; 7D1S2U, S: 6D 4G 4U. Propagation condition; HST-single tap. TRS periodicity; 10ms, 2slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Ds and Dmin; Ds= 300m, Dmin=2m. Rank; Rank = 1. BW; 10MHz; 40MHz. Maximum Doppler shift; Option 1: 1250Hz Option 2: 972Hz; 1667Hz. Testing metric; SNR @70% of maximum throughput.

We provide our simulations as per the simulation assumptions in [1] for HST single-tap as following:

Table 3-1: Ideal Simulation results for NR PDSCH under HST single-tap

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 972 | 2Tx 2Rx ULA Low | 4 | -3.60 |
| 2 |  |  |  | 13 | 4.42 |
| 3 |  |  |  | 17 | 8.45 |
| 4 |  |  | 2Tx 4Rx ULA Low | 4 | -6.37 |
| 5 |  |  |  | 13 | 2.27 |
| 6 |  |  |  | 17 | 6.48 |
| 7 |  | 1250 | 2Tx 2Rx ULA Low | 4 | -3.59 |
| 8 |  |  |  | 13 | 4.45 |
| 9 |  |  |  | 17 | 8.68 |
| 10 |  |  | 2Tx 4Rx ULA Low | 4 | -5.66 |
| 11 |  |  |  | 13 | 2.40 |
| 12 |  |  |  | 17 | 6.66 |
| 13 | 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 4 | -3.66 |
| 14 |  |  |  | 13 | 4.44 |
| 15 |  |  |  | 17 | 9.36 |
| 16 |  |  | 2Tx 4Rx ULA Low | 4 | -6.55 |
| 17 |  |  |  | 13 | 2.35 |
| 18 |  |  |  | 17 | 8.24 |

Figure 3-1: Ideal Simulation results for NR PDSCH under HST single-tap

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under single-tap channel model. Our observations and proposals are:

Observation1: To align with BS, it is needed to set carrier frequency 1.9GHz and the maximum Doppler shift 875Hz.

Observation2：There is no enough margin for UE for maximum Doppler shift 972Hz or 1250Hz if UE is configured with DRX or at some bad situations, such as a lower SNR.

Observation 3: For single-tap, MCS 4, 13 and 17 are all feasible. MCS 13 has better performance considering balance between throughput and SNR.

Proposal 1: For single-tap scenario, it is proposed to adopt maximum Doppler shift 875Hz for FDD 15 kHz.

Proposal 2: It is proposed to adopt MCS 13 for single-tap.

Reference

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC

- R4-1912809, Way forward on NR HST PUSCH demodulation requirements, RAN4#92Bis, Huawei, HiSilicon