3GPP TSG-RAN WG4 Meeting #93                                     R4-1915429

Reno, USA, 18th – 22th November, 2019

Source: 	Skyworks Solutions, Inc.

Title: 	Experimental Measurements for n1 40MHz CBW REFSENS and band n34 protection

Agenda Item:	10.18.1 UE RF (38.101-1) [NR_n1_BW-Core]

Document for:	Approval

# Introduction

At RAN plenary meeting 85, a new WI has been agreed to introduce NR Band n1 with up to 40 MHz UE channel bandwidth (CBW) support. This contribution provides transmitter (TX) noise measurements in receiver (RX) band and proposes reference sensitivity levels for UE operation in n1. Considering the large amount of back-off predicted by previous simulations to protect band n34, this document also provides some experimental intermodulation product measurements to help calibrate simulations. Considering experimental measurements are still ongoing at the time of writing, this contribution is a place holder to publish full results by the start of the meeting.

# Experimental measurements

## Power Amplifier Calibration and Test Waveforms

- A band 1 power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at channel edge.

- For Tx noise in Rx band power level measurements, the following waveforms are used in which the allocated Resource Blocks (RBs) are located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth.

  - 30 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz various RB allocations

  - 40 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz various RB allocations

  - Image and Local Oscillator (LO) rejection is set to -28 dBc

  - Counter-IM3 (C-IM3) set to -60 dBc.

  - Counter-IM5 (C-IM5) set to -70 dBc.

  - TX noise is measured at the band 1 duplex distance of 190 MHz.

Figure 1 below shows the spectrum of the PA input waveform used for TX noise measurement in RX band,. C-IM3 and C-IM5 products are visible, with C-IM5 falling in RX band.

Figure 1: PA input test waveform spectrum plot.

## 2.2. Experimental results

# MSD and Reference sensitivity estimation

We assume:

- 50 dB Transmit port to Receiver port duplexer rejection in Rx band, as a worst case assumption,

- 10 dB Primary antenna to diversity antenna isolation,

- 4 dB Post PA loss,

- 4 dB Antenna to LNA front-end losses,

- 3 dB Antenna diversity gain

- -1 dB SNR.

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

# Band 34 protection and A-MPR verification

# Conclusion

# References

[1]