3GPP TSG-WG RAN4 Meeting #94-e 	R4-2002568

- Online, 24th February – 6th March, 2020

Source:					Charter Communications

Title:  	TP for Rel-16 Intra-band CA for x CC DL/ y CC UL including contiguous and non-contiguous spectrum (x>=y) Bandwidth combination set, REFSENS and insertion loss parameters for CA_48B, CA_48A 48B, CA_48B-48B, CA_48B-48C, CA_48B-48D, CA_48B-48E with 1UL and 2UL configurations

Agenda item:  	7.1.2

Document for:  	Approval

# 1. Introduction

This contribution provides a TP for TR 36.716-01-01 to include the requirements for CA_48B, CA_48A-48B, CA48B-48B, CA_48B-48C, CA_48B-48D, CA_48B-48E with 1UL and 2UL configurations as defined in RP-192444 [1].

# 2. Text Proposal

##### ---Start of changes---

## 5.x	CA_48B

## CA_48A-48B

## CA_48B-48B

## CA_48B-48C

## CA_48B-48D

## CA_48B-48E

## CA_48B CA_48B

## CA_48A-48B CA_48B

## CA_48B-48B CA_48B

## CA_48B-48C CA_48B

## CA_48B-48D CA_48B

## CA_48B-48E CA_48B

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for Intra-band CA for x CC DL/ y CC UL including contiguous and non-contiguous spectrum (x>=y)

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48B | CA_48B | 10 | 10 |  |  |  | 20 | 0 |

| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | 1.4 |  | 3 |  | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz |  | MHz |  | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_48A-48B | CA_48B |  |  |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |
| CA_48B-48B | CA_48B | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  | 40 | 0 |
|  |  | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |
| CA_48B-48C | CA_48B | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  | 40 | 0 |
|  |  | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |
| CA_48B-48D | CA_48B | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  | 80 | 0 |
|  |  | See CA_48D Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |
| CA_48B-48E | CA_48B | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  | 100 | 0 |
|  |  | See CA_48E Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48B | CA_48B | 10 | 10 |  |  |  | 20 | 0 |

### 5.x.2	Co-existence studies

Table 5.x.2-1 summarizes frequency ranges where harmonics occur due to Band 48 CA with 1 UL.

Table 5.x.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  |  | 3rd Harmonic |  |  | 4th Harmonic |  |  |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | UL Low Band Edge |  | UL High Band Edge |  | UL Low Band Edge | UL High Band Edge |  | UL Low Band Edge | UL High Band Edge |  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge |  | UL High Band Edge |
| 48 |  | 3550 |  | 3700 |  | 7100 | 7400 |  | 10650 | 11100 |  | 14200 | 14800 | 17750 |  | 1850 |

Table 5.x.2-1 shows there are no harmonic issues for this combination.

So we can conclude that there is no issue on both DL/UL harmonic interference.

### 5.x.3	ΔTIB,c and ΔRIB,c values

No ΔTIB,c and ΔRIB,c values  for intra-band combinations are needed

### 5.x.4	MSD

Table 5.4.1A-0eA: Reference sensitivity QPSK PREFSENS (CA 48)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_48B | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |

The MSD analysis was conducted when CA_48Awas discussed. Because of similarity of CA_48B, we apply the MSD specified for CA_48A to that for CA_48

### 5.X.5	Vendor A, A-MPR for uplink CA bandwidth class B

#### 5.X.5.1	Contiguous RB Transmission

Results from these simulations are shown in the figures below.

Figure 5.x.5..1-1. Required A-MPR for 25 + 100 RB, QPSK

Based on the simulation results presented from Figure 5.x.5.1-1, the A-MPR requirements for contiguously allocated CA_48B is given in Table 5.x.5.1-1.

Table 5.x.5.1-1: A-MPR requirements for contiguously allocated CA_48B configuration

| CA_48B: CA_NS_xx | RBStart | L_CRB [RBs] | A-MPR for QPSK [dB] |
| --- | --- | --- | --- |
| 25 RB / 100 RB and 100 RB / 25 RB | 0 – 2 and 121 - 124 | >0 | ≤11dB |
|  | 3 – 27 | <85 | ≤3dB |
|  |  | >85 | ≤5dB |
|  | 27 – 120 | >0 | ≤2dB |

#### 5.X.5.2	2-cluster RB transmission

The simulations for 2-cluster RB transmission shall have the same simulation assumptions and requirement as Contiguous RB transmission.

Because of the large number of scenarios (almost 5 million combinations for 20 + 20 MHz) a set of around 20 000 randomly created allocation scenarios were simulated per bandwidth combination and appropriate back off value was searched.

Results from these simulations with proposed A-MPR mask are shown in the figures below

Figure 5.x.5.2-1: 2 clusters in 2CCs vs. allocation ratio, 25 RB + 100 RB

The mask shown in figure 5.X.5.2-1 can be formally defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =       17; 					0 ≤ A < 0.025

-40A + 18;			0.025 ≤ A < 0.1

-3.75A + 14.375;		0.1 ≤ A < 0.5

-7A + 16; 				0.5 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

### 5.X.5.3	Vendor B, A-MPR for uplink CA bandwidth class B

#### 5.x.5.3.1	A-MPR concept

Emission requirements are tighter outside of the band compared to inside the band with an exception that first 10 MHz on high side of that band has same -13 dBm requirement as within the band. Therefore, it would be beneficial to define two different A-MPRs firstly the band edge A-MPR which has higher -25 dBm...-40 dBm emission requirement on IMD3 region and secondly an inner-band A-MPR which would have lower A-MPR as -13 dBm requirement would apply on IMD3 region. This concept is presented in Figure 5.2.5.1-1.

Figure 5.X.5.3.1-1: Two different A-MPRs per CC combination

#### 5.x.5.3.2	Simulation assumptions

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

SEM, ACLR, spurious emission and additional CBRS emission limits are calculated for A-MPR.

The simulated A-MPR is assumed applied as max(A-MPR, MPR), where MPR is the 1CC MPR.

#### 5.X.5.3.3	Results for Contiguous CA Resource Allocations

The A-MPR regions illustrated in Figure5.X.5.3 1-1 are defined in Table 5.2.5.3-1.

Table 5.X.5.3-1: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC ≤ FH - BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 + 10 MHz | B |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC ≤ FH - BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 + 10 MHz | B |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC ≤ FH - BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 + 10 MHz | B |
| 20+20 | FL + BWCA/2 ≤ FC ≤ FH - BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 + 10 MHz | B |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

A-MPR at the band edge positions defined in Table 5.X.5.3-1 is given in Table 5.X.5.3-2 this is the A-case. For the lower A-MPR region, the results are given in Table 5.2.5.3-3 this is the B-case.

Herein, applied A-MPR should be considered as max(A-MRP, MPR), where A-MPR is given in Table 5.X.5.3-2 and Table 5.X.5.3-3 and MPR for single CC scenario in Table 6.2.3-1 of LTE specification 36.101.

Table 5.X.5.3-2: A-MPR regions for CA_48C at the band edge, (A)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 12 |
|  | 5 - 25 | < 85 | ≤ 5.5 |
|  |  | > 85 | ≤ 6.5 |
|  | 26 - 120 |  | ≤ 5 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 11 |
|  | 14 - 33 | < 85 | ≤ 5 |
|  |  | > 85 | ≤ 7 |
|  | 34 - 134 |  | ≤ 6 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 11 |
|  | 23 - 42 | < 95 | ≤ 5.5 |
|  |  | > 95 | ≤ 7 |
|  | 43 - 151 |  | ≤ 6 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 11 |
|  | 31 - 51 | < 100 | ≤ 5.5 |
|  |  | > 100 | ≤ 7 |
|  | 52 - 164 | < 100 | ≤ 4.5 |
|  |  | > 100 | ≤ 6 |

Table 5.X.5.3-3: A-MPR regions for CA_48C at the band center (“range for lower A-MPR”) (B)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 4 |
|  | 5 - 25 | < 85 | ≤ 2 |
|  |  | > 85 | ≤ 2.5 |
|  | 26 - 120 |  | ≤ 2 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 4.5 |
|  | 14 - 33 | < 85 | ≤ 1.5 |
|  |  | > 85 | ≤ 2.5 |
|  | 34 - 134 |  | ≤ 1.5 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 4.5 |
|  | 23 - 42 | < 95 | ≤ 1 |
|  |  | > 95 | ≤ 2.5 |
|  | 43 - 151 |  | ≤ 1 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 4.5 |
|  | 31 - 51 | < 100 | ≤ 1 |
|  |  | > 100 | ≤ 1.5 |
|  | 52 - 164 | < 100 | ≤ 1 |
|  |  | > 100 | ≤ 1 |

Furthermore, an example A-MPR scenario of 20+20 MHz CA combination with QPSK modulation is shown in Figure 3 with contiguous CA allocations, considering both the band edge scenario (“0 Hz offset”) and the lower A-MPR region (“29.8 MHz offset”).

Figure 5.X.5.3-1: The A-AMPR triangles for contiguous resource allocations of 20+20 MHz CA combination at the lower band edge (left) and at the lower edge of the range for lower A-MPR (right). EVM and IBE are not consider for A-MPR.

#### 5.X.5.3.4	Results for Non-contiguous CA Resource Allocations

As illustrated, with an example scenario of 20+20 MHz CA combination, in Figure 5.X.5.3-1 with non-contiguous CA allocations higher A-MPR is needed. In here, non-contiguous channel edge allocations are simulated as a worst-case-scenario.

For the channel edge positions indicated in Figure 5.X.5.1-1, 18 dB A-MPR is needed for the smallest allocation ratio resource allocations, i.e. the narrowest non-contiguous edge allocations.

For the lower A-MPR range indicated in Figure 5.X.5.1-1, 12 dB A-MPR is needed for the smallest allocation ratio resource allocations, i.e. the narrowest non-contiguous edge allocations.

Figure 5.2.5.4-1: The A-MPR for non-contiguous edge allocations of 20+20 MHz CA combination at the lower band edge (left) and at the lower edge of the range for lower A-MPR (right). EVM and IBE are not consider for A-MPR.

Considering the maximum A-MPR needed in the case of small allocation ratio, the results for other BW combinations are quite consistent with 20+20 MHz scenario presented in Figure 5.X.5.3-1.

Based on these results, the following formulation for A-MPR is proposed for the edge scenario:

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      18.00 - 10.00 A; 			0 ≤ A < 0.05

18.50 - 20.00 A;			0.05 ≤ A < 0.2

15.50 - 5.00 A;			0.2 ≤ A < 1

where A = NRB_alloc / NRB_agg.

Furthermore, the following formulation for A-MPR is proposed for the center scenario:

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      11.50 - 10.00 A; 			0 ≤ A < 0.15

10.88 -  5.88*A;			0.15 ≤ A < 1

where A = NRB_alloc / NRB_agg.

##### ---End of changes---

# Reference

[1]	RP-192444, “New WID on Rel16 LTE Intra-band CA for x DL/y UL including contiguous and non-contiguous spectrum(x>=y)”, RAN#80, Ericsson