3GPP RAN WG4 Meeting #94-e		R4-2000008

Online, 24 February – 6th March 2020

Agenda item:	9.19.1

Source:	Apple Inc.

Title:	A-MPR Proposal for n26

WI/SI:	NR_n26

Release:	Rel-16

Document for:	Approval

# 1	Introduction

A way forward was agreed in RAN4#85 [1] to introduce n26. This paper presents A-MPR values for modified n26 signaling requirements [2].

# 2  Discussion

In the following proposals for A-MPR values are provided. The emission requirements and signalling flags are not provided here but can be found in [2].

The following assumptions for the simulations were used:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

The simulations were checked for ACLR, SEM, Spur, EVM and IBE. Please note that with 256QAM the 28dBc image power assumption violates the maximum allowed EVM for some RB constellations independent of applied power reduction. Hence, for 256QAM the simulations were done for 28dBc (w/o EVM) and 30dBc (with EVM).

###### 2.1 A-MPR for NS_12

Table 1: Contiguous allocation A-MPR for NS_12

| Channel BW (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 5 | ≤1.26 | <1.08 | A1 |
|  | ≤1.26 | ≥1.08 | A2 |
|  | >1.26 & ≤1.8 | ≥1.8 | A2 |

Table 2: A-MPR for modulation and waveform type for NS_12

| Waveform | Modulation | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer / Inner | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.0 | 5.0 |
|  | QPSK | 4.5 | 5.0 |
|  | 16QAM | 5.0 | 5.5 |
|  | 64QAM | 5.0 | 5.5 |
|  | 256QAM | 5.0 | 6.0 |
| CP-OFDM | QPSK | 6.0 | 7.0 |
|  | 16QAM | 6.0 | 7.0 |
|  | 64QAM | 6.0 | 7.0 |
|  | 256QAM | 7.0 | 8.0 |

###### 2.2 A-MPR for NS_13

Table 3: Contiguous allocation A-MPR for NS_13

| Channel BW (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | LCRB – RBstart (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 | ≤1.44 | <1.08 | - | A1 |
|  | ≤1.44 | ≥1.08 | - | A2 |
|  | - | - | >1.08 | A3 |
|  | ≤0.90 | ≤1.62 | ≤1.08 | A4 |

Table 4: A-MPR for modulation and waveform type for NS_13

| Waveform | Modulation | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer / Inner | Outer / Inner | Outer | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | 3.5 | 4.5 | 4.5 | 4.5 |
|  | QPSK | 3.5 | 4.5 | 4.5 | 4.5 |
|  | 16QAM | 3.5 | 5.0 | 5.0 | 4.5 |
|  | 64QAM | 3.5 | 5.0 | 5.0 | 4.5 |
|  | 256QAM | 5.0 | 6.0 | 6.0 | 5.0 |
| CP-OFDM | QPSK | 4.5 | 6.5 | 6.5 | 4.5 |
|  | 16QAM | 4.5 | 6.5 | 6.5 | 4.5 |
|  | 64QAM | 4.5 | 6.5 | 6.5 | 5.0 |
|  | 256QAM | 7.0 | 8.0 | 8.0 | 7.0 |

Proposal: Use values found in Tables 1 to 4 to define n26 A-MPR.

# 3  Simulations

###### 3.1 Simulations results for NS_12

###### 3.2 Simulation results for NS_13

# 4	Conclusions

Proposal: Use values found in Tables 1 to 4 to define n26 A-MPR.

# 4	References

- RP-192273, “New WID on Introduction of NR Band n26“, Sprint Corp., AT&T, Dish Network, 3GPP TSG-WG4 Meeting RAN#85.

- R4-1913078, “n26 Emission requirements“, Sprint Corp., SouthernLINC, Dish Network, 3GPP TSG-WG4 Meeting RAN4#92-Bis, Congqing, China, USA, Oct., 2018.