3GPP RAN-WG4 Meeting #94-e		R4-2000015

Electronic meeting, 24 February – 6 March, 2020

Agenda item:	8.14.1.4

Source:	Apple Inc.

Title:	Views on FR2 DL intra-band CA REFSENS

WI/SI:	NR_RF_FR2_req_enh

Release:	Rel-16

Document for:	Approval

# 1	Introduction

In RAN#93 the scope of WF on FR2 inter-band DL CA [1] includes the further study of the relaxation values for the peak EIS requirements. Additionally, in the same RAN4 meeting the frequency separation classes (the maximum frequency span between lower edge of lowest CC and upper edge of highest CC) were extended to 2400 MHz. 
This contribution provides our view on the ΔRIB EIS relaxation for CA when considering a frequency separation span of up to 2400 MHz.

- CR to 38.101-2 on FR2 frequency separation class Frequency separation classes from IV to IX were included in [2] Table 5.3A.4-2: Frequency separation classes Frequency separation class; Frequency separation (Fs). I; Fs ≤ 800 MHz. II; Fs ≤ 1200 MHz. III; Fs ≤ 1400 MHz. IV; Fs ≤ 1000 MHz. V; Fs ≤ 1600 MHz. VI; Fs ≤ 1800 MHz. VII; Fs ≤ 2000 MHz. VIII; Fs ≤ 2200 MHz. IX; FS ≤ 2400 MHz. - Definition of ΔRIB EIS for Intra-band non-contiguous CA For each component carrier in the intra-band non-contiguous carrier aggregation, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity values determined from clause 7.3.2, and relaxation applied to peak reference sensitivity requirement as specified in Table 7.3A.2.2-1. The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs. Table 7.3A.2.2-1: ΔRIB EIS Relaxation for CA operation by cumulative aggregated channel bandwidth Cumulative Aggregated Channel BW (MHz); ΔRIB (dB). ≤ 800; 0.0. > 800 and ≤ 1400; 0.5.

# 2	Discussion

The REFSENS is defined as the EIS level in the RX beam peak direction, at which the throughput shall meet the requirements specified for the reference measurement channel. This contribution describes the impact of the wider frequency separation (e.g. 2400 MHz) on the REFSENS estimation for CA.

## 2.1	REFSENS for CA

The sensitivity of the receiver mainly consists of three portions, the noise figure NF of the entire receiver, the noise floor, kT in the environment and the frequency bandwidth of the receiver. The first LNA is a key component since its NF dominates the overall noise performance of the receiver. Among the noise source, thermal noise is the well-known.
The total thermal noise power is defined by the Boltzmann’s constant kb, the temperature in Kelvins and the overall bandwidth of the channel selectivity filter of the receiver. When we consider the normalized noise floor (1 Hz and 290 K temperature), the noise power level is calculated as follows:

Noise Power Level N dBm=-174 dBm+10*log10f

| Bandwidth f | Noise Power Level N [dBm] |
| --- | --- |
| 2400 MHz | -80.19 |
| 2000 MHz | -80.98 |
| 1800 MHz | -81.44 |
| 1400 MHz | -82.53 |

The table above provides the noise power level for frequency span from 1400 MHz to 2400 MHz. It can be observed that the noise power level increases with the bandwidth, for a bandwidth of 2400 MHz the noise power level is around 2 dB higher compared to 1400 MHz bandwidth.

Figure 1: Potential simplified reference architecture

In addition to the increase of the REFSENS in terms of thermal noise contribution, further challenges come for an LNA supporting wide bandwidth. A wideband LNA should provide a wideband impedance matching to the antenna, gain, good linearity and low noise figure. The matching of the circuit becomes a challenge, the input return loss S11 will increase in order to cover the wide bandwidth. Similarly, the wider bandwidth will lower the Q-factor, which will increase the noise figure of the receiver. Consequentially the increase of the NF will affect directly the receiver reference sensitivity, defined in FR2 with the following equation:

Sensitivity= -174 dBmkT+10logMax. Rx BW+NF-Total Ant.gain-diversity gain+SNR+ILS

Observation 1:	The wider bandwidth will lower the Q-factor, which will increase the noise figure of the receiver. Consequentially the increase of the NF will affect directly the receiver reference sensitivity.

The receiver chain (as shown in Figure 1) has to support wide bandwidth of up to 2400 MHz. Considering these challenges for the receiver when having a wide frequency separation class, we propose an additional relaxation for ΔRIB as provided in subsections 2.1.1 and 2.1.2.

For intra-band non-contiguous CA the EIS relaxation ΔRIB is defined in [3] up to 1400 MHz. The receiver chain (in Figure 1) is considered for the intra-band non-contiguous CA as reference architecture. Due to the challenges previously explained we propose to add an additional 0.5 dB on the REFSENS relaxation for a cumulative aggregated bandwidth between 1400 MHz and 2400 MHz as shown in the table below.

| Cumulative Aggregated Channel BW (MHz) | ΔRIB (dB) |
| --- | --- |
| ≤ 800 | 0.0 |
| > 800 and ≤ 1400 | 0.5 |
| > 1400 and ≤ 2400 | 1.0 |

Proposal 1:	Define 1.0 dB for the EIS relaxation ΔRIB for a cumulative aggregated channel BW from 1400 MHZ to 2400 MHz in intra-band non-contiguous CA reference sensitivity requirement.

# 3	Conclusions

This contribution has provided our view on the ΔRIB EIS relaxation for CA when considering the frequency separation span of up to 2400 MHz. In summary, we have made the following observation and proposals:

Observation 1:	The wider bandwidth will lower the Q-factor, which will increase the noise figure of the receiver. Consequentially the increase of the NF will affect directly the receiver reference sensitivity.

Proposal 1:	Define 1.0 dB for the EIS relaxation ΔRIB for a cumulative aggregated channel BW from 1400 MHZ to 2400 MHz in intra-band non-contiguous CA reference sensitivity requirement.

# 4	References

R4-1916024, “WF on FR2 inter-band DL CA”, Nokia, 3GPP RAN#93, November 2019

R4-1915475, “CR to 38.101-2 on FR2 frequency separation class enhancement”, Apple Inc, 3GPP RAN#93, November 2019

3GPP TS 38.101-2 V16.2.0

R4-1913540, “FR2 DL inter-band CA architecture considerations”, Apple Inc, 3GPP RAN#93, November 2019