3GPP TSG-WG RAN4 Meeting #94-e	R4-2000177

# Online, 24th February – 6th March, 2020

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP to TR 36.716-03-01: CA_2-7-26

Agenda Item: 			7.3.3

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 3 bands DL with 1 band UL for the CA bands CA_2-7-26.

- CA_2A-7A-26A-BCS0

# Discussion

Since the revised WID has 20 MHz channel bandwidth which is undefined for band 26, it shall be updated to remove 20MHz. A proposed WID update and TP to the TR 36.716-02-01 is provided in the following.

Required revions to WID revision: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL (RP-192811).

**************************** Start of changes ********************************************

| CA configuration | Uplink Configuration | BCS | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-7A-26A | - | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson, Nokia, , | On-going | DL_2A-7A_1UL_BCS0 (complete) DL_7A-26A_1UL_BCS0 DL_2A-66A_1UL_BCS0 (complete) |

**************************** Next changes ********************************************

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | Uplink CA configurations |  | E-UTRA Bands |  | 1.4 |  |  | 3 | 5 |  | 10 |  | 15 |  | 20 | Maximum aggregated bandwidth |  | Bandwidth combination set |
|  |  |  |  |  |  | MHz |  |  | MHz | MHz |  | MHz |  | MHz |  | MHz | [MHz] |  |  |
| CA_2A-7A-26A | - |  | 2 |  |  |  | Yes | Yes |  |  | Yes |  | Yes |  | Yes |  |  | 55 | 0 |
|  |  |  | 7 |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 26 |  |  |  | Yes | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |

**************************** End of changes ********************************************

TP to TR 36.716-03-01

**************************** Start of changes ********************************************

## 5.X	CA_2-7-26

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 3-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-7A-26A | 2 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  | 26 |  |  | Yes | Yes | Yes |  |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_2-7-26 is proposed to be based on the high-low band diplexer combining CA_2-7 quadplexer and band 26 chain.

Table 5.X.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-7-26 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 26 | 0.3 |

Table 5.X.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-7-26 | 2 | 0 |
|  | 7 | 0 |
|  | 26 | 0 |

### 5.X.3	REFSENS requirements

There is no REFSENS exception for CA_2A-7A-26A.

**************************** End of changes ********************************************