3GPP TSG-RAN4 Meeting #94-e    	　　　　　　　　　　　　　　　　           R4-2000230

Online, , 24th Feb 2020 - 6th Mar 2020

Source: 	NTT DOCOMO, INC.

Title:	EESS protection from n257

Agenda item:	6.5.7.3

Document for:	Approval

- Introduction

ITU WRC-19 concluded that in order to protect the EESS (passive) in the frequency band 23.6-24 GHz the unwanted emissions of IMT mobile stations operating in the frequency band 24.25-27.5 GHz (Active service band) shall meet -29 dB(W/MHz) until 1st Sep 2027 and with -35 dB(W/MHz) after 1st Sep 2027 as described in page 351 [1]. Note that -35 dB(W/MHz) limit will not apply to IMT mobile stations which have been brought into use prior to 1st Sep 2027. For those IMT mobile stations, the limit of -29 dB(W/200 MHz) will continue to apply after this date.

This contribution discusses how to incorporate the above conclusion into the n257 related requirements.

- Discussion

  - Protection requirement by WRC19

The below is an excerpt of the conclusion for the EESS (passive) protection of 23.6-24 GHz in [1] with respect to IMT mobile stations.

------------------------------------------------An excerpt of EESS protection of 23.6-24GHz --------------------------------------------

2.1 that, in order to protect the EESS (passive) in the frequency band 23.6-24 GHz, the following unwanted emissions of Active service band operating in the frequency band 24.25-27.5 GHz apply as specified in Table 1 below;

------------------------------------------End of an excerpt of EESS protection of 23.6-24GHz ------------------------------------------

2.2	Interpretation of the protection requirement by WRC19

The relationship among the frequency ranges for the EESS (passive), aggressor of Active service band and n257 is illustrated in Figure 2.2-1.

Figure 2.2-1: Frequency ranges relationship among EESS (passive), Active service band and Band n257

From the Figure 2.2-1, an overlapping portion between “Active service band” and n257 is 1 GHz, i.e., the frequency range of 26.5 - 27.5 GHz. Hence, an n257 capable UE using any portion of 26.5 - 27.5 GHz shall meet the required unwanted emission limits. It is, however, not clear the interpretation of “Active service band”. There would be multiple interpretations of this term in the following ways. Note: In NR, UL and DL channel bandwidth can be different.

- An n257 UE using DL channel bandwidth inside this band

- An n257 UE using UL channel bandwidth inside this band

- An n257 UE using UL resource blocks inside this band regardless of DL channel bandwidth condition

- Etc.

From coexistence perspective, it would be reasonable to focus on UL condition, though in NR, DL channel condition may affect UL requirements like MPR. The MPR, however, is not for how emission limit should be but rather for allowing UE to reduce its power to meet emission limit. Thus, it would be sufficient to make an n257 UE protect EESS with the required limit whenever the UE is transmitting UL resource element(s) whose any portion is inside 26.5-27.5GHz.

Proposal 1: n257 UEs shall meet the unwanted emission limits to protect the EESS (passive) only when any portion of the UL transmission bandwidth is inside 26.5 - 27.5GHz.

With respect to the emission limit, according to [1], transition period is set as follows.

- -29 dBW/200 MHz =  1 dBm/200 MHz until 1st Sep 2027

- -35 dBW/200 MHz = -5 dBm/200 MHz after 1st Sep 2027

Since the current requirement in TS38.101-2 for EESS protection is -8 dBm/200 MHz, whichever of the above two is compared, the current 3GPP requirement of -8 dBm/200 MHz is more stringent than the requirements by ITU WRC-19. Note that Band n257 CA requirement has this requirement of -8dBm/200MHz while Band n257 single carrier case is missing that requirement.

Considering even the current requirement of -8 dBm/200 MHz does not require any A-MPR for n257, it would be straightforward to select -5 dBm/200 MHz. This allows to avoid future spec changes before 2027 and make practical operation of n257 easier since we can reduce our efforts to distinguish Band n257 UEs satisfying -5 dBm/ 200 MHz and the UEs satisfying 1 dBm/200 MHz in the future.

.

Proposal 2: Specify -5 dBm/200MHz for Band n257 UEs from the beginning(No spec change from 1 to -5 dBm/200MHz in the future).

With respect to the necessity of a new NS, in our understanding, a new NS is not necessary since the main reason of NS is to avoid unnecessary burden for bands globally available by indicating when specific requirements are applicable in specific places. With this, UEs do not meet such specific requirements when the UEs are not in those places. For EESS protection, our understanding is that the requirement is globally applicable regardless of where the UEs are. Thus, it is all included in the Proposal 1 that the information that the UEs need to understand when EESS protection should be met. Concrete specification examples based on the TS38.101-2(16.2.0) are as follows.

Table 6.5.3.1-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |
|  | Frequency range3 | 23600 | - | 24000 | -5 | 200 |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The requirement applies when any portion of the UL transmission bandwidth is inside 26.5-27.5 GHz. |  |  |  |  |  |  |

Table 6.5A.3-1: Requirements for CA

| UL CA for any CA bandwidth class | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band / frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n257 | NR Band n259 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | -5 | 200 | 2, 3 |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: The protection of frequency range 23600 - 2400 MHz is meant for protection of satellite passive services. NOTE 3: The requirement applies when any portion of the UL transmission bandwidth is inside 26.5 – 27.5 GHz. |  |  |  |  |  |  |  |

- Conclusion

This contribution discussed how to incorporate ITU WRC-19 conclusion for the EESS (passive) protection in 23.6-24 GHz into 3GPP specification for Band n257 Power class 3. Based on the discussion, we propose the followings. We also provided companion CRs to incorporate specific requirements based on the proposals in [2].

Proposal 1: n257 UEs shall meet the unwanted emission limits to protect the EESS (passive) only when any portion of the UL transmission bandwidth is inside 26.5 - 27.5GHz.

Proposal 2: Specify -5 dBm/200MHz for Band n257 UEs from the beginning(No spec change from 1 to -5 dBm/200MHz in the future).

- References

[1] https://www.itu.int/en/ITU-R/conferences/wrc/2019/Documents/PFA-WRC19-E.pdf

[2] R4-2000231, Correction of EESS protection from n257,  NTT DOCOMO, INC., 3GPP TSG-RAN4 Meeting #94-e