3GPP TSG-RAN4 Meeting #94-e	R4-2000235

# Online, 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0096 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the FR2 multi-band requirement framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the analysis of testability concerns associated with the Rel-15 multi-band requirement (MBR) framework provided by RAN5 in R5-199424, a practical approach to address the concerns is to introduce a maximum cap on to the per-band relaxation factors, such that ∆MBP,n ≤ 0.75 dB and ∆MBS,n ≤ 0.75 dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a maximum cap on to the per-band relaxation factors, such that ∆MBP,n ≤ 0.75 dB and ∆MBS,n ≤ 0.75 dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MBR framework will continue to exhibit the testabilty challenges identified by RAN5 in R5-199424. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes >>

#### 6.2.1.3	UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of total component of EIRP (Link=Beam peak search grids, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.3-1 and Table 6.2.1.3-4.

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void |  |

The maximum output power values for TRP and EIRP are found on the Table 6.2.1.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.3-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.3-3 below. The requirement is verified with the test metric of the total component of EIRP (Link=Beam peak search grids, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-3. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.3-3 and Table 6.2.1.3-4.

Table 6.2.1.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.3-1 and 6.2.1.3-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n. For each combination of supported bands ΔMBP,n and ΔMBS,n apply to each supported band n, such that the total relaxations, ∑MBP and ∑MBS, across all supported bands shall not exceed the total value indicated in Table 6.2.1.3-4.

Table 6.2.1.3-4: UE multi-band relaxation factors for power class 3

| Supported bands | ∑MBP (dB) | ∑MBS (dB) |
| --- | --- | --- |
| n257, n258 | ≤ 1.3 | ≤ 1.25 |
| n257, n260 n258, n260 | ≤ 1.0 | ≤ 0.753 |
| n257, n261 | 0.0 | 0.0 |
| n258, n261 | ≤ 1.0 | ≤ 1.25 |
| n260, n261 | 0.0 | ≤ 0.752 |
| n257, n258, n260 n257, n258, n261 n257, n258, n260, n261 | ≤ 1.7 | ≤ 1.753 |
| n257, n260, n261 | ≤ 0.5 | ≤ 1.253 |
| n258, n260, n261 | ≤ 1.5 | ≤ 1.253 |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands NOTE 2: For supported bands n260 + n261, ΔMBS,n is not applied for band n260 NOTE 3: For n260, maximum applicable MBS,n is 0.4 dB and MBP,n is 0.75 dB NOTE 4: For all bands except n260, the maximum applicable MBP,n and MBS,n is 0.75 dB |  |  |

<< end of changes >>