3GPP TSG-RAN WG4 Meeting #94-e                                                            R4-2000239

Online, 24th Feb-6th Mar, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_2-5_n5 and DC_2-2-5_n5

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_2-5_n5 and DC_2-2-5_n5 according to the request in [1].

2. Reference

- RP-192815, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

5.1.x	DC_2-5_n5 and DC_2-2-5_n5

5.1.x.1		Operating bands for DC

Table 5.1.x.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n5 | CA_2-5 | n5 | No |
| DC_2-2-5_n5 | CA_2-2-5 | n5 | No |

5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A_n5A | DC_2A_n5A | CA_2A-5A | n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A | CA_2A-2A-5A | n5A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no harmonic or IMD issues for this band combination.

5.1.x.4	∆TIB and ∆RIB values

For DC_2-5_n5 and DC_2-2-5_n5, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_2A-5A.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n5 DC_2-2-5_n5 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n5 DC_2-2-5_n5 | 2 | 0 |
|  | 5 | 0 |
|  | n5 | 0 |

5.1.x.4	REFSENS requirements

There is no additional requirement for this band combination.

<End of Text Proposal>