3GPP TSG-RAN WG4 Meeting #94-e                                                            R4-2000246

Online, 24th Feb-6th Mar, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2 according to the request in [1].

2. Reference

- RP-192815, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n2 | CA_5-66 | n2 | No |
| DC_5-5-66_n2 | CA_5-5-66 | n2 | No |
| DC_5-66-66_n2 | CA_5-66-66 | n2 | No |
| DC_5-5-66-66_n2 | CA_5-5-66-66 | n2 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-66A_n2A | DC_5A_n2A | CA_5A-66A | n2A |
| DC_5B-66A_n2A | DC_5A_n2A | CA_5B-66A | n2A |
| DC_5A-5A-66A_n2A | DC_5A_n2A | CA_5A-5A-66A | n2A |
| DC_5A-66A-66A_n2A | DC_5A_n2A | CA_5A-66A-66A | n2A |
| DC_5B-66A-66A_n2A | DC_5A_n2A | CA_5B-66A-66A | n2A |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A | CA_5A-5A-66A-66A | n2A |

5.1.x.3	Coexistence studies

Based on co-existence studies of DC_5A_n2A in TR 37.863-01-01, and the impact to own Rx of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band5 + Band n2 may fall into own Rx of band 66.

5.1.x.4	∆TIB and ∆RIB values

For DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2, the TIB,c and RIB,c values reused the values for DC_5_n2 in TS 38.101-3.

Table 5.1.x.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.x.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 5 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.x.5	REFSENS requirements

able 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5A-66A_n2A DC_5BA-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | 66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 ¦2*fB5-2*fBn2¦ |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |

<End of Text Proposal>