3GPP TSG-RAN WG4 Meeting #94                                                             R4-2000248

Online, 24th Feb-6th Mar, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_5-66-66_n5

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_5-66-66_n5 referring TR37.716-21-11v0.9.0 according to the request in [1].

2. Reference

- RP-192815, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.64	DC_5A-66A_n5A and DC_5A-66A-66A_n5A

### 5.1.64.1	Operating bands for DC

Table 5.1.64.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n5 | CA_5-66 | n5 | No |
| DC_5-66-66_n5 | CA_5-66-66 | n5 | No |

### 5.1.64.2	Configuration for DC

Table 5.1.64.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-66A_n5A | DC_66A_n5A | CA_5A-66A | n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A | CA_5A-66A-66A | n5A |

5.1.64.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the second order IMD may fall into band 5 and/or n5.

5.1.64.4	∆TIB and ∆RIB values

For DC_5-66_n5 and DC_5-66-66_n5, the TIB,c and RIB,c values are derived from CA_5-66 in TS 36.101.

Table 5.1.64.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n5 DC_5-66-66_n5 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.64.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n5 DC_5-66-66_n5 | 5 | 0 |
|  | 66 | 0 |
|  | n5 | 0 |

### 5.1.64.5	REFSENS requirements

MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_66A_n5A) in TS 38.101-3, where band n5 MSD is specified. To avoid the specification complexity, band 5 MSD is not specified as the same MSD is applied if band n5 carrier frequency is replaced by band 5 carrier frequency.

<End of Text Proposal>