3GPP TSG-RAN WG4 Meeting #94-e            	R4-2000300

Online, 24th February – 6th March, 2020

Agenda item:	8.18.1.2

Source:	Samsung

Title:	Simulation results of PMI test cases

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the WF was approved regarding to the initial simulation assumptions for PMI reporting requirements with Tx port larger than 8 and up to 32. The agreements related to PMI performance requirements were reached in the WF as listed below:

- Type of requirements

  - PMI only

- Frequency range

  - FR1

- Number of Tx ports

  - 16 and 32 for defining requirement

  - Test applicability:

Conduct the tests based on UE capability, for UE supporting 16 and 32 Tx ports, conduct test for 32 Tx ports

- Codebook type

  - Type I single panel codebook

- PMI test

  - Wideband PMI requirement: 32Tx port

  - Subband PMI requirement for 16Tx port

- If subband PMI test for 16 Tx ports will be introduced, only 32 Tx ports will be covered in wideband PMI test.

- If subband PMI test for 16 Tx ports will not be introduced, both 16 and 32 Tx ports will be covered in wideband PMI test

- Decide whether to define subband requirements based on the simulation results

- Companies are encouraged to run simulation for the following 4 cases. Consider the following priority if no time to run simulation for all 4 cases.

- Priority for simulation: Follow subband PMI, Random subband PMI > Follow wideband PMI, Random wideband PMI

- For subband/wideband gain comparison, run all tests with channel model TDLC300-5

- Propagation condition:

  - TDLA30-5 for wideband PMI with 32Tx

  - TDLC300-5 for subband PMI and wideband PMI with 16Tx

- Test metric:

  - As baseline, Relative throughput ratio between following PMI and random PMI at SNR point corresponding to 90% TP with follow PMI

In this contribution, the initial simulation results are provided for alignment purpose.

# 2	Simulation results

We provide simulation results of PMI tests with 16 and 32 CSI-RS ports under 2Rx and 4Rx, respectively. In the tests of 32 Tx ports, the wideband PMI is considered.

Regarding 16Tx test, both wideband PMI and subband PMI are considered.

Fig2-1: SNR--Throughput of 32x2 and 32x4 for FDD

Fig2-2: SNR--Throughput of 32x2 and 32x4 for TDD

Fig2-3: SNR--Throughput of 16x2 for FDD wideband and subband PMI

Table2-1: SNR point at 90% of peak throughput and TP ratio for FDD and TDD with wideband PMI

| nTx ports | 32 (4,4,4,4) TDD |  | 32 (4,4,4,4) FDD |  |
| --- | --- | --- | --- | --- |
| NRx | 2 | 4 | 2 | 4 |
| SNR point/TP ratio | 12dB/6.025 | 7.74dB/9.21 | 11.15dB/7.38 | 7.37dB/10.15 |

Table2-2: SNR point at 90% of peak throughput and TP ratio for FDD

| nTx ports | 16 (4,2,4,4) FDD | 32 (4,4,4,4) FDD |
| --- | --- | --- |
| NRx | 2 | 2 |
| SNR point/TP ratio (Subband PMI) | 13.69 dB/3.54 | 12.2 dB / 7.51 |
| SNR point/TP ratio (Wideband PMI) | 13.69 dB/3.23 | 12.3dB / 7.03 |

Observation 1:  The gain of subband compared with wideband for SNR point at 90% is not significant for both 16 Tx and 32 Tx port with follow PMI.

# 3	Conclusion

In this contrition, the initial simulation results are provided with Type I PMI for Tx ports larger than 8 and up to 32 cases

Observation 1:  The gain of subband compared with wideband for SNR point at 90% is not significant for both 16 Tx and 32 Tx port with follow PMI.

# References

R4-1915858,”Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, Samsung, 3GPP RAN4 #93

R4-1911409,”Discussion on PMI simulation for 16Tx and 32Tx ports”, Samsung, 3GPP RAN4 #92bis

3GPP TS 36.101 V16.2.0, User equipment (UE) radio transmission and reception (Release 16)

# Appendix

The configurations are showed as following:

Table 1  Common parameters for all cases

| Parameter | Unit | Value |
| --- | --- | --- |
| DMRS parameters |  | Type 1, 1 front loaded symbol + 1 additional DMRS symbol |

Table 2  Simulation assumptions for 16x2

| Parameter |  | Unit | Value | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) (O1,O2) = (4,4) | High XP 16 x 2 (N1,N2) = (4,2) (O1,O2) = (4,4) |
| Rank |  |  | 2 | 2 |
| MCS |  |  | 20 | 20 |
| UL/DL Pattern |  |  |  | 7D1S2U |
| Correlation |  |  | XP-High | XP-High |
| Beamsteering Model |  |  | 38.101-4 Annex B.2.3.2.3 | 38.101-4 Annex B.2.3.2.3 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5 (4,-) | Row 5 (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 | 16 |
|  | CDM Type |  | CDM4 | CMD4 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3 ) |  | Row 11 (0,2,4,6) | Row 11 (0,2,4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (12,-) | (12,-) |
|  | CSI-RS interval and offset |  | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 | 5/1 |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) | (4,2) |
|  | CodebookSubsetRestriction |  | 11111111 | 11111111 |
|  | RI Restriction |  | 00000001 | 00000001 |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 | 13 |
| Maximum number of HARQ process |  |  | 4 | 8 |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 3 Antenna configuration for all 32 ports

| Parameter | Unit | Value |
| --- | --- | --- |
| Antenna configuration |  | High XP 32x 2 (N1,N2) = (4,4) (O1,O2) = (4,4) |