3GPP TSG-RAN WG4 Meeting #94-e                                                           R4-2000302

Online, 24th February- 6th March, 2020

Source:	Samsung

Title:	Test case design for PMI reporting with Type II codebook

Agenda item:			8.18.1.2

Document for:	Approval

# 1. Introduction

The latest work objectives for RAN4 leading Rel-16 performance requirements enhancement WI were updated in RAN#85 as below:

UE demodulation and CSI reporting requirements: - NR CA PDSCH normal demodulation and CA CQI reporting requirements for NR CA, EN-DC, NE-DC, NR-DC - All NR CA normal demodulation and CA CQI reporting requirements are applicable to EN-DC, NE-DC, NR-DC with CA. For EN-DC and NE-DC, EUTRA performance will not be verified. For NR-DC, no specific requirements will be specified. - Note: the work for CA CQI reporting requirements can be started from March 2020. - NR FR1 CA PDSCH demodulation performance requirement with power imbalance - Intra-band contiguous 2CC CA with 6dB power imbalance is assumed - Requirements for the number of TX ports larger than 8 and up to 32 - Type of requirements: RAN4 should further decide if only PMI and/or CRI reporting needs to defined or both PMI and/or CRI reporting and PDSCH demodulation requirements are needed - LTE-NR co-existence demodulation test cases for TDD mode - Decide the applicable release for new UE requirements BS demodulation requirements: - To study the tests coverage of PUSCH requirements for 30% TP test point. Limited test cases will be introduced if the existing test coverage is insufficient. - Additional BS demodulation requirements - For FR2 PUSCH 2T2R 16QAM, specify requirements for MCS lower than 16, considering the OTA testability limit, to replace the existing requirements for MCS 16.

In RAN4#92b meeting, RAN4 agreed to introduce PMI test case with Type II codebook besides 16Tx/32Tx PMI test cases for Type-I single panel codebook under performance enhancement WI. As agreed in RAN#93 meeting, it is agreed to encourage companies to provide the input on the details test parameters.

In this contribution, we provide detailed test case design for PMI reporting with Type II codebook.

# 2. Discussion

NR Type II codebook is similar as LTE eFD-MIMO advanced codebook which targets to enable multi-beams with linear combinations.

Codebook construction:

NR Type II codebook consists of below parameters

- (N1, N2): Number of antennas  in each direction per polarization

- (O1, O2): Oversampling rate

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 4 | (2,1) | (4,1) |
| 8 | (2,2) | (4,4) |
|  | (4,1) | (4,1) |
| 12 | (3,2) | (4,4) |
|  | (6,1) | (4,1) |
| 16 | (4,2) | (4,4) |
|  | (8,1) | (4,1) |
| 24 | (4,3) | (4,4) |
|  | (6,2) | (4,4) |
|  | (12,1) | (4,1) |
| 32 | (4,4) | (4,4) |
|  | (8,2) | (4,4) |
|  | (16,1) | (4,1) |

- L (numberOfBeams): number of beams, L =2 for P_CSI-RS = 4, L  {2,3,4} for P_CSI-RS >4

- Npsk (phaseAlphabetSize) : beam combining coefficient (phase) set size (QPSK or 8PSK),

- subbandAmplitude：'true' or 'false' to enable/disable sub-band amplitude selection

In order to provide comparable performance with LTE, 16 Tx ports with (N1, N2) = (4, 2), (O1, O2) = (4, 4) can be taken as a starting point. Also considering the test complexity issue, L=2 with 2 beams can be selected.  For value of Npsk (phaseAlphabetSize) and subbandAmplitude configuration can be further discussed.

Proposal 1: For NR Type II codebook construction, using below configurations

- Number of ports: 16 Tx ports with (N1, N2) = (4,2) and (O1, O2) = (4,4)

- L (numberOfBeams): 2

- Npsk (phaseAlphabetSize) : FFS for beam combining coefficient (phase) set size, Npsk =4 can be taken for initial simulation purpose

- subbandAmplitude: FFS for  'true' or 'false'

Beam steering model：

The multi-beam steering approach as specified in B.2.3B.4A of TS36.101 can be used as starting point to enable dual-cluster beam directions during the test with relative power ration (p) among two beams as equal to 1.

Proposal 2: Reusing beam steering approach with dual-cluster beams as as specified in B.2.3B.4A of TS36.101

- Relative power ratio among two beams can be fixed as 1 (p =1)

Test metric:

Proposal 3: For test metric, several approaches can be further considered:

- Alt1 : TP ratio between following PMI under Type II codebook and following PMI under  Type-I single panel codebook

- Alt2: TP ratio between following PMI and rand PMI

Others parameter:

Proposal 4: below parameters can be used as starting point for initial summation purpose:

- MCS and rank: 16QAM ½, rank2

- MIMO correlation: XP High

- Channel model: TDLA30-5

# 3. Conclusion

In this contribution, detailed test case design for detailed test case design for PMI reporting with Type II codebook provided.

Proposal 1: For NR Type II codebook construction, using below configurations

- Number of ports: 16 Tx ports with (N1, N2) = (4,2) and (O1, O2) = (4,4)

- L (numberOfBeams): 2

- Npsk (phaseAlphabetSize) : FFS for beam combining coefficient (phase) set size, Npsk =4 can be taken for initial simulation purpose

- subbandAmplitude: FFS for  'true' or 'false'

Proposal 2: Reusing beam steering approach with dual-cluster beams as as specified in B.2.3B.4A of TS 36.101

- Relative power ratio among two beams can be fixed as 1 (p =1)

Proposal 3: For test metric, several approaches can be further considered:

- Alt1 : TP ratio between following PMI under Type II codebook and following PMI under  Type-I single panel codebook

- Alt2: TP ratio between following PMI and rand PMI

Proposal 4: below parameters can be used as starting point for initial summation purpose:

- MCS and rank: 16QAM ½, rank2

- MIMO correlation: XP High

- Channel model: TDLA30-35

# 4. Reference

[1] RP-192281,   “Revised WID on NR performance requirement enhancement”, China Telecom