3GPP TSG-RAN WG4 Meeting #94-e              	R4-2000306

Online, 24th February – 6th March, 2020

Agenda item:	8.17.2.2.1

Source:	Samsung

Title:	Discussion and initial simulation results for NR HST PUSCH

Document for:	Discussion

# 1	Introduction

During last RAN4#93 meeting, the general issues about BS demodulation requirement under high speed scenario were discussed and the agreements are captured in the WF [1].

In this contribution, the initial simulation results are provided for alignment purpose.

# 2	Discussion and simulation results

## 2. 1 Discussion

In this section, the view on remained issue of HST requirements is presented.

Antenna configuration

As agreed, the following antenna configuration options are considered:

- Tunnel scenario: 1x2,FF2 1x1 - Test setup for conducted test with 1x1 needs further discussion - FFS on OTA test for 1x1 - Whether to introduce test for 1x1 needs to consider the testable SNR value - For open space scenario: 1x2 and 1x8 - As per BS declaration, only one antenna configuration is selected for test - Define conducted tests for 1x2 and 1x8 - Define OTA tests for 1x2 only

Generally, it would be relatively easy for uplink to use more than 2Rx for better tracking and channel estimation performance. For HST requirement in LTE, during discussion, the performance requirements are specified for both with and without Rx diversity. To minimize the number of tests, only 1x2 antenna configuration is specified requirement for open space. While for 1x1 antenna configuration for tunnel scenario, it is chosen with the realistic operation as well as in [3], deployed with W-CDMA BS.

Since the test purpose is to verify the HST function, the Doppler shift variation is the same between antennas. Meanwhile, the Doppler value tracking and estimation is performed per antenna. In our view, we do not think it is necessary to test all the antenna configurations.

In LTE Rel-8, the typical antenna configuration is uniform linear antenna array (ULA ) structure with one polarization, due to the number of antenna is limited to 4, the antenna size is acceptable.

In terms of testing, LTE can support conducted test, it is easy to connect with one antenna (polarization) for testing. While for NR, excepting for conducted test for BS type 1-C, OTA testing is supported for BS type 1-O. In case of OTA testing, the different polarization of test antennas may result in performance different with certain isolation.

As indicated in eAAS spec,

- OTA performance requirements are only specified for up to 2 demodulation branches - If the OTA AAS BS uses polarization diversity and has the ability to maintain isolation between signals for each of the demodulation branches, the OTA performance requirements can be tested for up to two demodulation branches (i,e 1Tx-1Rx or 1Tx-2Rx test setup). When tested for two demodulation branches, each demodulation branch maps to one polarization. - If the OTA AAS BS does not use polarization diversity then OTA performance requirements can only be tested for a single demodulation branch (i.e, 1Tx-1Rx test setup)

- Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarization supported by the BS, with the required SNR applied separately per polarization - Only 2RX BS performance requirements apply when OTA AAS BS supports and is tested with dual polarization.

Based on the eAAS spec for OTA test, only 2Rx BS with dual polarization can be tested. As mentioned, dual polarization antenna structure should be the typical deployment in NR with considering antenna size and polarization diversity. So, with considering the test effort and realistic operation, we prefer to only define the HST requirement with 1x2 antenna configuration for tunnel scenario.

Proposal 1: Only define HST requirement with 1x2 antenna configuration for tunnel scenario.

Test parameters

In LTE, the test parameters for high speed train conditions as follows:

Subframes in which PUSCH is transmitted; For FDD: subframe #0 and #8 in radio frames for which SFN mod 4 = 0 subframe #6 in radio frames for which SFN mod 4 = 1 subframe #4 in radio frames for which SFN mod 4 = 2 subframe #2 in radio frames for which SFN mod 4 = 3 For TDD: Subframe #2 in each radio frames.

The subframes in which PUSCH is transmitted is related with HARQ process, which can allow a maximum of 4 transmissions. In FDD, the feedback delay is 8ms, and 10ms in TDD with UL-DL configuration 2. Since we agree the maximum throughput is an FRC equals to the payload size* the number of uplink subframes per second in which PUSCH is transmitted, similarly, we propose to indicate the PUSCH transmission subframes in the test parameters as follows:

| Slots in which PUSCH is transmitted | For FDD : slot #0 and #8 in radio frames for which SFN mod 4 = 0 slot #6 in radio frames for which SFN mod 4 = 1 slot #4 in radio frames for which SFN mod 4 = 2 slot #2 in radio frames for which SFN mod 4 = 3 For TDD in 15KHz SCS: slot #4 in each radio frames For TDD in 30KHz SCS slot #8 and slot#18 in radio frames |
| --- | --- |

Proposal 2: The Slots in which PUSCH is transmitted is proposed as follow

| Slots in which PUSCH is transmitted | For FDD : slot #0 and #8 in radio frames for which SFN mod 4 = 0 slot #6 in radio frames for which SFN mod 4 = 1 slot #4 in radio frames for which SFN mod 4 = 2 slot #2 in radio frames for which SFN mod 4 = 3 For TDD in 15KHz SCS: slot #4 in each radio frames For TDD in 30KHz SCS slot #8 and slot#10 in radio frames |
| --- | --- |

## 2. 2 Simulation results

In this subsection, both ideal and impairment results are provided to requirement finalization

Table 1: Ideal and impairment results for NR HST PUSCH with CP-OFDM for open scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal ) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | -5.857 | -3.857 |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 1340Hz | -5.867 | -3.867 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1334Hz | 7.514 | 9.514 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 1334Hz | 7.218 | 9.218 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | -5.835 | -3.835 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 2334Hz | -5.841 | -3.841 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | 7.525 | 9.525 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 2334Hz | 7.152 | 9.152 |
| 15KHz, 10MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 1340Hz | -11.26 | -9.26 |
| 15KHz, 10MHz | 2 | 1T8R | 1+1+1(3,7,11) | AWGN 1340Hz | -11.24 | -9.24 |
| 15KHz, 10MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 1340Hz | 1.55 | 3.55 |
| 15KHz, 10MHz | 16 | 1T8R | 1+1+1(3,7,11) | AWGN 1340Hz | 1.59 | 3.59 |
| 30KHz, 40MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 2334Hz | -11.24 | -9.24 |
| 30KHz, 40MHz | 2 | 1T8R | 1+1+1(3,7,11) | AWGN 2334Hz | -11.22 | -9.22 |
| 30KHz, 40MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 2334Hz | 1.49 | 3.49 |
| 30KHz, 40MHz | 16 | 1T8R | 1+1+1(3,7,11) | AWGN 2334Hz | 1.51 | 3.51 |

Table 2: Ideal and impairment results for NR HST PUSCH with CP-OFDM for tunnel scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | -5.754 | -3.754 |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 1340Hz | -5.797 | -3.797 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | 8.031 | 10.031 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 2334Hz | 7.646 | 9.646 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | -5.750 | -3.75 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 1340Hz | -5.780 | -3.78 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | 8.006 | 10.006 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 2334Hz | 7.481 | 9.481 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 1340Hz | -2.80 | -0.8 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(3,7,11) | AWGN 1340Hz | -2.79 | -0.79 |
| 15KHz, 10MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 1340Hz | 10.74 | 12.74 |
| 15KHz, 10MHz | 16 | 1T1R | 1+1+1(3,7,11) | AWGN 1340Hz | 11.05 | 13.05 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 2334Hz | -2.78 | -0.78 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(3,7,11) | AWGN 2334Hz | -2.77 | -0.77 |
| 30KHz, 40MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 2334Hz | 10.72 | 12.72 |
| 30KHz, 40MHz | 16 | 1T1R | 1+1+1(3,7,11) | AWGN 2334Hz | 11.65 | 13.65 |

Table 3: Ideal results and impairment results for NR HST PUSCH with CP-OFDM for open scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | -5.79 | -3.79 |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 1740Hz | -5.79 | -3.79 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | 7.79 | 9.79 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 1740Hz | 7.81 | 9.81 |
| 15KHz, 10MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 1740Hz | -11.20 | -9.2 |
| 15KHz, 10MHz | 2 | 1T8R | 1+1+1(3,7,11) | AWGN 1740Hz | -11.18 | -9.18 |
| 15KHz, 10MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 1740Hz | 2.16 | 4.16 |
| 15KHz, 10MHz | 16 | 1T8R | 1+1+1(3,7,11) | AWGN 1740Hz | 2.18 | 4.18 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | -5.744 | -3.744 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 3334Hz | -5.764 | -3.764 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | 8.383 | 10.383 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 3334Hz | 7.854 | 9.854 |
| 30KHz, 40MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 3334Hz | -11.17 | -9.17 |
| 30KHz, 40MHz | 2 | 1T8R | 1+1+1(3,7,11) | AWGN 3334Hz | -11.16 | -9.16 |
| 30KHz, 40MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 3334Hz | 2.21 | 4.21 |
| 30KHz, 40MHz | 16 | 1T8R | 1+1+1(3,7,11) | AWGN 3334Hz | 2.21 | 4.21 |

Table 4: Ideal and impairment results for NR HST PUSCH with CP-OFDM for tunnel scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | 4.85 | 6.85 |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 1740Hz | 0.65 | 2.65 |
| 15KHz,10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | 9.79 | 11.79 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 1740Hz | 8.67 | 10.67 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | -5.651 | -3.651 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(3,7,11) | AWGN 3334Hz | -5.651 | -3.651 |
| 30KHz,40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | 8.544 | 10.544 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(3,7,11) | AWGN 3334Hz | 8.545 | 10.545 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 1740Hz | 4.12 | 6.12 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(3,7,11) | AWGN 1740Hz | 0.46 | 2.46 |
| 15KHz, 10MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 1740Hz | 11.72 | 13.72 |
| 15KHz, 10MHz | 16 | 1T1R | 1+1+1(3,7,11) | AWGN 1740Hz | 12.04 | 14.04 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 3334Hz | -2.66 | -0.66 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(3,7,11) | AWGN 3334Hz | -2.65 | -0.65 |
| 30KHz, 40MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 3334Hz | 11.78 | 13.78 |
| 30KHz, 40MHz | 16 | 1T1R | 1+1+1(3,7,11) | AWGN 3334Hz | 12.19 | 14.19 |

PUSCH mapping type A

As agreed in the last meeting,

- L0 for PUSCH mapping type A - If no performance different between l0=2 and l0=3, define performance requirements based on l0=2 - L0 value for testing is based on BS declaration

As indicated in tables, with l0=3 DMRS configuration, the performance is better compared with l0=2 for the condition of large maximum Doppler.

Observation 1: L0=3 with 1+1+1 DMRS configuration is better compared with L0=2 with 1+1+1 DMRS configuration with large maximum Doppler.

Proposal 3:  HST requirement with PUSCH mapping type A under L0=3 can be defined.

MCS

- 500km/h - Open space scenario: MCS2 and MCS16 - Tunnel scenario: MCS2, FFS on MCS16

As indicated, the maximum TP with MCS 16 is feasible for tunnel scenario.

Proposal 4: HST requirement with tunnel scenario with MCS 2 and MCS 16 can be defined.

# 3	Conclusion

In this contribution, the view of simulation assumption for NR HST requirement was provided. Also, the initial ideal simulation results are provided to check the feasibility of high Doppler.

Proposal 1: Only define HST requirement with 1x2 antenna configuration for tunnel scenario.

Observation 1: L0=3 with 1+1+ 1 DMRS configuration has better compared with L0=2 with 1+1+1 DMRS configuration with large Doppler value.

Proposal 2: The Slots in which PUSCH is transmitted is proposed as follow

| Slots in which PUSCH is transmitted | For FDD : slot #0 and #8 in radio frames for which SFN mod 4 = 0 slot #6 in radio frames for which SFN mod 4 = 1 slot #4 in radio frames for which SFN mod 4 = 2 slot #2 in radio frames for which SFN mod 4 = 3 For TDD in 15KHz SCS: slot #4 in each radio frames For TDD in 30KHz SCS slot #8 and slot#10 in radio frames |
| --- | --- |

Proposal 3: Define the HST requirement with PUSCH mapping type A under L0=3.

Proposal 4: HST requirement with tunnel scenario with MCS 2 and MCS 16 can be defined.

# References

R4-1912809 Way forward for NR HST PUSCH demodulation requirements, Huawei