3GPP TSG-RAN WG4 Meeting #94-e              	R4-2000308

Online, 24th February– 6th March, 2020

Agenda item:	8.17.2.3.3

Source:	Samsung

Title:	Discussion and initial simulation results for NR HST UL timing requirement

Document for:	Discussion

# 1	Introduction

During the last RAN4#93 meeting, the general issues about requirements for UL timing adjustment were discussed and captured in the WF [1].

In this contribution, we present our views on the remaining issues. Meanwhile, based on simulation assumption, the initial simulation results are provided for alignment.

# 2	Discussion and simulation results

## 2.1	Performance requirements for UL timing adjustment

- SRS configuration

Generally, PUSCH resource blocks should be included in SRS resource blocks. In LTE, 40RBs is allocated for 10, 15 and 20MHz channel BW. In LTE, the SRS resource blocks are configured with the SRS bandwidth configuration and SRS-bandwidth, differentiated with uplink bandwidth. For example, in case of 10 MHz channel BW, 40RBs are allocated for SRS resource, the configuration table is illustrated as following table:

Table 5.5.3.2-2:  and, , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 1 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 2 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 3 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 4 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 5 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 6 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 7 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |

Based on the this mapping table, the related SRS-bandwidth configuration CSRS=2 and SRS-Bandwidth BSRS=0 (b=0), which is aligned with the FRC table as

| Reference channel | A7-1 | A7-2 | A7-3 | A7-4 | A7-5 | A7-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 3 | 6 | 12 | 25 | 25 | 25 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Payload size (bits) | 1288 | 2600 | 5160 | 10680 | 10680 | 10680 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 1 | 1 | 2 | 2 | 2 |
| Coded block size including 12bits trellis termination (bits) | 3948 | 7884 | 15564 | 16140 | 16140 | 16140 |
| Total number of bits per sub-frame | 1728 | 3456 | 6912 | 14400 | 14400 | 14400 |
| Total symbols per sub-frame | 432 | 864 | 1728 | 3600 | 3600 | 3600 |
| SRS bandwidth configuration (See TS 36.211, 5.5.3) (Note 1) | 7 | 5 | 3 | 2 | 5 | 2 |
| SRS-Bandwidth b (See TS 36.211, 5.5.3) (Note 1, 2) | 0 | 0 | 0 | 0 | 0 | 1 |
| NOTE 1. The transmission of SRS is optional NOTE 2. PUSCH resource blocks shall be included in SRS resource blocks |  |  |  |  |  |  |

In the last meeting, it is agreed that

- Allocated RBs for SRS - 10MHz CBW/15KHz SCS: 40 contiguously allocated RBs starting from PRB index 0 - 40MHz CBW/30KHz SCS: 80 contiguously allocated RBs starting from PRB index 0

Base on the RAN1 spec, the length of SRS sequence is given by

Msc,bSRS=mSRS,bNscRBKTC

Where mSRS,b is selected with  and CSRS  in Table 6.4.1.4.3-1 [2]

Since there is no frequency hopping for PUSCH, it is straightforward SRS frequency hopping should be disable. So, the value of BSRS is 0.

The SRS bandwidth configuration is proposed as

C_SRS = 11, B_SRS =0, for 40RB

C_SRS = 21, B_SRS =0, for 80RB

Proposal 1:  SRS bandwidth configuration is proposed as

C_SRS = 11, B_SRS =0, for 40RB

C_SRS = 21, B_SRS =0, for 80RB

Transmission comb

For transmission comb, NR can support with 2 or 4.The maximum number of cyclic shift is 8 for transmission comb 2, and cyclic shift is 12 for transmission comb 4. For simplicity, to align with LTE, we prefer to use transmission comb 2.

Proposal 2:  SRS transmission comb is proposed as

K_TC: 2

SRS periodic

For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity  (in slots) and slot offset  are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying

To align with LTE, 10ms periodicity is preferred.

Proposal 3:  SRS transmission periodicity is proposed as

=10.

SRS frequency domain starting position

To align with PUSCH, it is agreed that the frequency domain starting position of SRS is k0=0.

- SRS slot configuration

As checked in LTE, we found there are some mismatches with between with spec 36.104 and spec 36.141 for the SRS transmission configuration for TDD duplex mode.

In spec 36.104, the test parameters for testing UL timing adjustment are indicated as follows

Table 8.2.2-1 Test parameters for testing UL timing adjustment Parameter; Value. Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1, 0, 2, 3, 1. Uplink-downlink allocation for TDD; Configuration 1 (2:2). Subframes in which PUSCH is transmitted; For FDD: subframe #0, #2, #4, #6, and #8 in radio frames For TDD: subframe #2, #3, #7 and #8 in radio frames. Subframes in which sounding RS is transmitted (Note 1); For FDD: subframe #1 in radio frames For TDD: UpPTS in subframe #1 in radio frames. Note 1. The transmission of SRS is optional..

While in spec 36.141, the test parameters are provided as follows:

Table 8.2.2.4.2-2 Test parameters for testing UL timing adjustment Parameter; Value. Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1, 0, 2, 3, 1. Uplink-downlink allocation for TDD; Configuration 1 (2:2). Subframes in which PUSCH is transmitted; For FDD: subframe #0, #2, #4, #6, and #8 in radio frames For TDD: Subframe #2, #3, #7, #8 in each radio frame. Subframes in which sounding RS is transmitted (Note 1); For FDD: subframe #1 in radio frames For TDD: UpPTS in each radio frame. Note 1: The transmission of SRS is optional..

With checking the previous approved CR history, the TDD configuration in 104 and 141 are adding in [2], since the test parameters for UL timing adjustment are not suitable for TDD. So, the configuration “UpPTS in subframe #1 in radio frames” is added for TDD configuration.

However, there is no reason to explain the different test configuration in 104 and 141

For NR HST UL timing adjustment requirement, it is agreed that SRS transmission configuration is indicated as follows:

- SRS transmission (optional) - FDD - Slot #1 in radio frames - TDD - The last symbol in the special slot - 15KHz SCS: last symbol in slot#3 in radio frames - 30KHz SCS: last symbol in slot#7 in radio frames

In order to align with NR spec 104,141-1 and 141-2, we propose to use the same parameters for requirements and testing.

Proposal 4:  Keep the same SRS parameters specified in 38.104/141-1/141-2 for requirements and testing.

- Moving propagation conditions

In LTE, as illustrates in Figure1, the time difference between the reference timing and the first tap is according the follow equation. The timing difference between moving UE and stationary UE is equal to Δτ - (TA 31)16Ts. The relative timing among all taps is fixed.

Figure 1: Moving propagation conditions

In NR, for a SCS of  kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of .  As indicated in 38.213 spec

In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of a current value, , to the new value, , by index values of = 0, 1, 2,..., 63, where for a SCS of kHz, .

So, in our view, the related timing different between moving UE and stationary UE should be scaled with related SCS as

15KHz:   Δτ - (TA 31)16*64Tc

30KHz:   Δτ - (TA 31)16*32Tc

Proposal 5:  The timing difference between moving UE and stationary UE should be scaled with

15KHz SCS:   Δτ - (TA 31)16*64Tc

30KHz SCS:   Δτ - (TA 31)16*32Tc

- FRC table

Based on the above proposal and agreed assumption [1], the FRC table for UL timing adjustment can be calculated as follows:

| Reference channel |  |  |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 30 |
| Allocated resource blocks | 25 | 50 |
| CP-OFDM Symbols per slot (Note 1) | 11 | 11 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 8456 | 16896 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 |
| Number of code blocks - C | 2 | 3 |
| Code block size including CRC (bits) (Note 2) | 4264 | 5664 |
| Total number of bits per slot | 13200 | 26400 |
| Total symbols per slot | 3300 | 6600 |
| SRS bandwidth configuration CSRS(See Ts 38.211, 6.4.1.4) (Note 3) | 11 | 21 |
| SRS-Bandwidth BSRS (See Ts 38.211, 6.4.1.4) (Note 4) | 0 | 0 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos2, l0= 2 , l=7 and l=11 for PUSCH mapping type A, l0= 0, l=5 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [15]. NOTE 3: The transmission of SRS is optional NOTE 4: PUSCH resource blocks shall be included in SRS resource blocks, the SRS frequency-domain start position k0=0 |  |  |

## 2.2	Simulation Results

In this subsection, based on the agreed assumption, the initial simulation results are provided for alignment purpose.

Table 1: Ideal and impairment simulation results for UL timing adjustment

| SCS,BW | MCS | Antenna Configuration | DMRS configuration | Mapping Type | Symbol length | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz: 10MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 |  |  |
| 15kHz: 10MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 |  |  |
| 30kHz: 40MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 |  |  |
| 30kHz: 40MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 |  |  |

# 3	Conclusion

In this contribution, the view on UL timing requirement for HR HST is provided. Meanwhile, we presented initial ideal and impairment simulation results for alignment purpose.

Proposal 1:  SRS bandwidth configuration is proposed as

C_SRS = 11, B_SRS =0, for 40RB

C_SRS = 21, B_SRS =0, for 80RB

Proposal 2:  SRS transmission comb is proposed as

K_TC: 2

Proposal 3:  SRS transmission periodicity is proposed as

=10.

Proposal 4:  Keep the same SRS parameters specified in 38.104/141-1/141-2 for requirements and testing.

Proposal 5:  The timing difference between moving UE and stationary UE should be scaled with

15KHz SCS:   Δτ - (TA 31)16*64Tc

30KHz SCS:   Δτ - (TA 31)16*32Tc

# References

R4-1915886, WF on high speed related requirements for NR BS demodulation, Huawei

R4-090455, CR on modifications on UL timing adjustment test case, CATT