3GPP TSG-RAN WG4 Meeting #94-e                                          R4-2000325

Electronic Meeting, 24th Feb– 6th Mar, 2020

Source:	Samsung

Title:	Introduction of LTE re-farming band n53 to NR specification

Agenda item:			9.26

Document for:	Approval

# 1. Introduction

In RAN#86 meeting the WI on Introduction of NR Band n53 was agreed with objective as below:

- Specify a new NR TDD operating Band n53 in consideration of E-UTRA Band 53 requirements.

- Specify the following SCS and channel bandwidths:

| NR Band | SCS kHz | 5 MHz | 10 MHz |
| --- | --- | --- | --- |
| n53 | 15 | Yes | Yes |
|  | 30 |  | Yes |
|  | 60 |  | Yes |

This contribution provides analysis on UE and BS specification impact due to introduction of n53.

# 2. Discussion

System parameters

NR band n53 specific system parameters, which are common in UE and BS specification, include new NR operating band n53 as table below, SCS and channel bandwidth as indicated in WID, channel raster and sync raster.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |

The NR global channel raster and sync raster design since Rel-15 definitely should be applied for n53 as well. Furthermore, for channel raster, 100 kHz should be applied for n53 as most below 3GHz NR operating bands. And for sync raster, it is proposed to use 15 kHz SSB SCS plus SSB pattern A for n53. It should be pointed that the DSS LTE/NR band 53/n53 is not considered in this discussion. If DSS is requested, additional specification impact may be involved in.

Proposal 1: NR operating band n53, SCS and channel bandwidth should be introduced to NR specification according to WID

Proposal 2: Channel raster of n53 should be defined as 100 kHz.

Proposal 3: Sync raster for n53 should be defined based on 15 kHz SSB SCS and SSB pattern A.

Proposal 4: NR-ARFCN and GSCN range of n53 should be derived based on existing global channel raster and sync raster design respectively.

BS specific requirement for n53

The band specific RF requirements should be updated to incorporate operating band n53 are the operating band unwanted emission and spurious emissions include:

- Basic limits for Wide Area BS(Category A) of OBUE

- Additional operating band unwanted emission

- Additional spurious emissions requirements

- Co-location with other base stations under spurious emissions

UE specific requirement for n53

- UE maximum output power

- UE maximum output power for UL-MIMO

- UE maximum output power with additional requirement

- Spurious emission band UE co-existence

- Spurious emission band UE co-existence for CA

- Special emission with NS value 45

- REFSENS

- Blocking requirement

# 3. Conclusion

In this contribution, the specification impact to introduce n53 is discussed with below proposals.

Proposal 1: NR operating band n53, SCS and channel bandwidth should be introduced to NR specification according to WID

Proposal 2: Channel raster of n53 should be defined as 100 kHz.

Proposal 3: Sync raster for n53 should be defined based on 15 kHz SSB SCS and SSB pattern A.

Proposal 4: NR-ARFCN and GSCN range of n53 should be derived based on existing global channel raster and sync raster design respectively.

# 4. Reference

[1] RP-192409, WID on Introduction of NR Band n53