3GPP TSG-RAN4 Meeting #94-e	R4-2000354

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0205 | rev | - | Current version: | 15.8.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UL MIMO Emission requirements and alignement with RAN1 terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UL MIMO output power and emission are defined to different reference planes and this allows UE to pass 3GPP but fail regulatory requirements. RAN1 alsi changed their terminology to apply UL MIMO dedicated requirements to UE only when UE is configured for UL MIMO, not when UE supports UL MIMO. Ref: RAN1 TS 38.213 CR 0039 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Where it says “UE with two transmit connectors” and “ UE supporting UL MIMO” is replaced with “For UE configured for two logical antenna ports” and in the following the sentence “of sum of powers fromall transmit antenna connectors” is also added instead of “each antenna connector”. For transmit signal quality, “each antenna connector” is kept. “The requirements shall be met with UE configuration specified in subsection 6.2D.1.” is added and in some cases reference to Table 6.2D.1-2 is removed since table alone is not explicit. New section headings “6.4D.2.0 General” and “6.5D.0 General” are added to remove the hanging paragraph “Codebook index” changed to “TPMI index” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications are not clear for UE that supports UL MIMO and may cause different interpretations on correct implmentations. UL MIMO requirements do not support all planned implmentation and allow UEs to pass test when they would be failing regulatory requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2, 6.2D.3, 6.2D.4, 6.3D.1, 6.3D.2, 6.3D.3, 6.3D.4, 6.4D.1, 6.4D.2.0, 6.4D.2.1, 6.4D.2.2, 6.4D.2.3, 6.4D.2.4, 6.4D.3, 6.4D.4, 6.5D.0, 6.5D.1, 6.5D.2, 6.5D.3, 6.5D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

start of changes >

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For power class 2 UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. For configured for two logical antenna ports, the maximum output power is measured as the sum of the maximum output power from all UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | TPMI Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

< end of changes >

< start of changes >

### 6.2D.2	UE maximum output power reduction for UL MIMO

For UE configured for two logical antenna ports in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

.

For UE configured for two logical antenna ports, the maximum output power is measured as the sum of the maximum output power from all UE antenna connectors.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4 apply.

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

For UE configured for two logical antenna ports in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1.. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

For UE configured for two logical antenna ports, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4 apply.

### 6.2D.4	Configured transmitted power for UL MIMO

For UE configured for two logical antenna ports, the configured transmitted power is defined for UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.4 shall apply to UE supporting UL MIMO, where

-	PPowerClass, ΔPPowerClass and ∆TC,c are specified in subclause 6.2D.1;

-	MPRc is specified in subclause 6.2D.2;

-	A-MPRc is specified in subclause 6.2D.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1-1 for the applicable operating band.

For UE configured for two logical antenna ports in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2D.4-1. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

Table 6.2D.4-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

< end of changes >

< start of changes >

## 6.3D	Output power dynamics for UL MIMO

### 6.3D.1	Minimum output power for UL MIMO

For UE configured for two logical antenna ports in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1-1. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

### 6.3D.2	Transmit OFF power for UL MIMO

The transmit OFF power for UE is defined as the mean power of sum of powers fromall transmit antenna connectors in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power for UE shall not exceed the values specified in Table 6.3.2-1. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

### 6.3D.3	Transmit ON/OFF time mask for UL MIMO

For UE configured for two logical antenna ports the ON/OFF time mask requirements in subclause 6.3.3 apply for sum of powers from all transmit antenna connectors.

For UE configured for two logical antenna ports in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in subclause 6.3.3.1 apply to sum of powers of from each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in subclause 6.2D.1.

### 6.3D.4	Power control for UL MIMO

For UE configured for two logical antenna ports the power control tolerance applies to the sum of output power from all transmit antenna connectors.

The power control requirements specified in subclause 6.3.4 apply to UE configured for two logical antenna ports in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations specified in subsection 6.2D.1.

< end of changes >

< start of changes >

## 6.4D	Transmit signal quality for UL MIMO

### 6.4D.1	Frequency error for UL MIMO

For configured for two logical antenna ports, the basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

### 6.4D.2	Transmit modulation quality for UL MIMO

#### 6.4D.2.0 General

For UE configured for two logical antenna ports, the transmit modulation quality requirements are specified at each transmit antenna connector.

If UE is configured for transmission on single-antenna port, the requirements specified for single carrier apply.

The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in subclause 6.4D.2.2 and 6.4D.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4D.2.1	Error Vector Magnitude

For UE configured for two logical antenna portsin closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in Table 6.4.2.1-1 which is defined in subclause 6.4.2.1 apply at each transmit antenna connector. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

#### 6.4D.2.2	Carrier leakage

For UE configured for two logical antenna portsin closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2-1 which is defined in subclause 6.4.2.2 apply at each transmit antenna connector. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

#### 6.4D.2.3	In-band emissions

For UE configured for two logical antenna portsin closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.4.2.3-1 which is defined in subclause 6.4.2.3 apply at each transmit antenna connector. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

#### 6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO

For UE configured for two logical antenna portsin closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.4.2.4-1 and Table 6.4.2.4-2 which are defined in subclause 6.4.2.4 apply at each transmit antenna connector. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

### 6.4D.3	Time alignment error for UL MIMO

For UE configured for two logical antenna ports, this requirement applies to frame timing differences between transmissions on multiple transmit antenna connectors in the closed-loop spatial multiplexing scheme.

The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors.

For UE configured for two logical antenna ports, the Time Alignment Error (TAE) shall not exceed 130 ns.

### 6.4D.4	Requirements for coherent UL MIMO

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna ports in any slot within the specified time window from the last transmitted SRS on the same antenna ports, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna port is larger than 0 dBm for SRS transmission and for the duration of time window. The requirements shall be met with UE configuration specified in subsection 6.2D.1.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

The above requirements when all the following conditions are met within the specified time window:

-	UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config

-	UE remains in DRX active time (UE does not enter DRX OFF time)

-	No measurement gap occurs

-	No instance of SRS transmission with the usage antenna switching occurs

-	Active BWP remains the same

-	EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PSCell or SCell(s))

< end of changes >

< start of changes >

## 6.5D	Output RF spectrum emissions for UL MIMO

### 6.5D.0 General

## The output RF spectrum emission requirements apply to the sum of power measured over all transmit antenna connectors.

### 6.5D.1	Occupied bandwidth for UL MIMO

For UE configured for two logical antenna ports, the requirements for occupied bandwidth is specified for a sum of output power from all  transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel.

For UE configured for two logical antenna portsin closed-loop spatial multiplexing scheme, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in table 6.5.1-1. The requirements shall be met with UL MIMO configurations specified in subsection 6.2D.1.

### 6.5D.2	Out of band emission for UL MIMO

For UE configured for two logical antenna ports, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters are specified for a sum of all powers from all transmit antenna connectors.

For UEconfigured for two logical antenna portsin closed-loop spatial multiplexing scheme, the requirements in subclasuse 6.5.2 apply to sum of all powers from all transmit antenna connectors. The requirements shall be met with UL MIMO configurations specified in subsection 6.2D.1.

### 6.5D.3	Spurious emission for UL MIMO

For UE configured for two logical antenna ports, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products are specified for UE.

For UEconfigured for two logical antenna portsin closed-loop spatial multiplexing scheme, the requirements specified in subclasuse 6.5.3 apply to sum of all powers from all transmit antenna connectors. The requirements shall be met with the UL MIMO configurations specified in subsection 6.2D.1.

### 6.5D.4	Transmit intermodulation for UL MIMO

For UE configured for two logical antenna ports, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UE configured for two logical antenna portsin closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.4 apply to  sum of all powers from all transmit antenna connectors. The requirements shall be met with the UL MIMO configurations specified in subsection 6.2D.1.

< end of changes >