3GPP TSG-RAN WG4 Meeting #94-e	R4-2000404

Electronic meeting, 24th February – 6th March 2020

Source:	Ericsson

Title:	Simulation results for Rel-16 NR HST PUSCH demodulation performance at UE speed of 350 km/h

Agenda item:	8.17.2.2.1

Document for:	Discussion

# Introduction

In this contribution, we presented our simulation results for NR HST PUSCH demodulation performance for both HST scenarios and UL timing adjustment for HST speed = 350 km/h.

# Simulation assumptions and simulation results

In this section, we presented our simulation assumptions based on Table 1 for HST scenarios. The corresponding ideal simulation results and results with impairments for HST scenarios are given in Table 2 and Table 3. The 1x1 antenna configuration in the tunnel scenario is still FFS and needs to be decided, however we have captured results in this document for the case that the 1x1 scenario is included (the scenario is feasible).

| Parameters | Values |
| --- | --- |
|  | FR1 |
| Waveform | CP-OFDM |
| Number of Tx and Rx | Tunnel scenario (Dmin = 2 m, Ds = 300 m): [1Tx 1Rx] and 1Tx 2Rx Open space scenario (Dmin = 150 m, Ds = 700 m): 1Tx 2Rx and 1Tx 8Rx |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 DM-RS symbols with l0 = 2 |
| Number of symbols | 14 |
| Time domain resource allocation type | type A |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 2, 16 |
| Carrier frequency (fc) | fc = 2.1 GHz for SCS = 15 kHz fc = 3.6 GHz for SCS = 30 kHz |
| Propagation channel | HST tunnel channel model: Ds = 300 m and Dmin = 2 m HST open space channel model: Ds = 700 m and Dmin = 150 m |
| SCS and BW | 15kHz: 10 MHz 30kHz: 40 MHz |
| Speed (v) | 350 km / h |
| Maximum Doppler shift (fd) | 15 kHz: 1340 Hz 30 kHz: 2334 Hz |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for HST single tap scenarios at 350 km/h [1].

Note that in our previous contribution [2], we’ve demonstrated that there is no difference in performance when the front-loaded DM-RS symbol is located at l0 = 2 or l0 = 3. Thus, in this contribution, we propose l0 = 2 and show results for l0 = 2.

Table 2. Ideal simulation results for NR HST PUSCH demodulation performance.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 10 MHz | 2 | -3,0 |
|  |  | 16 | 9,7 |
|  | 30 kHz / 40 MHz | 2 | -3,0 |
|  |  | 16 | 9,5 |
| Tunnel (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -6,0 |
|  |  | 16 | 6,6 |
|  | 30 kHz / 40 MHz | 2 | -6,0 |
|  |  | 16 | 6,5 |
| Open space (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -6,1 |
|  |  | 16 | 6,3 |
|  | 30 kHz / 40 MHz | 2 | -6,0 |
|  |  | 16 | 6,2 |
| Open space (1Tx8Rx) | 15 kHz / 10 MHz | 2 | -12 |
|  |  | 16 | 0,3 |
|  | 30 kHz / 40 MHz | 2 | -12,0 |
|  |  | 16 | 0,3 |

Table 3. Results with impairments for NR HST PUSCH demodulation performance.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 10 MHz | 2 | -0,9 |
|  |  | 16 | 12,1 |
|  | 30 kHz / 40 MHz | 2 | -1,0 |
|  |  | 16 | 11,7 |
| Tunnel (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -3,6 |
|  |  | 16 | 9,1 |
|  | 30 kHz / 40 MHz | 2 | -3,6 |
|  |  | 16 | 9,0 |
| Open space (1Tx2Rx) | 15 kHz / 10 MHz | 2 | -4,0 |
|  |  | 16 | 8,5 |
|  | 30 kHz / 40 MHz | 2 | -3,6 |
|  |  | 16 | 8,5 |
| Open space (1Tx8Rx) | 15 kHz / 10 MHz | 2 | -9,9 |
|  |  | 16 | 2,6 |
|  | 30 kHz / 40 MHz | 2 | -9,5 |
|  |  | 16 | 2,4 |

Observation 1: From the simulation results, we showed that it is feasible to achieve reasonable PUSCH demodulation performance at 350 km/h with both MCS = 2 and MCS = 16 for both open space and tunnel scenarios.

# Conclusion

In this contribution, we summarized our simulation results for NR HST at UE speed of 350 km/h.

Observation 1: From the simulation results, we showed that it is feasible to achieve reasonable PUSCH demodulation performance at 350 km/h with both MCS = 2 and MCS = 16 for both open space and tunnel scenarios.

# References

R4-1915870, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon.

R4-1914549, Further elaboration on front-loaded DM-RS symbol for NR HST PUSCH demodulation requirements, Ericsson.