3GPP TSG-RAN WG4 Meeting #94-e	R4-2000407

Electronic meeting, 24th February – 6th March 2020

Source:	Ericsson

Title:	Simulation results for PRACH HST with 350km/hour

Agenda item:	8.17.2.2.2

Document for:	Discussion

# Introduction

The following consensuses, on the test assumptions/requirements of PRACH high speed scenario with velocity at 350km/h, were captured in the WF [1], as part of the outcomes of the meeting RAN4#93.

- PRACH format: for 350km/h velocity, use PRACH format 0

- Restricted set for PRACH format 0: both type A and type B

- Frequency offset under AWGN for PRACH format 0 for 350km/h

  - Restricted set Type A

    - High-mobility scenario: 1340Hz

    - Low-mobility scenario: 625Hz

  - Restricted set Type B

    - High-mobility scenario: 2334Hz

    - Low-mobility scenario: 625Hz

The agreed test preamble parameters are listed below:

- Restricted set type A (Format 0)

  - {Ncs, logical sequence index, V} = {15, 384,0}

    - Common understanding:  For restricted set type A, under high Doppler more than 1340Hz other preambles than the tested one may deviate in performance

- Restricted set type B (Format 0)

  - {Ncs, logical sequence index, V} = {15, 30,30}

- Timing error tolerance for AWGN and TDLC 300-100

  - Reuse the time error tolerance for corresponding NR PRACH normal performance requirements, i.e.

| Format | SCS | AWGN | TDLC300-100 |
| --- | --- | --- | --- |
| 0 | 1.25 | 1.04us | 2.55us |
| A2,B4,C2 | 15 | 0.52us | 2.03us |
|  | 30 | 0.26us | 1.77us |

Some other assumptions agreed in [2] are provided as below,

- Number of receiver antennas: 2 Rx, 4 Rx, 8 Rx

- Test metric

  - False alarm probability: 0.1%

  - Missed detection probability: 1%

In this contribution, we will provide the simulation results of HST with speed being 350km/h.

# Discussion

To avoid any confusion, the simulation parameters are listed here.

- Carrier frequency for velocity at 350km/h

  - Restricted set A: 2.1GHz

  - Restricted set B: 3.6GHz

- Sub-carrier spacing: 1.25kHz for format 0

- Number of transmitter antennas: 1 Tx

- Total number of PRACH preambles: 64

The simulation results are included in the following tables.

Table 1. ideal simulation results for 2Rx PRACH format 0 restricted set

| Restricted set Type | Channel Type | Frequency offset (Hz) | SNR (dB) |
| --- | --- | --- | --- |
| Restricted set A | AWGN | 1340 | -15,5 |
|  |  | 625 | -14,3 |
|  | TDL-C 300-100 | 400 | -7,6 |
| Restricted set B | AWGN | 2334 | -14,4 |
|  |  | 625 | -15,7 |
|  | TDL-C 300-100 | 400 | -8,3 |

Table 2. ideal simulation results for 4Rx PRACH format 0 restricted set

| Restricted set Type | Channel Type | Frequency offset (Hz) | SNR (dB) |
| --- | --- | --- | --- |
| Restricted set A | AWGN | 1340 | -17,9 |
|  |  | 625 | -16,7 |
|  | TDL-C 300-100 | 400 | -13,0 |
| Restricted set B | AWGN | 2334 | -16,7 |
|  |  | 625 | -17,9 |
|  | TDL-C 300-100 | 400 | -13,7 |

Table 3. ideal simulation results for 8Rx PRACH format 0 restricted set

| Restricted set Type | Channel Type | Frequency offset (Hz) | SNR (dB) |
| --- | --- | --- | --- |
| Restricted set A | AWGN | 1340 | -20,1 |
|  |  | 625 | -18,9 |
|  | TDL-C 300-100 | 400 | -17,3 |
| Restricted set B | AWGN | 2334 | -18,9 |
|  |  | 625 | -20,1 |
|  | TDL-C 300-100 | 400 | -17,7 |

Table 4. impaired results for 2Rx PRACH format 0 restricted set

| Restricted set Type | Channel Type | Frequency offset (Hz) | SNR (dB) |
| --- | --- | --- | --- |
| Restricted set A | AWGN | 1340 | -13,4 |
|  |  | 625 | -12,0 |
|  | TDL-C 300-100 | 400 | -5,1 |
| Restricted set B | AWGN | 2334 | -11,9 |
|  |  | 625 | -13,6 |
|  | TDL-C 300-100 | 400 | -5,8 |

Table 5. impaired results for 4Rx PRACH format 0 restricted set

| Restricted set Type | Channel Type | Frequency offset (Hz) | SNR (dB) |
| --- | --- | --- | --- |
| Restricted set A | AWGN | 1340 | -15,4 |
|  |  | 625 | -14,5 |
|  | TDL-C 300-100 | 400 | -10,6 |
| Restricted set B | AWGN | 2334 | -14,6 |
|  |  | 625 | -15,7 |
|  | TDL-C 300-100 | 400 | -11,2 |

Table 6. impaired results for 8Rx PRACH format 0 restricted set

| Restricted set Type | Channel Type | Frequency offset (Hz) | SNR (dB) |
| --- | --- | --- | --- |
| Restricted set A | AWGN | 1340 | -17,7 |
|  |  | 625 | -16,4 |
|  | TDL-C 300-100 | 400 | -15,0 |
| Restricted set B | AWGN | 2334 | -16,9 |
|  |  | 625 | -17,7 |
|  | TDL-C 300-100 | 400 | -15,2 |

# Conclusion

Simulation results for PRACH HST scenario with format 0, when velocity is 350km/h, are provided as shown in Section 2.

# References

| [1] | Samsung, R4-1915914, Way forward on NR HST PRACH demodulation requirement, Reno, US, November 18-22, 2019. |
| --- | --- |
| [2] | Huawei, R4-1910126, Way forward on NR HST BS demodulation requirements, Ljubljana, Slovenia, August 26-30, 2019. |