3GPP TSG-RAN WG4 Meeting #94-e                 	 R4-2000456

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.14.2.1

Source:	MediaTek Inc.

Title:	MRTD requirements for FR2 inter-band DL CA

Document for:		Discussion

# Introduction

In last meeting, a WF [1] was agreed to further discuss the MRTD requirements for FR2 inter-band CA in RRM session of RAN4#94 meeting.

- In RRM session in RAN4#94, discuss if MRTD can be reduced or not. - Alt.1: Max propagation delay difference is 1 us, and MRTD can be revised to 4 us - Alt.2: No change in MRTD, i.e., 8us. - Alt.3: Other values not precluded.

In this paper, we provide our views on this issue.

# Discussion

Before going directly to the requirement, one thing to mention is on how RAN4 RF session discusses FR2 inter-band CA. As we know, current FR2 bands can be categorized into 2 groups: 28GHz or 39GHz. FR2 inter-band CA within the same group may share similar requirements as FR2 intra-band CA. In our view, it is good for RRM session to align the methodology in RF session for consistency among specs. Therefore, RRM session should also consider to further update existing MRTD requirements based on whether the FR2 inter-band CA is from the same group of different groups.

Proposal 1: To align with the discussion in FR session, RRM session should also consider 2 different MRTD requirements for FR2 inter-band CA from the same group (28+28 or 39+39) and different groups (28+39).

In the following, when inter-band CA is mentioned, it means 28GHz + 39GHz, because we believe that the MRTD requirements for intra-band CA can be directly applicable to inter-band CA for 28 GHz + 28 GHz or 39 GHz + 39 GHz.

In TS38.133, the MRTD requirement of FR2 inter-band CA was specified within Rel-15 timeline, although there were no corresponding band combinations defined in TS38.101-2 yet. The current requirement is 8us.

For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below. Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2; 8. Between FR1 and FR2; 25.

Now, RAN4 starts to specify the band combinations and corresponding RF requirements for FR2 inter-band CA in Rel-16. We believe that there is not yet any deployments or devices for FR2 inter-band CA at this moment. So the concern of backward compatibility due to revising the MRTD requirements should not be a problem here.

Observation 1: Given no deployments or devices for FR2 inter-band CA at this moment, the concern of backward compatibility due to revising the MRTD requirements should not be a problem here.

The MRTD requirement is the maximum received timing difference that should be supported by UE. It is contributed by 2 component. One from the timing alignment error (TAE) at BS side, and the other from the difference between the propagation paths of 2 carriers, as illustrated in Figure 1.

Figure 1. MRTD = TAE + difference in propagation delay

According to TS38.104, the TAE for inter-band CA shall not exceed 3us.

#### 6.5.3.2 Minimum requirement for BS type 1-C and BS type 1-H For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

Taking 3us as the worst case, the remaining margin to tolerance the difference in propagation delay is

8us – 3us = 5us

The propagation delay in terms of 5us means 1500m in distance. Some more examples for the mapping between propagation distance and propagation delay are provided in Table 1.

Table 1. Mapping between propagation distance and propagation delay

| Difference in propagation delay (us) | Difference in propagation distance (m) |
| --- | --- |
| 1 | 300 |
| 2 | 600 |
| 3 | 900 |
| 4 | 1200 |
| 5 | 1500 |

This 1500m is obviously an over design of the system. As we know, typical FR2 cell sizes is around 200m. Allowing this large difference in propagation delay is not going to further provide benefit to the deployment, but only demands extra UE complexity. Therefore, we believe that it is possible to reduce the MRTD so that we can save some UE complexity without scarifying the flexibility in FR2 deployment.

Observation 2: According to current inter-band CA MRTD and TAE requirements, the max difference in propagation delay is 1500m which is obvious an over design of the FR2 system. It is possible to reduce the MRTD so that we can save some UE complexity without scarifying the flexibility in FR2 deployment.

Of course, some margin is needed when we consider FR2 network deployment. We think the new MRTD could be some value within 4us to 5us, which still provides the deployment flexibility with cell size 300m to 600ms.

Proposal 2: Revise the MRTD requirement for FR2 inter-band CA to

- Within the same group (28+28 or 39+39): 260ns

- Across different groups (28+39): [4 or 5]us

# Summary

In this paper, we provide our view on the MRTD requirement for FR2 inter-band CA. We have the following observations and proposal.

Observation 1: Given no deployments or devices for FR2 inter-band CA at this moment, the concern of backward compatibility due to revising the MRTD requirements should not be a problem here

Observation 2: According to current inter-band CA MRTD and TAE requirements, the max difference in propagation delay is 1500m which is obvious an over design of the FR2 system. It is possible to reduce the MRTD so that we can save some UE complexity without scarifying the flexibility in FR2 deployment.

Proposal 1: To align with the discussion in FR session, RRM session should also consider 2 different MRTD requirements for FR2 inter-band CA from the same group (28+28 or 39+39) and different groups (28+39).

Proposal 2: Revise the MRTD requirement for FR2 inter-band CA to

- Within the same group (28+28 or 39+39): 260ns

- Across different groups (28+39): [4 or 5]us

# Reference

[1] R4-1916024, “WF on FR2 inter-band DL CA”, Nokia