3GPP TSG-RAN4 Meeting #94-e	R4-2000509

# Online, 24th Feb 2020 - 6th Mar 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FR2 Intraband Non-contiguous UL CA

Agenda Item: 			8.14.1.6

Document for:	Discussion

# 1	Introduction

This contribution is based on [1] but adds more simulation results, namely for 50+50 MHz and 200+200 MHz.

# 2	Discussion

## 2.1	Simulation scenarios

Two different sub-block scenarios were simulated, one for 2 sub-block case and another for 3 sub-block case. In both cases sub-block bandwidths were 100 MHz and the gap(s) were also 100 MHz. For 2 sub-block case and edge carriers in 3 sub-block case the allocations were always in the outer edges of the sub-block that is in the low edge of the lowest   sub-block and high edge of the highest sub-block. Middle sub-block allocation in 3 sub-block case was placed such that the center of the allocation was equal distance from the center of the lower sub-block allocation and center of the higher sub-block allocation. For 2 sub-block case all possible allocation size combinations were simulated but for 3   sub-block case only a subset.

Figure 1: Allocation placement

## 2.2	Simulation assumptions

PA calibration point was 100 MHz, 120 kHz, QPSK, DFT-S-OFMA, 20RB23with 0 dB MPR

IQ-Image and LO leakage = 25 dBc

CIM3 = 60 dBc

SEM and as in [1]

Table 6.5.2.1-1: General NR spectrum emission mask for frequency range 2.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |

General spur = -13 dB/1 MHz and as in [1]

ACLR=17 dB and as in [1]

OBW = 99% of the emissions of ALL allocations were inside sub-block bandwidths.

Equal PSD in sub-blocks and equal back-off.

## 2.3	Simulation results

### 2.3.1	2 sub-blocks

Results are presented as squares where x-axis represent allocation size in lower sub-block and y-axis represent allocation size in upper sub-block. Lower left-hand corner is the 1RB+1RB case and higher right hand corner is the full allocation on both sub-blocks.

Left hand side figure is the required back-off and right-hand side figure is the gating factor. When comparing the amount of required backoff i.e. MPR we can see that CP-OFDM and DFT-s-OFDM do not differ much. This is due to the fact that as DFT-s-OFDM has two sub-blocks the PAPR increases compared to single carrier case and approaches CP-OFDM PAPR. From the results in appendix can be seen that inner modulation has virtually no effect to MPR either.

To improve readability of DFT-S-OFDM figures in the new simulation results for 50+50 and 200+200, empty space in single-color regions was filled with color.

Most demanding case from BO point of view is 1RB+1RB as always and easiest is when only one of the sub-blocks has very narrow allocation.

Figure 2: 50+50 CP-OFDM

Figure 3: 100+100 CP-OFDM

Figure 4: 200+200 CP-OFDM

Figure 5: 50+50 DFT-s-OFDM

Figure 6: 100+100 DFT-s-OFDM

Figure 7: 200+200 DFT-s-OFDM

### 2.3.2	3 sub-blocks

For aggregation of three CCs, we run a simulation on the so-called triple-beat scenario where the center frequencies of allocations in all three CCs are equally spaced. As a result, several IMD components fall onto each other on an equally spaced grid, thus maximizing the power of IMD lobes in the spectrum. The configuration was 100+100+100MHz (one CC per subblock) with 100 MHz subblocks gaps, QPSK, SCS = 60 kHz.

Figure 8 depicts the simulated back-off as function of the total allocation size and also shows the gating factor of each allocation.

Figure 8: Back-off as function of total allocation size for OFDM (left) and DFT-S-OFDM (right) in triple-beat scenario with 100+100+100 MHz channel combination with 100 MHz gaps between channels.

Another scenario with high emissions is when the centre CC has no allocation. Then all power is concentrated in the allocations of the outermost CCs, resulting in high emission lobes, especially if the CC allocations are narrow. These cases are practically equivalent with the two-CC scenario. They could apply the MPR specification of the two-CC case and do not need to be simulated separately.

### 2.3.3	New simulation results for Un-even CC bandwidths

New simulations were run to study the effect of different CC bandwidths. The conclusions made below in Section 2.4 are valid also in the light of these new results.

Figure 9: 50+100 MHz aggregation with 100 MHz gap. CP-OFDM

Figure 10: 50+100MHz with 100 MHz gap. DFT-s-OFDM

Figure 11: 50+200MHz with 100 MHz gap. QPSK only

## 2.4	How to write MPR

### 2.4.1	2CC case

As can be seen from Figure 12 MPR is basically same for all modulations. It is rather difficult to craft MPR scheme based on these plots as MPR is quite the same except when other allocation is very narrow and the other allocation medium size. Perhaps a single MPR = 5 dB for all callocations is best fit or alternatively 5 dB for spur-limited and 4 dB for the rest.

Figure 12: 2x100MHz CC with 100 MHz gap. CP-OFDM

For DFT-s-OFDM there is bit more variation on MPR compared to CP-OFDM. Perhaps 2 MPR values could be consider that is 5 dB for spur-limited lower left-hand side corner allocations and 3 dB for rest, see Figure 6.

Figure 13: 2x100MHz CC with 100 MHz gap. DFT-s-OFDM

### 2.4.2	3CC case

3 CC case cannot be presented same way as 3 CC case instead we have used simple format explained in clause 2.3.2. Similarly as for DFT-s-OFDM 2CC case 2 MPR solution can be consider that is one MPR for spurious limited case and another one for the rest (OBW limited) for which MPR would be in the order of 5 and 3.5 dB respectively. This definition would assume equal PSD and is total backoff.

Figure 14: 3x100MHz CC with 100 MHz gaps. CP-OFDM (left) and DFT-s-OFDM (right)

# 4	Conclusion

In this contribution we have provided more MPR simulation results which are for 50+50 MHz and 200+200 MHz cases.

# 5	References

[1] R4-1913981	FR2 Intraband Non-contiguous UL CA 	Nokia, Nokia Shanghai Bell

# 6	Appendix