3GPP TSG-RAN WG4 Meeting #94-e	R4-2000561

Electronic Meeting, 24th Feb. – 6th Mar., 2020

Agenda item:	8.15.1.6

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on mandatory MG patterns for FR2

Document for:	Discussion

- Introduction

Agreement

- Confirm R15 capability information elements cannot be changed in meaning by any new capability signaling introduced in R16

- Confirm R15 gap pattern applicability rule is not changed due to introduction of additional mandatory gap patterns in R16

WF

- Introduce a new Rel-16 UE capability to indicate measurement gap patterns for 'NR only measurements'

  - NR only measurement is to be clarified.

- Applicability of additional mandatory gap patterns and new UE capability signalling

  - Additional mandatory gap patterns and new UE capability in Rel-16 are applicable for at least NR SA and NR-DC mode

  - FFS applicability for EN-DC, NE-DC, and LTE SA mode

- FFS Mandatory gap patterns for FR1

- FFS Mandatory gap patterns for FR2

At the last RAN4 #93 meeting, mandatory MG patterns enhancement was discussed and WF was agreed [1]. The agreements and WF are as follows:

In this contribution, we propose our views on mandatory gap patterns for FR2.

- Discussion

In previous RAN4 discussions about mandatory MG patterns, many companies shared their views of the importance of MG patterns with 3.5ms MGL. However, in Rel-15, only Gap Pattern Id #0, #1, #13, and #14 were mandated because of its versatility and affinity for 6ms MGL of LTE gap patterns. As we proposed in previous document [2], at least for UE with per-FR gap capability, Gap Pattern Id #17, #18, and #19 are beneficial because the length of SS burst set of FR2 (SCS = 120kHz, 240kHz) is less than 3ms in many cases. Table 1 shows the typical maximum SS burst set length of FR2 (SCS = 120kHz or 240kHz, TDD config = DDDSU).

Table 1: Calculated length of SS burst set respect to SSB SCS and number of SSB

| SSB SCS | the number of SSB in SS burst set |  |
| --- | --- | --- |
|  | 32 | 64 |
| 120kHz | 2.75ms | 5.5ms |
| 240kHz | 1.375ms | 2.75ms |

Also as we mentioned in previously, current spec restricts the measurement configurations between RLM-RS, SMTC, and measurement gap in FR2 as follows:

- UE cannot perform RLM within measurement gap

- UE needs to share measurement gap occasion with other inter-frequency/inter-RAT measurement if SMTC for intra-frequency measurement without gap is fully overlapped with measurement gap

- UE needs to scale RLM periodicity when RLM-RS outside of measurement gap is fully overlapped with SMTC window.

In order to efficiently perform RLM, intra-frequency measurement and inter-frequency/inter-RAT measurement, NW may configure each periodicity satisfying TSSB < TSMTCperiod < MGRP. Moreover the shorter SSB periodicity cause more scheduling restriction because UE would be always expected to perform Rx beam sweeping on SSB utilized for RLM and intra-frequency measurement, i.e. N = 8. According to this observation, the gap patterns with longer MGRP than 40ms may have flexibility and applicability for several use cases. Thus, such gap patterns shall be mandated.

Observation 1: Gap patterns with shorter MGL are valuable in FR2 since the length of SS burst set is within 3ms in most cases.

Observation 2: Gap patterns with longer MGRP are valuable in FR2 considering some scheduling restrictions in FR2.

Proposal 1: Gap patterns from #17 to #19 shall be mandatory support for per-FR gap for FR2 in addition to gap patterns mandated in Rel-15.

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |

- Conclusion

In this contribution, we proposed our views on additional mandatory gap patterns for FR2.

Observation 1: Gap patterns with shorter MGL are valuable in FR2 since the length of SS burst set is within 3ms in most cases.

Observation 2: Gap patterns with longer MGRP are valuable in FR2 considering some scheduling restrictions in FR2.

Proposal 1: Gap patterns from #17 to #19 shall be mandatory support for per-FR gap for FR2 in addition to gap patterns mandated in Rel-15.

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |

References

[1] 3GPP, R4-1914794, ZTE, “Way forward on RRM enhancement: mandatory of gap patterns”.

[2] 3GPP, R4-1905868, NTT DOCOMO, “Default gap patterns for per-FR measurement gap in FR2”.