3GPP TSG-RAN WG4 Meeting #94-e	R4-2000563

# Electronic meeting, 24 Feb. – 6 Mar., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0226 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 on corrections to NR CA bandwidth classes (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra |  |  |  |  |  | Date: |  |  | 2020-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The aggregated chanel bandwidth for NR CA bandwidth class B should be in the range of 20 MHz ≤ BWChannel_CA ≤ 100 MHz. In addition, NR CA bandwidth class F is no longer defined in Table 5.3A.5-1 and should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the aggregated channel bandwidth for NR CA bandwidth class B. - Remove NR CA bandwidth class F in Table 5.3A.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of NR CA bandwidth class will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

5.3A.5	UE channel bandwidth per operating band for CA

The requirements for carrier aggregation in this specification are defined for carrier aggregation configurations.

For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets specified in clause 5.5A.1. For each carrier aggregation configuration, requirements are specified for all aggregated channel bandwidths contained in a bandwidth combination set, a UE can indicate support of several bandwidth combination sets per carrier aggregation configuration. For intra-band non-contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting two or more sub-blocks, each supporting a carrier aggregation bandwidth class.

For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class.

Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
|  |  |  | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group |  |  |  |

==============================================================

<< End of changes >>